Table of Contents

Cover Sheet	2

Vessel Design Summary:	3

Nozzle Summary:	6

Nozzle Schedule:	8

Bill of Material:	10

MDMT Summary:	11

Warnings and Errors:	13

Input Echo:	14

XY Coordinate Calculations:	22

Flg Calc [Int P]:   BODY FLANGE	23

Flg Calc [Int P]:   COVER FLANGE	28

Internal Pressure Calculations:	32

Element: BOTTOM DISH	33

Element: SHELL	35

Pressure Test Results: Shop Test	37

Pressure Test Results: Field Test	39

External Pressure Calculations:	41

Element and Detail Weights:	42

Nozzle Flange MAWP:	45

Natural Frequency Calculation:	46

Wind Load Calculation:	47

Earthquake Load Calculation:	50

Wind/Earthquake Shear, Bending:	52

Wind/Earthquake Deflection:	53

Longitudinal Stress Constants:	54

Longitudinal Allowable Stresses:	55

Longitudinal Stresses due to:	56

Stress due to Combined Loads:	58

Center of Gravity Calculation:	62

Lifting Lug Calcs:	64

Leg Check, (Operating Case):	72

Leg Check, (Filled w/Water):	84

Nozzle Calcs.:  N8	95

Nozzle Calcs.:  N5	101

Nozzle Calcs.:  N6A	108

Nozzle Calcs.:  N6B	111

Nozzle Calcs.:  N4A	114

Nozzle Calcs.:  N4B	117

Nozzle Calcs.:  M1	120

Nozzle Calcs.:  N1	125

Nozzle Calcs.:  N7	136

Nozzle Calcs.:  N9	147

Nozzle Calcs.:  N3	150

Nozzle Calcs.:  N2	155

Tag No.44-21-U-6209-05

Biocide Chemical Injection

DESIGN CALCULATION

In Accordance with ASME Section VIII Division 1

ASME Code Version     : 2023

Analysis Performed by :

Job File              : ...\Biocide Chemical Injection.pvdb

Date of Analysis      : Mar 12,2025   9:37pm

PV Elite 26 SP2, August 2024

DESIGN DATA

| S No | DESCRIPTION | UNIT | VALUES |  | REMARKS |
| --- | --- | --- | --- | --- | --- |
| 1 | Code Of Construction | ASME SEC VIII DIV 1 2023 EDITION |  |  |  |
| 2 | Type of Vessel | Vertical |  |  |  |
| 3 | Shell Size | mm | 1000 ID and 2019.35 Length (TL to TL) Shell 6mm and Dish end 8mm (Nom) /6mm (Min) thk. |  |  |
| 4 | Design Internal Pressure | Barg | 0.5 + Full of liquid |  | UG-21 |
| 5 | Design External Pressure | Barg | NIL |  | UG-21 |
| 6 | Design Temperature (Min/Max) | C | 4 / 85 |  | UG-20 |
| 7 | Operating Temperature | C | 58 |  |  |
| 8 | Min. Design Metal Temperature | C | 4 |  | UG-20 |
| 9 | Metal Temperature during Hydrotest | C | 17/45 |  | UG-99(c) & (h) |
| 10 | MAWP (Hot and Corroded) | Barg | 0.83 |  | UG-98 |
|  | MAWEP |  | NA |  |  |
|  | MAP (New and Cold) |  | NA |  |  |
| 11 | Hydrotest Pressure at Top of Vessel (Shop / Site) | Barg | 1.035 / 0.744 |  | UG-99(c) / UG-99(b) |
| 12 | Operating Pressure (Int/Ext) | Barg | ATM / NIL |  |  |
| 13 | Operating Medium | CORROSION INHIBITOR |  |  |  |
|  | Medium Density | kg/m3 | 1038 |  |  |
|  | Capacity (Working/Nominal) | m3 | 0.77 / 1.57 |  |  |
| 14 | Corrosion Allowance (Int/Ext) | mm | NA |  |  |
| 15 | Joint Efficiency |  |  |  |  |
|  | Shell Long Seam |  | 1 |  |  |
|  | Shell to Head Circ Seam |  | 1 |  |  |
|  | Shell to Flange Circ Seam |  | 1 |  |  |
| 16 | Radiography (RT-1) |  |  |  |  |
|  | Shell Long Seam |  | Full | UW-11(a) |  |
|  | Shell to Head Circ Seam |  | Full | UW-11(a) |  |
|  | Nozzles |  | Full | UW-11(a) |  |
|  | Dished End |  | Seamless/Full RT |  |  |
| 17 | PWHT |  | Exempted | As per UHA Table UHA-32-3 |  |
| 18 | Impact Testing |  | Exempted | As per UHA 51(d)(1)(a) |  |
| 19 | Special Service |  | NA |  |  |
| 20 | Inspection Opening |  | Yes |  |  |
| 21 | Insulation |  | No |  |  |
| 22 | Fire Proofing |  | NA |  |  |
| 23 | Wind Load Calculation | As per ASCE - 07 (2016) Basic wind speed 60.2 m/s |  |  |  |
| 24 | Seismic Load Calculation | As per ASCE-7-16 Ss=0.5, S1=0.15, SITE CLASS-D, IF =1.25 |  |  |  |

Vessel Design Summary:

ASME Code, Section VIII Division 1, 2023

Diameter Spec : 1000.000 mm. ID

Vessel Design Length, Tangent to Tangent           2017.35  mm.

Distance of Bottom Tangent above Grade              305.00  mm.

Specified Datum Line Distance                         0.00  mm.

Internal Design Temperature                             85  °C

Internal Design Pressure                             0.522  bars

External Design Temperature                              0  °C

Maximum Allowable Working Pressure                    0.52  bars

Shop Test Pressure                                   0.777  bars

Field Test Pressure                                  0.777  bars

Required Minimum Design Metal Temperature              4.0  °C

Warmest Computed Minimum Design Metal Temperature    -48.0  °C

Wind Design Code                             ASCE/SEI 7-16

Earthquake Design Code                       ASCE/SEI 7-16

Materials of Construction:

Component |                 |       |                        |         |Normal |  Impact |

Type |        Material | Class |              Thickness |   UNS # |  ized |  Tested |

----------------------------------------------------------------------------------------------

Shell |     SA-240 316L |   ... |                    ... |  S31603 |    No |      No |

Head |     SA-240 316L |   ... |                    ... |  S31603 |    No |      No |

Flange |     SA-240 316L |   ... |                    ... |  S31603 |    No |      No |

Nozzle |   SA-312 TP316L |   ... |                    ... |  S31603 |    No |      No |

Re-Pad |     SA-240 316L |   ... |                    ... |  S31603 |    No |      No |

Nozzle Flg |    SA-182 F316L |   ... |                 <= 130 |  S31603 |    No |      No |

Leg Baseplate |          S275JR |   ... |                    ... |  K02600 |    No |      No |

Flg Bolting |      SA-193 B8M |     1 |                    ... |  S31600 |    No |      No |

Leg Bolting |          SA 325 |   ... |                    ... |         |    No |      No |

Normalized is determined based on the UCS-66 material curve selection and Figure UCS-66.

Impact Tested is based on material selection and material data properties.

Element Pressures and MAWP (bars & mm.):

|        Design |         |           |       Total | Str. |    In |

Element Description |      Pressure |    Ext. |   Element |   Corrosion | Flg. | Creep |

or Type |  + Stat. head |  Press. |   M.A.W.P |   Allowance | Gov. | Range |

--------------------------------------------------------------------------------------------

BOTTOM DISH |         0.750 |    0.00 |    11.800 |      0.0000 |   No |    No |

SHELL |         0.720 |    0.00 |    11.800 |      0.0000 |  N/A |    No |

BODY FLANGE |         0.522 |    0.00 |     0.640 |      0.0000 |  N/A |    No |

COVER FLANGE |         0.522 |    0.00 |     0.910 |      0.0000 |  N/A |    No |

Liquid Level: 2242.00 mm.  Dens.: 0.001 kg./cm³    Sp. Gr.: 1.040

Element Types and Properties:

Element |"To" Elev |  Element |  Nominal | Finished | Reqd Thk | Reqd Thk |   Long |   Circ |

|          |   Length |Thickness |Thickness | Internal | External |    Eff |    Eff |

Type |      mm. |      mm. |      mm. |      mm. |      mm. |      mm. |        |        |

------------------------------------------------------------------------------------------------

Ellipse  |     50.0 |     50.0 |      8.0 |      6.0 |      1.5 |      ... |   1.00 |   1.00 |

Cylinder |   1992.0 |   1942.0 |      6.0 |      6.0 |      1.5 |      ... |   1.00 |   1.00 |

Body Flg |   1998.2 |     16.0 |     16.0 |     16.0 |     14.5 |     12.4 |   1.00 |   1.00 |

Body Flg |   2017.4 |     16.0 |     16.0 |     16.0 |     12.1 |     12.4 |   1.00 |   1.00 |

Loads for Foundation/Support Design:

Total Wind Shear on top of all Legs                        2247.  N

Total Earthquake Shear on top of all Legs                  4995.  N

Total Wind Moment at top of all Legs                       1263.  N-m

Total Earthquake Moment at top of all Legs                 5435.  N-m

Wind Load Case Test Moment on Support                       631.  N-m

Test Case Wind Shear on Support                            1123.  N

Max. Wind Shear on one Leg (top & bottom)                   813.  N

Max. Earthq. Shear on one Leg (top & bottom)               1807.  N

Max. Wind Moment at base of one Leg                        1179.  N-m

Max. Earthquake Moment at base of one Leg                  2621.  N-m

Max. Vertical Load (Wt. + Wind) on one Leg                 8306.  N

Max. Vertical Load (Wt. + Eq.) on one Leg                 12032.  N

For the Pressure Test Case:

Max. Wind Shear on one Leg (top & bottom)                   406.  N

Max. Wind Moment at base of one Leg                         589.  N-m

Max. Vertical Load (Wt. + Wind) on one Leg                 8013.  N

Max. Vertical Load (Wt. + Eq.) on one Leg                  7449.  N

The Wind and Seismic loads were not load factored in this analysis.

Wind and Earthquake moments include the effects of user defined forces and moments if any exist in

the job and were specified to act (compute loads and stresses) during these cases.  Also included

are moment effects due to eccentric weights if any are present in the input.

Local Stress Analysis Results:

|        Analysis |    Max Stress |   High Stress |          Pass |

Description |            Type |         Ratio |      Location |          Fail |

------------------------------------------------------------------------------------

Lift Lug |     WRC-107/537 |         0.717 |           n/a |        Passed |

LEGS |     WRC-107/537 |         0.291 |           n/a |        Passed |

LEGS |     WRC-107/537 |         0.076 |           n/a |        Passed |

N8 |     WRC-107/537 |         0.707 |           n/a |        Passed |

N5 |     WRC-107/537 |         0.707 |           n/a |        Passed |

N1 |         WRC-297 |         0.355 |         Shell |        Passed |

N7 |         WRC-297 |         0.355 |         Shell |        Passed |

Weights:

Fabricated - Bare  W/O Removable Internals                 910.4  kg.

Shop Test  - Fabricated + Water ( Full )                  3005.1  kg.

Shipping   - Fab. + Rem. Intls.+ Shipping App.             944.0  kg.

Erected    - Fab. + Rem. Intls.+ Insul. (etc)             1344.0  kg.

Empty      - Fab. + Intls. + Details + Wghts.             1344.0  kg.

Operating  - Empty + Operating Liquid (No CA)             3105.9  kg.

Field Test - Empty Weight + Water (Full)                  3038.7  kg.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Nozzle Calculation Summary (bars & mm.):

Description        |       MAWP |    Ext |      MAPNC | UG-45    [tr] |    Weld |   Areas or |

|            |        |            |       |       |    Path |   Stresses |

----------------------------------------------------------------------------------------------

N8                 |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N5                 |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N6A                |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N6B                |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N4A                |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N4B                |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

M1                 |       0.52 |    ... |        ... |   ... |       |     OK  |     Passed |

N1                 |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N7                 |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N9                 |       0.52 |    ... |        ... |    OK |  1.50 |     OK  | No Calc[*] |

N3                 |       0.52 |    ... |        ... |    OK |  5.26 |     OK  |     Passed |

N2                 |       0.52 |    ... |        ... |    OK |  3.42 |     OK  | No Calc[*] |

----------------------------------------------------------------------------------------------

Nozzle MAWP Summary:

Minimum MAWP Nozzles         :            0.52 Nozzle  : N2

Minimum MAWP Shells/Flanges  :            0.52 Element : BODY FLANGE

Minimum MAPnc Shells/Flanges :            0.74 Element : BODY FLANGE

--------------------------------------------------------------------

Computed Vessel M.A.W.P.     :            0.52 bars

[*] - This was a small opening and the areas were not computed or

the MAWP of this connection could not be computed because

the longitudinal bending stress was greater than the hoop stress.

Multiple output lines for the same nozzle indicates required Code calculations in both the

longitudinal and circumferential planes of reinforcement where applicable.

Check the Spatial Relationship between the Nozzles:

From Node | Nozzle Description | Y Coordinate | Layout Angle |    Dia. Limit |

|                    |          mm. |          deg |           mm. |

------------------------------------------------------------------------------

10 |                 N8 |        0.000 |        0.000 |        85.700 |

10 |                 N5 |        0.000 |        0.000 |        85.700 |

20 |                N6A |     1800.000 |       90.000 |        85.700 |

20 |                N6B |      150.000 |       90.000 |        85.700 |

20 |                N4A |     1800.000 |      180.000 |        85.700 |

20 |                N4B |      150.000 |      180.000 |        85.700 |

20 |                 M1 |      864.000 |      270.000 |      1193.800 |

20 |                 N1 |     1800.000 |      225.000 |        85.700 |

20 |                 N7 |     1800.000 |      135.000 |        85.700 |

20 |                 N9 |       50.000 |       45.000 |        85.700 |

40 |                 N3 |        0.000 |      270.000 |       204.521 |

40 |                 N2 |        0.000 |      180.000 |        92.325 |

The nozzle spacing is computed by the following:

= sqrt( ll² + lc² ) where

ll - Arc length along the inside vessel surface in the long. direction.

lc - Arc length along the inside vessel surface in the circ. direction

If any interferences/violations are found, they will be noted below.

No interference violations have been detected!

Checking Multiple Nozzles on Flat Head per ASME Sec. VIII Div. 1 UG-39

Comparing Nozzles on Element: COVER FLANGE

UG-39 Average Nozzle Diameter and Ligament Checks:

Nozzle |    Avg. | Head Dia. | Ligament |  Min. dn |

Pair Description |    dia. |        /4 |    Width |       /4 |

|     mm. |       mm. |      mm. |      mm. |

------------------------------------------------------------------

N3 & N2              |  72.555 |     265.5 |   407.66 |   10.712 |

------------------------------------------------------------------

UG-39 Nozzle Spacing and Average Area Checks:

Nozzle |  Center | Avg. dia. | Avg. Area | Avg. Area |

Pair Description | Spacing |    * 1.25 | Available |     Reqd. |

|     mm. |       mm. |       cm² |       cm² |

--------------------------------------------------------------------

N3 & N2              |  494.97 |    90.694 |   No Calc |   No Calc |

--------------------------------------------------------------------

UG-39 Nozzle Diameter and Distance to Edge Checks:

Nozzle |   Nozzle |   Head Dia. |   Distance |    Nozzle |

Description |     dia. |          /2 |  from Edge |    dia./4 |

|      mm. |         mm. |        mm. |       mm. |

-------------------------------------------------------------------

N3              |   102.26 |         531 |     157.85 |    25.565 |

N2              |    42.85 |         531 |     184.84 |    10.712 |

-------------------------------------------------------------------

No Multiple Nozzle spacing violations detected!

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Nozzle Schedule:

Nominal or|   Schd|  Flg|  Nozzle|    Wall| Reinforcing Pad |    Cut|   Flg|

Actual| or FVC| Type|   O/Dia|     Thk| Diameter    Thk | Length| Class|

Description            Size|   Type|     |      mm|      mm|              mm |     mm|      |

---------------------------------------------------------------------------------------------

N8              |     50 mm|   160 |  WNF|    60.3|   8.738|     ... |   ... |    156|   150|

N5              |     50 mm|   160 |  WNF|    60.3|   8.738|     ... |   ... |    165|   150|

N6A             |     50 mm|   160 |  WNF|    60.3|   8.738|     ... |   ... |    156|   300|

N6B             |     50 mm|   160 |  WNF|    60.3|   8.738|     ... |   ... |    156|   300|

N4A             |     50 mm|   160 |  WNF|    60.3|   8.738|     ... |   ... |    156|   300|

N4B             |     50 mm|   160 |  WNF|    60.3|   8.738|     ... |   ... |    156|   300|

N1              |     50 mm|   160 |  WNF|    60.3|   8.738|  162.00 |  6.00 |    156|   150|

N7              |     50 mm|   160 |  WNF|    60.3|   8.738|  162.00 |  6.00 |    156|   150|

N9              |     50 mm|   160 |  WNF|    60.3|   8.738|     ... |   ... |    156|   150|

N2              |     50 mm|   160 |  WNF|    60.3|   8.738|  162.00 |  6.00 |    166|   150|

N3              |    100 mm|   40S |  WNF|   114.3|   6.020|  214.30 |  6.00 |    166|   300|

M1              |    600 mm|   10S |  WNF|   609.6|   6.350| 1050.00 |  6.00 |    259|   150|

General Notes for the above table:

The cut length is the outside projection + inside projection + drop + in-plane shell

thickness.  This value does not include weld gaps, nor does it account for shrinkage.

In the case of oblique nozzles, the outside diameter must be increased. The re-pad width

around the nozzle is calculated as follows:

Width of Pad = (Pad Outside Dia. (per above) - Nozzle Outside Dia.)/2

For hub nozzles, the thickness and diameter shown are those of the smaller and thinner section.

Nozzle Material and Weld Fillet Leg Size Details (mm.):

|                    | Shl Grve | Noz Shl/Pad |    Pad OD |   Pad Grve |   Inside |

Description |           Material |     Weld |        Weld |      Weld |       Weld |     Weld |

-----------------------------------------------------------------------------------------------

N8          |      SA-312 TP316L |    6.000 |       8.000 |       ... |        ... |     ...  |

N5          |      SA-312 TP316L |    6.000 |       8.000 |       ... |        ... |     ...  |

N6A         |      SA-312 TP316L |    6.000 |       6.000 |       ... |        ... |     ...  |

N6B         |      SA-312 TP316L |    6.000 |       6.000 |       ... |        ... |     ...  |

N4A         |      SA-312 TP316L |    6.000 |       6.000 |       ... |        ... |     ...  |

N4B         |      SA-312 TP316L |    6.000 |       6.000 |       ... |        ... |     ...  |

N1          |      SA-312 TP316L |    6.000 |       6.000 |     6.000 |      6.000 |     ...  |

N7          |      SA-312 TP316L |    6.000 |       6.000 |     6.000 |      6.000 |     ...  |

N9          |      SA-312 TP316L |    6.000 |       6.000 |       ... |        ... |     ...  |

N2          |      SA-312 TP316L |   16.000 |       6.000 |     6.000 |      6.000 |     ...  |

N3          |      SA-312 TP316L |   16.000 |       6.000 |     6.000 |      6.000 |     ...  |

M1          |      SA-312 TP316L |    6.000 |       6.000 |     6.000 |      6.000 |     ...  |

Note: The Outside projections below do not include the flange thickness.

Nozzle Miscellaneous Data:

|   Elev/Distance |   Layout |        Proj |    Proj |        Installed in |

Description      |      From Datum |    Angle |     Outside |  Inside |           Component |

|              mm.|      deg |          mm.|      mm.|                     |

---------------------------------------------------------------------------------------------

N8              |             ... |      0.0 |      150.00 |    0.00 |         BOTTOM DISH |

N5              |             ... |      0.0 |      150.00 |    0.00 |         BOTTOM DISH |

N6A             |        1800.000 |     90.0 |      150.00 |    0.00 |               SHELL |

N6B             |         150.000 |     90.0 |      150.00 |    0.00 |               SHELL |

N4A             |        1800.000 |    180.0 |      150.00 |    0.00 |               SHELL |

N4B             |         150.000 |    180.0 |      150.00 |    0.00 |               SHELL |

N1              |        1800.000 |    225.0 |      150.00 |    0.00 |               SHELL |

N7              |        1800.000 |    135.0 |      150.00 |    0.00 |               SHELL |

N9              |          50.000 |     45.0 |      150.00 |    0.00 |               SHELL |

N2              |             ... |    180.0 |      150.00 |    0.00 |        COVER FLANGE |

N3              |             ... |    270.0 |      150.00 |    0.00 |        COVER FLANGE |

M1              |         864.000 |    270.0 |      150.00 |    0.00 |               SHELL |

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Bill of Materials:

QTY |                                                     DESCRIPTION |        MATERIAL |

-------------------------------------------------------------------------------------------

1 |   ELLIPTICAL HEAD: 2.0 X 1, 8.0mm. THK X 1000.0mm. ID X 50.0mm. |     SA-240 316L |

-------------------------------------------------------------------------------------------

1 |                 CYLINDER: 6.0mm. THK X 1000.0mm. ID X 1942.0mm. |     SA-240 316L |

-------------------------------------------------------------------------------------------

1 |                      SLIP ON FLANGE: 16.0mm. THK X 1130.0mm. OD |     SA-240 316L |

-------------------------------------------------------------------------------------------

1 |                        BLIND FLANGE: 16.0mm. THK X 1130.0mm. OD |     SA-240 316L |

-------------------------------------------------------------------------------------------

1 |                                   PLATFORM: 1100mm X 914mm WIDE |                 |

-------------------------------------------------------------------------------------------

1 |                                                LADDER: 1998.0mm |             ... |

-------------------------------------------------------------------------------------------

4 |                                    LEGS, SHAPE: HE100A 1450.0mm |             ... |

-------------------------------------------------------------------------------------------

4 |                          BASEPLATES: 300.0mm X 280.0mm X 30.0mm |           S275JR |

-------------------------------------------------------------------------------------------

16 |                                         BASEPLATE BOLTS: 24.0mm |        SA 325 |

-------------------------------------------------------------------------------------------

4 |                    LEG RE-PADS: 200.000mm X 200.000mm X 6.000mm |     SA-240 316L |

-------------------------------------------------------------------------------------------

2 |                                                   LIFTING LUGS: |     SA-240 316L |

-------------------------------------------------------------------------------------------

2 |                 LIFTING LUG PADS: 130.0mm X 270.0mm X 8.0mm THK |                 |

-------------------------------------------------------------------------------------------

1 |                       CLASS 150 GR 2.3, 600.0mm BLIND FLANGE(S) |    SA-182 F316L |

-------------------------------------------------------------------------------------------

1 |                             GASKET: 1130.0mm. OD X 1013.0mm. ID |             ... |

-------------------------------------------------------------------------------------------

56 |                                  BODY FLANGE BOLTS: 16.0mm. DIA |      SA-193 B8M |

-------------------------------------------------------------------------------------------

112 |                         NUTS FOR BODY FLANGE BOLTS: 16.0mm. DIA |             ... |

-------------------------------------------------------------------------------------------

1 |                                                       NAMEPLATE |             ... |

-------------------------------------------------------------------------------------------

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Minimum Design Metal Temperature Results Summary (°C):

Curve  Basic  Reduced  UG-20(f)  Thickness     Gov      E*   PWHT

Description               MDMT     MDMT      MDMT      ratio     Thk           reqd

Notes                                                mm.

-----------------------------------------------------------------------------------

BODY FLANGE  [11]         -196

COVER FLANGE [11]         -196

BOTTOM DISH  [16]         -196

SHELL        [16]         -196

N8           [15]         -196

N5           [15]         -196

N6A          [15]         -196

N6B          [15]         -196

N4A          [15]         -196

N4B          [15]         -196

M1           [15]         -196

N1           [15]         -196

N7           [15]         -196

N9           [15]         -196

N3           [15]         -196

N2           [15]         -196

Nozl. Bolting[22]          -48                         1.000  64.000    1.00    Yes

-----------------------------------------------------------------------------------

Warmest MDMT:              -48

Required Minimum Design Metal Temperature              4.0  °C

Warmest Computed Minimum Design Metal Temperature    -48.0  °C

Notes:

[  ! ] - This was an impact tested material.

[  1] - Governing Nozzle Weld.

[  4] - ASME Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-c).

[  5] - ASME Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-b).

[  6] - MDMT Calculations at the Shell/Head Joint.

[  7] - MDMT Calculations for the Straight Flange.

[  8] - Cylinder/Cone/Flange Junction MDMT.

[  9] - Calculations in the Spherical Portion of the Head.

[10] - Calculations in the Knuckle Portion of the Head.

[11] - Calculated (Body Flange) Flange MDMT.

[12] - Calculated Flat Head MDMT per UCS-66.3

[13] - Tubesheet MDMT, shell side, if applicable

[14] - Tubesheet MDMT, tube side, if applicable

[15] - Nozzle Material

[16] - Shell or Head Material

[17] - Impact Testing required

[18] - Impact Testing not required, see UCS-66(b)(3)

[20] - Cylinder/Cone Junction MDMT based on Longitudinal Stress considerations

[21] - Body Flange Bolting Material

[22] - Nozzle Flange Bolting Material

[23] - Stiffening Ring to Shell Weld

[24] - Saddle to Shell Weld

UG-84(b)(2) was not considered.

UCS-66(g) was not considered.

UCS-66(i) was not considered.

Notes:

Impact test temps were not entered in and not considered in the analysis.

UCS-66(i) applies to impact tested materials not by specification and

UCS-66(g) applies to materials impact tested per UG-84.1 General Note (c).

The Basic MDMT includes the (30F) PWHT credit if applicable.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Class From To : Basic Element Checks.

--------------------------------------------------------------------------

Class From To: Check of Additional Element Data

--------------------------------------------------------------------------

There were no geometry errors or warnings.

PV Elite performs all calculations internally in Imperial Units

to remain compliant with the ASME Code and any built in assumptions

in the ASME Code formulas. The finalized results are reflected to show

the set of selected units for this analysis.

Information Regarding Nozzle Loads

Please note that nozzle loadings, if included, are assumed to be local in nature

and will not contribute to or create a net section bending moment.  Therefore, the

addition of nozzle loads will not affect the support load calculation.  If you wish

to create a load on the support(s) from nozzle loads, you can enable that feature in

tools->configuration->Job Specific Settings.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Units used in this Analysis (bars 1):

Name            System Unit         Constant        User Unit

-------------------------------------------------------------

Length          Feet                304.8000        mm.

Force           Pounds                4.4480        N

Mass            Pounds                0.4536        kg.

Area            sq. inches            6.4516        cm²

Moment          ft. lbs.              1.3563        N-m

Stress          lbs./sq.in.           0.0069        N./mm²

Temperature     Degrees F             0.5556        °C

Pressure        psig                  0.0689        bars

Elast. Modulus  lbs./sq.in.           6.8946        KPa.

Pipe Density    lbs./cu.in.           0.0277        kg./cm³

Ins. Density    lbs./cu.ft.       0.1600E-04        kg./cm³

Fluid Density   lbs./cu.ft.       0.1600E-04        kg./cm³

Wind Speed      miles/hr              1.6093        Km/hr

Tray Weight     lbs./sq.ft.           0.0005        Kgs/cm²

Inertia         in.^4                 41.6231        cm^4

G Load          G's                   1.0000        g's

Wind Load       lbs./sq.ft.           4.8825        Kgs/m²

Elevation       Feet                304.8000        mm.

Volume          in.^3                 16.3900        cm³

Diameter        inches               25.4000        mm.

Thickness       inches               25.4000        mm.

PV Elite Vessel Analysis Program: Input Data

Tag No.44-21-U-6209-05

Biocide Chemical Injection

Design Internal Pressure (for Hydrotest)               0.522  bars

Design Internal Temperature                             85.0  °C

Projection of Nozzle from Vessel Top                       0  mm.

Projection of Nozzle from Vessel Bottom                    0  mm.

Minimum Design Metal Temperature                         4.0  °C

Type of Construction                                  Welded

Special Service                                       Lethal

Degree of Radiography                                   RT-1

Use Higher Longitudinal Stresses (Flag)                    Y

Select t for Internal Pressure (Flag)                      N

Select t for External Pressure (Flag)                      N

Select t for Axial Stress (Flag)                           N

Select Location for Stiff. Rings (Flag)                    N

Consider Vortex Shedding                                   N

Shop Pressure Test:

Type of Pressure Test                               UG-99(b)

Pressure Test Position                              Vertical

Test Performed in Corroded Condition                     Yes

Field Pressure Test:

Type of Pressure Test                     UG-99(b) Note [35]

Pressure Test Position                              Vertical

Test Performed in Corroded Condition                     Yes

Load Case 1                                   NP+EW+WI+FW+BW

Load Case 2                                   NP+EW+EE+FS+BS

Load Case 3                                   NP+OW+WI+FW+BW

Load Case 4                                   NP+OW+EQ+FS+BS

Load Case 5                                         NP+HW+HI

Load Case 6                                         NP+HW+HE

Load Case 7                                   IP+OW+WI+FW+BW

Load Case 8                                   IP+OW+EQ+FS+BS

Load Case 9                                   EP+OW+WI+FW+BW

Load Case 10                                  EP+OW+EQ+FS+BS

Load Case 11                                        HP+HW+HI

Load Case 12                                        HP+HW+HE

Load Case 13                                        IP+WE+EW

Load Case 14                                        IP+WF+CW

Load Case 15                                        IP+VO+OW

Load Case 16                                        IP+VE+EW

Load Case 17                                        NP+VO+OW

Load Case 18                                     FS+BS+IP+OW

Load Case 19                                     FS+BS+EP+OW

Load Case 20                                        BL+IP+OW

Wind Design Code                               ASCE/SEI 7-16

Wind Load Reduction Scale Factor                       1.000

Basic Wind Speed                     [V]              216.72  Km/hr

Surface Roughness Category                   C: Open Terrain

Type of Surface                            Moderately Smooth

Base Elevation                                           305  mm.

Percent Wind for Hydrotest                              50.0

Using User defined Wind Press. Vs Elev.                    N

Height of Hill or Escarpment     H or Hh                   0  mm.

Distance Upwind of Crest              Lh                   0  mm.

Distance from Crest to the Vessel      x                   0  mm.

Type of Terrain ( Hill, Escarpment )                    Flat

Damping Factor (Beta) for Wind (Ope)                  0.0100

Damping Factor (Beta) for Wind (Empty)                0.0000

Damping Factor (Beta) for Wind (Filled)               0.0000

Site Elevation above Sea Level                          1100  mm.

Seismic Design Code                            ASCE/SEI 7-16

Seismic Load Reduction Scale Factor                    1.000

Importance Factor                                      1.250

Table Value Fa                                         1.400

Table Value Fv                                         2.300

Max. Mapped Res. Acceleration       [Ss]               0.500

Max. Eff. Ground Acceleration        [S]               0.150

Force Modification Factor R                            3.000

Site Class                                                 D

Component Elevation Ratio            z/h               0.000

Amplification Factor                  Ap               0.000

Force Factor                                           0.000

Consider Vertical Acceleration                            No

Minimum Acceleration Multiplier                        0.000

User Value of Sds (used if > 0 )                       0.000

User Value of Sd1 (used if > 0 )                       0.000

Moment Reduction Factor Tau                            1.000

M.A.W.P + Static Head to Nozzle                            Y

Consider MAP New and Cold in Noz. Design                   N

Consider External Loads for Nozzle Des.                    Y

Use ASME VIII-1 Appendix 1-9                               N

Perform Blast Load Analysis                               No

Material Database Year        Current w/Addenda or Code Year

Configuration Directives:

g0 is taken as min( g0, attached shell thk )             Yes

Do not use Nozzle MDMT Interpretation VIII-1 01-37        No

Use Table G instead of exact equation for "A"            Yes

Shell Head Joints are Tapered                            Yes

Compute "K" in corroded condition                        Yes

Use Code Case 2286                                        No

Use the MAWP to compute the MDMT                         Yes

For thickness ratios <= 0.35, MDMT will be -155F (-104C) Yes

For PWHT & P1 Materials the MDMT can be < -55F (-48C)     No

Using Metric Material Databases, ASME II D               Yes

Calculate B31.3 type stress for Nozzles with Loads       Yes

Reduce the MDMT due to lower membrane stress             Yes

Consider Longitudinal Stress in MDMT Calculations        Yes

Complete Listing of Vessel Elements and Details:

Element#                                                 1/4

Element From Node                                         10

Element To Node                                           20

Element Type                                      Elliptical

Description                                      BOTTOM DISH

Distance "FROM" to "TO"                                   50  mm.

Inside Diameter                                         1000  mm.

Element Thickness                                          6  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          8  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                               0.522  bars

Design Temperature Internal Pressure                      85  °C

Design External Pressure                                   0  bars

Design Temperature External Pressure                       0  °C

Effective Diameter Multiplier                            1.2

Material Name                                    SA-240 316L

Allowable Stress, Ambient                            115  N./mm²

Allowable Stress, Operating                        100.5  N./mm²

Allowable Stress, Hydrotest                          153  N./mm²

Material Density                                0.008027  kg./cm³

P Number Thickness                                     0  mm.

Yield Stress, Operating                            150.1  N./mm²

External Pressure Chart Name                        HA-4

UNS Number                                        S31603

Product Form                                       Plate

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Elliptical Head Factor                                   2.0

Weld is pre-Heated                                        No

Element From Node                                     10

Detail Type                                       Liquid

Detail ID                                      Liquid 10

Dist. from "FROM" Node / Offset dist                -250  mm.

Height/Length of Liquid                              300  mm.

Liquid Density                                  0.001038  kg./cm³

Element From Node                                     10

Detail Type                                       Nozzle

Detail ID                                             N8

Dist. from "FROM" Node / Offset dist                   0  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         150

Layout Angle                                         0.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                    45.8  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     10

Detail Type                                       Nozzle

Detail ID                                             N5

Dist. from "FROM" Node / Offset dist                 250  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         150

Layout Angle                                         0.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                    45.8  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

--------------------------------------------------------------------

Element#                                                 2/4

Element From Node                                         20

Element To Node                                           30

Element Type                                        Cylinder

Description                                            SHELL

Distance "FROM" to "TO"                                 1942  mm.

Inside Diameter                                         1000  mm.

Element Thickness                                          6  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          6  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                               0.522  bars

Design Temperature Internal Pressure                      85  °C

Design External Pressure                                   0  bars

Design Temperature External Pressure                       0  °C

Effective Diameter Multiplier                            1.2

Material Name                                    SA-240 316L

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     20

Detail Type                                     Platform

Detail ID                                  PLAT:[1 OF 1]

Dist. from "FROM" Node / Offset dist                1800  mm.

Platform Start Angle (degrees)                       0.0

Platform End Angle   (degrees)                     360.0

Platform Wind Area                                  27.5  cm²

Platform Weight                                   299.52  N

Platform Railing Weight                           0.1961  N/mm.

Platform Grating Weight                            0.005  Kgs/cm²

Platform Width                                        25  mm.

Platform Height                                     1100  mm.

Platform Clearance or End Offset                       0  mm.

Platform Force Coefficient                           0.1

Ladder Layout Angle                                  0.0

Ladder Start Elevation                                 2  mm.

Ladder End Elevation                                2000  mm.

Unit Weight of Ladder                              0.145  N/mm.

Platform Length (top head platform)                    0  mm.

Element From Node                                     20

Detail Type                                       Liquid

Detail ID                                     Liquid: 20

Dist. from "FROM" Node / Offset dist                   0  mm.

Height/Length of Liquid                             1942  mm.

Liquid Density                                  0.001038  kg./cm³

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                            N6A

Dist. from "FROM" Node / Offset dist                1750  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         300

Layout Angle                                        90.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                  45.725  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                            N6B

Dist. from "FROM" Node / Offset dist                 100  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         300

Layout Angle                                        90.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                  45.725  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                            N4A

Dist. from "FROM" Node / Offset dist                1750  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         300

Layout Angle                                       180.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                       0  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                            N4B

Dist. from "FROM" Node / Offset dist                 100  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         300

Layout Angle                                       180.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                  45.725  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                             M1

Dist. from "FROM" Node / Offset dist                 814  mm.

Nozzle Diameter                                      600  mm

Nozzle Schedule                                      10S

Nozzle Class                                         150

Layout Angle                                       270.0

Blind Flange (Y/N)                                     Y

Weight of Nozzle ( Used if > 0 )                  410.73  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                             N1

Dist. from "FROM" Node / Offset dist                1750  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         150

Layout Angle                                       225.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                  53.984  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                             N7

Dist. from "FROM" Node / Offset dist                1750  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         150

Layout Angle                                       135.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                  54.185  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                       Nozzle

Detail ID                                             N9

Dist. from "FROM" Node / Offset dist                   0  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         150

Layout Angle                                        45.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                  54.185  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     20

Detail Type                                          Leg

Detail ID                                           LEGS

Dist. from "FROM" Node / Offset dist                 300  mm.

Diameter at Leg Centerline                        1119.4  mm.

Leg Orientation                                        3

Number of Legs                                         4

Section Identifier                                HE100A

Length of Legs                                      1450  mm.

--------------------------------------------------------------------

Element#                                                 3/4

Element From Node                                         30

Element To Node                                           40

Element Type                                          Flange

Description                                      BODY FLANGE

Distance "FROM" to "TO"                                   16  mm.

Flange Inside Diameter                                  1013  mm.

Element Thickness                                         16  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                         16  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                               0.522  bars

Design Temperature Internal Pressure                      85  °C

Design External Pressure                                   0  bars

Design Temperature External Pressure                       0  °C

Effective Diameter Multiplier                            1.2

Material Name                                    SA-240 316L

Perform Flange Stress Calculation (Y/N)                    Y

Weld is pre-Heated                                        No

--------------------------------------------------------------------

Element#                                                 4/4

Element From Node                                         40

Element To Node                                           50

Element Type                                          Flange

Description                                     COVER FLANGE

Distance "FROM" to "TO"                                   16  mm.

Flange Outside Diameter                                 1130  mm.

Element Thickness                                         16  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                         16  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                               0.522  bars

Design Temperature Internal Pressure                      85  °C

Design External Pressure                                   0  bars

Design Temperature External Pressure                       0  °C

Effective Diameter Multiplier                            1.2

Material Name                                    SA-240 316L

Perform Flange Stress Calculation (Y/N)                    Y

Weld is pre-Heated                                        No

Element From Node                                     40

Detail Type                                       Nozzle

Detail ID                                             N3

Dist. from "FROM" Node / Offset dist                 350  mm.

Nozzle Diameter                                      100  mm

Nozzle Schedule                                      40S

Nozzle Class                                         300

Layout Angle                                       270.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                       0  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     40

Detail Type                                       Nozzle

Detail ID                                             N2

Dist. from "FROM" Node / Offset dist                 350  mm.

Nozzle Diameter                                       50  mm

Nozzle Schedule                                      160

Nozzle Class                                         150

Layout Angle                                       180.0

Blind Flange (Y/N)                                     N

Weight of Nozzle ( Used if > 0 )                       0  N

Grade of Attached Flange                          GR 2.3

Nozzle Matl                                SA-312 TP316L

Element From Node                                     40

Detail Type                                       Weight

Detail ID                                  WGHT:[1 OF 1]

Dist. from "FROM" Node / Offset dist                   0  mm.

Miscellaneous Weight                              3922.4  N

Offset from Element Centerline                         0  mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

XY Coordinate Calculations:

|    |            |            |             |             |

From     | To |  X (Horiz.)|  Y (Vert.) |  DX (Horiz.)|  DY (Vert.) |

|    |        mm. |        mm. |         mm. |         mm. |

----------------------------------------------------------------------

BOTTOM DISH|        ... |         50 |         ... |          50 |

SHELL|        ... |       1992 |         ... |        1942 |

BODY FLANGE|        ... |    1998.18 |         ... |         -16 |

COVER FLANGE|        ... |    2017.35 |         ... |          16 |

----------------------------------------------------------------------

PV Elite includes an 1/8 inch (3.175mm) raised face and gasket thicknesses for girth

flanges and tubesheet thicknesses where applicable in the Tangent to Tangent length calculation.

The calculated dimensions are based on the given element lengths.  Due to variability in

manufacturing (weld gaps etc.), the Tangent to Tangent length may not be exact.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Flange Input Data Values                         Description: BODY FLANGE   :

BODY FLANGE

Flange Type                                    Integral Ring

Design Pressure                            P            0.52  bars

Design Temperature                                        85  °C

Internal Corrosion Allowance              ci          0.0000  mm.

External Corrosion Allowance              ce          0.0000  mm.

Use Corrosion Allowance in Thickness Calcs.               No

Attached Shell Inside Diameter             B       1013.0000  mm.

Integral Ring Inside Diameter                      1025.0000  mm.

Flange Outside Diameter                    A        1130.000  mm.

Flange Thickness                           t         16.0000  mm.

Flange Material                                  SA-240 316L

Flange Material UNS number                            S31603

Flange Allowable Stress At Temperature   Sfo          100.46  N./mm²

Flange Allowable Stress At Ambient       Sfa          115.00  N./mm²

Bolt Material                                     SA-193 B8M

Bolt Allowable Stress At Temperature      Sb          123.57  N./mm²

Bolt Allowable Stress At Ambient          Sa          130.00  N./mm²

Length of Weld Leg at Back of Ring        tw          0.0000  mm.

Diameter of Bolt Circle                    C        1062.000  mm.

Nominal Bolt Diameter                      a         16.0000  mm.

Type of Threads                                  TEMA Metric

Number of Bolts                                           56

Flange Face Outside Diameter             Fod        1130.000  mm.

Flange Face Inside Diameter              Fid        1013.000  mm.

Flange Facing Sketch                       1, Code Sketch 1a

Gasket Outside Diameter                   Go        1130.000  mm.

Gasket Inside Diameter                    Gi        1013.000  mm.

Gasket Factor                              m          3.0000

Gasket Design Seating Stress               y            1.38  N./mm²

Column for Gasket Seating                  2, Code Column II

Gasket Thickness                          tg          3.0000  mm.

ASME Code, Section VIII Division 1, 2023

Distance to Full Face Gasket ID Centroid  hg             12.154  mm.

Distance to Full Face Gasket OD Centroid h'g             17.176  mm.

Corroded Flange ID,      Bcor = B+2*ci                 1013.000  mm.

Corroded Large Hub,     g1Cor = g1-ci                     0.000  mm.

Corroded Small Hub,     g0Cor = go-ci                     0.000  mm.

Code R Dimension,           R = (C-B)/2 - g1             24.500  mm.

Basic Gasket Width,        bo = Full Face                29.250 mm.

Effective Gasket Width,     b = (C-Gi)/4                 12.250 mm.

Gasket Reaction Diameter,   G = C - 2 * HG             1037.693 mm.

The gasket extends beyond the Bolt circle diameter. This flange will be designed as a full

face flange per Taylor Forge bulletin 502.

Basic Flange and Bolt Loads:

Hydrostatic End Load due to Pressure [H]:

= pi/4 * G^2 * Peq

= pi/4 * 1038^2 * 0.52

= 44145.492 N

Contact Load on Gasket Surfaces [Hp]:

= 2 * b * pi * G * m * P

= 2 * 12.3 * pi * 1038 * 3 * 0.52

= 12507.339 N

Full Face ID Gasket Compression [H'p]:

= Hp * hg / h'g

= ( 12507 * 12.2/17.2 )

= 8850.182 N

Full Face ID Pressure Load [H'GY]:

= b * pi * G * y * hg / h'g

= 12.3 * pi * 1038 * 1.38 * 12.2/17.2

= 38992.793 N

Hydrostatic End Load at Flange ID [Hd]:

= pi/4 * Bcor^2 * P

= pi/4 * 1013^2 * 0.52

= 42069.523 N

Pressure Force on Flange Face [Ht]:

= H - Hd

= 44145 - 42070

= 2075.969 N

Operating Bolt Load [Wm1]:

= max( H + Hp + H'p, 0 )

= max( 44145 + 12507 + 8850, 0 )

= 65503.012 N

Gasket Seating Bolt Load [Wm2]:

= y * b * pi * G  + yPart * bPart * lp + H'GY

= 1.38*12.3*pi*1038+0*0*0+38993

= 94098.562 N

Required Bolt Area [Am]:

= max( Wm1/Sb, Wm2/Sa )

= max( 65503/124, 94099/130 )

= 7.239 cm²

ASME Maximum Circumferential Spacing between Bolts per App. 2 eq. (3) [Bsmax]:

= 2a + 6t/(m + 0.5)

= 2 * 16 + 6 * 16/(3 + 0.5)

= 59.429 mm.  Maximum Bolt Spacing Exceeded !

For vessels in lethal service or when specified by the user/agent, the

maximum bolt spacing shall not exceed the value of [Bsmax]. See ASME

VIII-1 Appendix 2 paragraph 2-5(d) for more information.

Actual Circumferential Bolt Spacing [Bs]:

= C * sin( pi / n )

= 1062 * sin( pi/56 )

= 59.547 mm.

ASME Moment Multiplier for Bolt Spacing per App. 2 eq. (7) [Bsc]:

= max( sqrt( Bs/( 2a + t )), 1 )

= max( sqrt( 59.5/( 2 * 16 + 16 )), 1 )

= 1.1138

Bolting Information for TEMA Metric Thread Series (Non Mandatory):

Distance Across Corners for Nuts                      31.180  mm.

Circular Wrench End Diameter                     a    47.000  mm.

------------------------------------------------------------------------

Minimum     Actual    Maximum

------------------------------------------------------------------------

Bolt Area:                                   7.239     77.461

Radial Distance between Hub and Bolts:      19.500     24.500

Radial Distance between Bolts and Edge:     19.500     34.000

Circ. Spacing between the Bolts:            39.700     59.547     59.429

------------------------------------------------------------------------

Flange Design Bolt Load, Gasket Seating [W]:

= Sa * Ab

= 130 * 77.5

= 1006895.00 N

Gasket Load for the Operating Condition [HG]:

= W - H

= 1006895 - 44145

= 962749.50 N

Moment Arm Calculations:

Distance to Gasket Load Reaction [hg]:

= (C - Gi) * (2 * Gi + C) / (6 * (Gi + C))

= (1062 - 1013 ) * (2 * 1013 + 1062 )/

(6 * (1013 + 1062 ))

= 12.1536 mm.

Reverse Moment arm[h"g]:

= hg * h'g / ( hg + h'g )

= 12.2 * 17.2/(12.2 + 17.2 )

= 7.1173 mm.

Distance to Face Pressure Reaction [ht]:

= (R + g1Cor + hg ) / 2.0

= ( 24.5 + 0 + 12.2 )/2.0

= 18.3268 mm.

Distance to End Pressure Reaction [hd]:

= R + giCor

= 24.5 + 0

= 24.5000 mm.

Summary of Moments for Internal Pressure: (N-m)

Loading                 Force  |   Distance |  Bolt Corr |         Moment  |

----------------------------------------------------------------------------

End Pressure,   Md      42070. |    24.5000 |     1.1138 |           1148. |

Face Pressure,  Mt       2076. |    18.3268 |     1.1138 |             42. |

Gasket Load,    Mg          0. |     0.0000 |     1.0000 |              0. |

Gasket Seating, Matm  1006895. |    12.1536 |     1.1138 |          13636. |

Full Face Load, Mr     962750. |     7.1173 |     1.0000 |           6855. |

Total Moment for Operation,      Mop                                 1191. N-m

Total Moment for Gasket seating, Matm                               13636. N-m

Effective Hub Length, ho =                             0.000 mm.

Hub Ratio,          h/h0 = Defined as 0                0.000

Thickness Ratio,   g1/g0 = Defined as 0.0              0.000

Factors from Figure 2-7.1                     K =      1.115

T =      1.872            U =     19.509

Y =     17.753            Z =      9.185

Tangential Flange Stress, Operating [Sto]:

= ( Y * Mop )/( t² * Bcor )

= (17.8*1191)/(16²*1013)

= 81.50 N./mm²

Bolt Stress, Operating [BSo]:

= Wm1 / Ab

= 65503/77.5

= 8.46 N./mm²

Bolt Stress, Seating [BSa]:

= ( Wm2 / Ab )

= (94099/77.5)

= 12.15 N./mm²

Radial Stress at Bolt Circle, Full Face Gaskets, Operating [SFo]:

= 6 *MR/(t²*(Pi*C-n*dB))

= 6*6855/(16²*(3.14*1062-56*16))

= 65.81 N./mm²

Flange Stress Analysis Results: N./mm²

Operating |            Gasket Seating |

Actual       Allowed |      Actual       Allowed |

-----------------------------------------------------------------------------

Tangential Flange           81.50 |      100.46 |        0.00 |      115.00 |

Bolting                      8.46 |      123.57 |       12.15 |      130.00 |

Full Faced Gasket           65.81 |      100.46 |        0.00 |      115.00 |

Minimum Required Flange Thickness                     14.529 mm.

Estimated M.A.W.P. ( Operating )                        0.64 bars

Estimated Finished Weight of Flange at given Thk.       25.3 kg.

Estimated Unfinished Weight of Forging at given Thk     25.3 kg.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

Required Fillet Weld Leg size for Slip on Flanges:

= max( tn, ( 6 mm or 1/4 inch ) )  no hub size given

= max( 6, 6 )

= 6.000 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Flange Input Data Values                         Description: COVER FLANGE  :

COVER FLANGE

Flange Type                                            Blind

Design Pressure                            P            0.52  bars

Design Temperature                                        85  °C

Internal Corrosion Allowance              ci          0.0000  mm.

External Corrosion Allowance              ce          0.0000  mm.

Use Corrosion Allowance in Thickness Calcs.              Yes

Flange Outside Diameter                    A        1130.000  mm.

Flange Thickness                           t         16.0000  mm.

Flange Material                                  SA-240 316L

Flange Material UNS number                            S31603

Flange Allowable Stress At Temperature   Sfo          100.46  N./mm²

Flange Allowable Stress At Ambient       Sfa          115.00  N./mm²

Bolt Material                                     SA-193 B8M

Bolt Allowable Stress At Temperature      Sb          123.57  N./mm²

Bolt Allowable Stress At Ambient          Sa          130.00  N./mm²

Perimeter along the Center of the Bolts    L        3336.372  mm.

Diameter of Bolt Circle                    C        1062.000  mm.

Nominal Bolt Diameter                      a         16.0000  mm.

Type of Threads                                  TEMA Metric

Number of Bolts                                           56

Flange Face Outside Diameter             Fod        1130.000  mm.

Flange Face Inside Diameter              Fid        1013.000  mm.

Flange Facing Sketch                       1, Code Sketch 1a

Gasket Outside Diameter                   Go        1130.000  mm.

Gasket Inside Diameter                    Gi        1013.000  mm.

Gasket Factor                              m          3.0000

Gasket Design Seating Stress               y            1.38  N./mm²

Column for Gasket Seating                  2, Code Column II

Gasket Thickness                          tg          3.0000  mm.

ASME Code, Section VIII Division 1, 2023

Distance to Full Face Gasket ID Centroid  hg             12.154  mm.

Distance to Full Face Gasket OD Centroid h'g             17.176  mm.

Basic Gasket Width,        bo = Full Face                29.250 mm.

Effective Gasket Width,     b = (C-Gi)/4                 12.250 mm.

Gasket Reaction Diameter,   G = C - 2 * HG             1037.693 mm.

The gasket extends beyond the Bolt circle diameter. This flange will be designed as a full

face flange per Taylor Forge bulletin 502.

Basic Flange and Bolt Loads:

Hydrostatic End Load due to Pressure [H]:

= pi/4 * G^2 * Peq

= pi/4 * 1038^2 * 0.52

= 44145.492 N

Contact Load on Gasket Surfaces [Hp]:

= 2 * b * pi * G * m * P

= 2 * 12.3 * pi * 1038 * 3 * 0.52

= 12507.339 N

Full Face ID Gasket Compression [H'p]:

= Hp * hg / h'g

= ( 12507 * 12.2/17.2 )

= 8850.182 N

Full Face ID Pressure Load [H'GY]:

= b * pi * G * y * hg / h'g

= 12.3 * pi * 1038 * 1.38 * 12.2/17.2

= 38992.793 N

Operating Bolt Load [Wm1]:

= max( H + Hp + H'p, 0 )

= max( 44145 + 12507 + 8850, 0 )

= 65503.012 N

Gasket Seating Bolt Load [Wm2]:

= y * b * pi * G  + yPart * bPart * lp + H'GY

= 1.38*12.3*pi*1038+0*0*0+38993

= 94098.562 N

Required Bolt Area [Am]:

= max( Wm1/Sb, Wm2/Sa )

= max( 65503/124, 94099/130 )

= 7.239 cm²

Bolting Information for TEMA Metric Thread Series (Non Mandatory):

------------------------------------------------------------------------

Minimum     Actual    Maximum

------------------------------------------------------------------------

Bolt Area:                                   7.239     77.461

Radial Distance between Bolts and Edge:     19.500     34.000

Circ. Spacing between the Bolts:            39.700     59.547     48.000

------------------------------------------------------------------------

Flange Design Bolt Load, Gasket Seating [W]:

= Sa * Ab

= 130 * 77.5

= 1006895.00 N

Gasket Load for the Operating Condition [HG]:

= W - H

= 1006895 - 44145

= 962749.50 N

Moment Arm Calculations:

Distance to Gasket Load Reaction [hg]:

= (C - Gi) * (2 * Gi + C) / (6 * (Gi + C))

= (1062 - 1013 ) * (2 * 1013 + 1062 )/

(6 * (1013 + 1062 ))

= 12.1536 mm.

Reverse Moment arm[h"g]:

= hg * h'g / ( hg + h'g )

= 12.2 * 17.2/(12.2 + 17.2 )

= 7.1173 mm.

Distance to Face Pressure Reaction [ht]:

= (R + g1Cor + hg ) / 2.0

= ( 24.5 + 0 + 12.2 )/2.0

= 18.3268 mm.

Distance to End Pressure Reaction [hd]:

= R + giCor

= 24.5 + 0

= 24.5000 mm.

Tangential Flange Stress, Flat Head (UG-34), Operating [STo]:

= C * Z * [Peq] * d²/t²  (no bolt load moment for full faced designs)

= 0.25*1*[0.52]*1062²/16²

= 57.50 N./mm²

Bolt Stress, Operating [BSo]:

= Wm1 / Ab

= 65503/77.5

= 8.46 N./mm²

Bolt Stress, Seating [BSa]:

= ( Wm2 / Ab )

= (94099/77.5)

= 12.15 N./mm²

Radial Stress at Bolt Circle, Full Face Gaskets, Operating [SFo]:

= 6 *MR/(t²*(Pi*C-n*dB))

= 6*6855/(16²*(3.14*1062-56*16))

= 65.81 N./mm²

Flange Stress Analysis Results: N./mm²

Operating |            Gasket Seating |

Actual       Allowed |      Actual       Allowed |

-----------------------------------------------------------------------------

Tangential Flange           57.50 |      100.46 |        0.00 |      115.00 |

Bolting                      8.46 |      123.57 |       12.15 |      130.00 |

Full Faced Gasket           65.81 |      100.46 |        0.00 |      115.00 |

Reqd. Blind Flange Thickness at Center                12.105 mm.

Estimated M.A.W.P. ( Operating )                        0.91 bars

Estimated Finished Weight of Flange at given Thk.      128.8 kg.

Estimated Unfinished Weight of Forging at given Thk    128.8 kg.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Internal Pressure Results Summary:

Element Thickness, Pressure, Diameter and Allowable Stress :

|    | Int. Press |    Nominal | Total Corr |    Element |  Allowable | Element# |

From      | To | + Liq. Hd  |  Thickness |  Allowance |   Diameter |  Stress(SE)|          |

|    |       bars |        mm. |        mm. |        mm. |     N./mm² |          |

---------------------------------------------------------------------------------------------

BOTTOM DISH|     0.7505 |          8 |        ... |       1000 |      100.5 |        1 |

SHELL|     0.7199 |          6 |        ... |       1000 |     100.46 |        2 |

BODY FLANGE|      0.522 |         16 |        ... |       1013 |     100.46 |        3 |

COVER FLANGE|      0.522 |         16 |        ... |       1130 |     100.46 |        4 |

---------------------------------------------------------------------------------------------

Element Required Thickness and MAWP :

|    |     Design |   M.A.W.P. |     M.A.P. |    Minimum |   Required | Element# |

From      | To |   Pressure |   Corroded | New & Cold |  Thickness |  Thickness |          |

|    |       bars |       bars |       bars |        mm. |        mm. |          |

---------------------------------------------------------------------------------------------

BOTTOM DISH|      0.522 |       11.8 |       13.8 |          6 |        1.5 |        1 |

SHELL|      0.522 |       11.8 |       13.7 |          6 |        1.5 |        2 |

BODY FLANGE|      0.522 |       0.64 |       0.74 |         16 |    14.5288 |        3 |

COVER FLANGE|      0.522 |       0.91 |       1.04 |         16 |    12.1046 |        4 |

---------------------------------------------------------------------------------------------

Minimum                           0.52         0.74

MAWP: 0.52 bars, limited by: DESIGN (user specified)

Elements Suitable for Design Internal Pressure.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Internal Pressure Calculation Results:

ASME Code, Section VIII Division 1, 2023

Internal Pressure Results for: BOTTOM DISH

|                                               |

Elliptical Head Design Information:              |                                               |

---------------------------------------------------------------------------------------------------

Design Pressure   P           0.750 bars       | Int. Design Temperature            85 °C      |

Material         SA-240 316L            | Ext. Design Temperature             0 °C      |

Length              50.000 mm.        | External Pressure Chart          HA-4         |

|              UNS Number        S31603         |

Int. Corr. All.   c          0.0000 mm.        |  Allowable Stress (ope)  S      100.5 N./mm²  |

Ext. Corr. All.  ce          0.0000 mm.        |  Allowable Stress (amb) Sa      115.0 N./mm²  |

Inside Diameter  Di        1000.000 mm.        |           Actual Stress           6.3 N./mm²  |

Spec. Min. Thk t or e          6.0000 mm.        |      Spec. Nominal Thk.         8.000 mm.     |

Surface Area             12785.4 cm²        |          SG of Contents         1.040         |

Element Volume            170200.1 cm³        |      Weight of Contents        1734.1 N       |

Empty Weight               865.5 N          |        Operating Weight        2599.6 N       |

|  Comp. All. Ope. Stress          76.8 N./mm²  |

|            Aspect Ratio          2.00         |

|                                               |

Radiography Information:                        |                                               |

---------------------------------------------------------------------------------------------------

User Defined                                    |  User Defined                                 |

Circ. Joint Efficiency           100 %          |  Long. Joint Efficiency  E        100 %       |

---------------------------------------------------------------------------------------------------

Tolerance for Formed Heads per UG-81(a):

Head inner surface maximum deviation outside the specified shape,  1.25% of D: 12.500 mm.

Head inner surface maximum deviation inside the specified shape,  0.625% of D: 6.250 mm.

Required Thickness due to Internal Pressure [tr]:

= (P*D*Kcor)/(2*S*E-0.2*P) Appendix 1-4(c)

= (0.75*1000*1)/(2*100*1-0.2*0.75)

= 0.3734 + 0.0000 = 0.3734 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

Less Operating Hydrostatic Head Pressure of 0.228 bars

= (2*S*E*t)/(Kcor*D+0.2*t) per Appendix 1-4 (c)

= (2*100*1*6)/(1*1000+0.2*6)

= 12 - 0.23 = 11.8  bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t)/(K*D+0.2*t) per Appendix 1-4 (c)

= (2*115*1*6)/(1*1000+0.2*6)

= 13.8 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(Kcor*D+0.2*t))/(2*E*t)

= (0.75*(1*1000+0.2*6))/(2*1*6)

= 6.262 N./mm²

Straight Flange Required Thickness:

= (P*R)/(S*E-0.6*P) + c    per UG-27 (c)(1)

= (0.75*500)/(100*1-0.6*0.75)+0

= 0.374 mm.

Straight Flange Maximum Allowable Working Pressure:

Less Operating Hydrostatic Head Pressure of 0.203 bars

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (100 * 1 * 8 )/(500 + 0.6 * 8 )

= 15.9 - 0.2 = 15.7  bars

% Elongation per Table UG-79-1 (75*tnom/Rf)*(1-Rf/Ro)  3.448 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

Tolerance for Formed Heads per UG-81(a):

Head inner surface maximum deviation outside the specified shape,  1.25% of D: 12.500 mm.

Head inner surface maximum deviation inside the specified shape,  0.625% of D: 6.250 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Internal Pressure Calculation Results:

ASME Code, Section VIII Division 1, 2023

Internal Pressure Results for: SHELL

|                                               |

Cylindrical Shell Design Information:            |                                               |

---------------------------------------------------------------------------------------------------

Design Pressure   P           0.720 bars       | Int. Design Temperature            85 °C      |

Material         SA-240 316L            | Ext. Design Temperature             0 °C      |

Length            1942.000 mm.        | External Pressure Chart          HA-4         |

|              UNS Number        S31603         |

Int. Corr. All.   c          0.0000 mm.        |  Allowable Stress (ope)  S      100.5 N./mm²  |

Ext. Corr. All.  ce          0.0000 mm.        |  Allowable Stress (amb) Sa      115.0 N./mm²  |

Inside Diameter  Di        1000.000 mm.        |           Actual Stress           6.0 N./mm²  |

Spec. Min. Thk t or e          6.0000 mm.        |      Spec. Nominal Thk.         6.000 mm.     |

Surface Area             61741.9 cm²        |          SG of Contents         1.040         |

Element Volume           1525516.6 cm³        |      Weight of Contents       15543.1 N       |

Empty Weight              2898.8 N          |        Operating Weight       18441.8 N       |

|  Comp. All. Ope. Stress          76.8 N./mm²  |

|                                               |

Radiography Information:                        |                                               |

---------------------------------------------------------------------------------------------------

User Defined                                    |  User Defined                                 |

Circ. Joint Efficiency           100 %          |  Long. Joint Efficiency  E        100 %       |

---------------------------------------------------------------------------------------------------

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.72*500)/(100*1-0.6*0.72)

= 0.3585 + 0.0000 = 0.3585 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

Less Operating Hydrostatic Head Pressure of 0.198 bars

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (100*1*6)/(500+0.6*6)

= 12 - 0.2 = 11.8  bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (115*1*6)/(500+0.6*6)

= 13.7 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.72*(500+0.6*6))/(1*6)

= 6.043 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.596 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Hydrostatic Test Pressure Results:

UG99(b)          = 1.30 * M.A.W.P. * Sa/S               0.777 bars

UG99b(b)(35)     = 1.30 * Design Pres * Sa/S            0.777 bars

UG99(c)          = 1.30 * M.A.P. - Head(Hyd)            0.956 bars

UG100            = 1.10 * M.A.W.P. * Sa/S               0.657 bars

PED              = max(1.43*DP, 1.25*DP*ratio)          0.747 bars

PED&UG99(b)(35)  = max(1.43*DP, 1.3*DP*ratio)           0.777 bars

App 27-4         = M.A.W.P.                             0.522 bars

UG-99(b), Test Pressure Calculation:

= Test Factor * MAWP * Stress Ratio

= 1.3 * 0.52 * 1.14

= 0.777 bars

Vertical Test performed per: UG-99b

Please note that Nozzle, Shell, Head, Flange, etc MAWPs are all considered

when determining the hydrotest pressure for those test types that are based

on the MAWP of the vessel.

Stresses on Elements due to Test Pressure (N./mm² & bars):

From To              |    Stress |  Allowable |       Ratio | Pressure |

------------------------------------------------------------------------

BOTTOM DISH          |       8.3 |      153.0 |       0.054 |     1.00 |

SHELL                |       8.1 |      153.0 |       0.053 |     0.97 |

------------------------------------------------------------------------

Theoretical Maximum Allowable test pressure is : 18.116  bars

Stress ratios for Nozzle and Pad Materials (N./mm²):

Description          | Pad/Nozzle |   Ambient |    Operating |   Ratio |

------------------------------------------------------------------------

N8                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N5                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N6A                  |     Nozzle |    115.00 |       100.50 |   1.144 |

N6B                  |     Nozzle |    115.00 |       100.50 |   1.144 |

N4A                  |     Nozzle |    115.00 |       100.50 |   1.144 |

N4B                  |     Nozzle |    115.00 |       100.50 |   1.144 |

M1                   |     Nozzle |    115.00 |       100.50 |   1.144 |

M1                   |        Pad |    115.00 |       100.50 |   1.144 |

N1                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N1                   |        Pad |    115.00 |       100.50 |   1.144 |

N7                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N7                   |        Pad |    115.00 |       100.46 |   1.145 |

N9                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N3                   |     Nozzle |    115.00 |       100.46 |   1.145 |

N3                   |        Pad |    115.00 |       100.50 |   1.144 |

N2                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N2                   |        Pad |    115.00 |       100.50 |   1.144 |

------------------------------------------------------------------------

Minimum                                                          1.144

Stress ratios for Pressurized Vessel Elements (N./mm²):

Description                       |   Ambient |    Operating |   Ratio |

------------------------------------------------------------------------

BOTTOM DISH                       |    115.00 |       100.50 |   1.144 |

SHELL                             |    115.00 |       100.46 |   1.145 |

BODY FLANGE                       |    115.00 |       100.46 |   1.145 |

COVER FLANGE                      |    115.00 |       100.46 |   1.145 |

------------------------------------------------------------------------

Minimum                                                          1.144

Hoop Stress in Nozzle Wall during Pressure Test (N./mm²):

Description                       |   Ambient |    Operating |   Ratio |

------------------------------------------------------------------------

N8                                |      0.30 |       153.00 |   0.002 |

N5                                |      0.30 |       153.00 |   0.002 |

N6A                               |      0.24 |       153.00 |   0.002 |

N6B                               |      0.29 |       153.00 |   0.002 |

N4A                               |      0.24 |       153.00 |   0.002 |

N4B                               |      0.29 |       153.00 |   0.002 |

M1                                |      4.23 |       153.00 |   0.028 |

N1                                |      0.24 |       153.00 |   0.002 |

N7                                |      0.24 |       153.00 |   0.002 |

N9                                |      0.30 |       153.00 |   0.002 |

N3                                |      0.71 |       153.00 |   0.005 |

N2                                |      0.24 |       153.00 |   0.002 |

------------------------------------------------------------------------

Elements Suitable for Test Pressure.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Hydrostatic Test Pressure Results:

UG99(b)          = 1.30 * M.A.W.P. * Sa/S               0.777 bars

UG99b(b)(35)     = 1.30 * Design Pres * Sa/S            0.777 bars

UG99(c)          = 1.30 * M.A.P. - Head(Hyd)            0.956 bars

UG100            = 1.10 * M.A.W.P. * Sa/S               0.657 bars

PED              = max(1.43*DP, 1.25*DP*ratio)          0.747 bars

PED&UG99(b)(35)  = max(1.43*DP, 1.3*DP*ratio)           0.777 bars

App 27-4         = M.A.W.P.                             0.522 bars

UG-99(b) Note 35, Test Pressure Calculation:

= Test Factor * Design Pressure * Stress Ratio

= 1.3 * 0.52 * 1.14

= 0.777 bars

Vertical Test performed per: UG-99b (Note 35)

Please note that Nozzle, Shell, Head, Flange, etc MAWPs are all considered

when determining the hydrotest pressure for those test types that are based

on the MAWP of the vessel.

Stresses on Elements due to Test Pressure (N./mm² & bars):

From To              |    Stress |  Allowable |       Ratio | Pressure |

------------------------------------------------------------------------

BOTTOM DISH          |       8.3 |      153.0 |       0.054 |     1.00 |

SHELL                |       8.1 |      153.0 |       0.053 |     0.97 |

------------------------------------------------------------------------

Theoretical Maximum Allowable test pressure is : 18.116  bars

Stress ratios for Nozzle and Pad Materials (N./mm²):

Description          | Pad/Nozzle |   Ambient |    Operating |   Ratio |

------------------------------------------------------------------------

N8                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N5                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N6A                  |     Nozzle |    115.00 |       100.50 |   1.144 |

N6B                  |     Nozzle |    115.00 |       100.50 |   1.144 |

N4A                  |     Nozzle |    115.00 |       100.50 |   1.144 |

N4B                  |     Nozzle |    115.00 |       100.50 |   1.144 |

M1                   |     Nozzle |    115.00 |       100.50 |   1.144 |

M1                   |        Pad |    115.00 |       100.50 |   1.144 |

N1                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N1                   |        Pad |    115.00 |       100.50 |   1.144 |

N7                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N7                   |        Pad |    115.00 |       100.46 |   1.145 |

N9                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N3                   |     Nozzle |    115.00 |       100.46 |   1.145 |

N3                   |        Pad |    115.00 |       100.50 |   1.144 |

N2                   |     Nozzle |    115.00 |       100.50 |   1.144 |

N2                   |        Pad |    115.00 |       100.50 |   1.144 |

------------------------------------------------------------------------

Minimum                                                          1.144

Stress ratios for Pressurized Vessel Elements (N./mm²):

Description                       |   Ambient |    Operating |   Ratio |

------------------------------------------------------------------------

BOTTOM DISH                       |    115.00 |       100.50 |   1.144 |

SHELL                             |    115.00 |       100.46 |   1.145 |

BODY FLANGE                       |    115.00 |       100.46 |   1.145 |

COVER FLANGE                      |    115.00 |       100.46 |   1.145 |

------------------------------------------------------------------------

Minimum                                                          1.144

Hoop Stress in Nozzle Wall during Pressure Test (N./mm²):

Description                       |   Ambient |    Operating |   Ratio |

------------------------------------------------------------------------

N8                                |      0.30 |       153.00 |   0.002 |

N5                                |      0.30 |       153.00 |   0.002 |

N6A                               |      0.24 |       153.00 |   0.002 |

N6B                               |      0.29 |       153.00 |   0.002 |

N4A                               |      0.24 |       153.00 |   0.002 |

N4B                               |      0.29 |       153.00 |   0.002 |

M1                                |      4.23 |       153.00 |   0.028 |

N1                                |      0.24 |       153.00 |   0.002 |

N7                                |      0.24 |       153.00 |   0.002 |

N9                                |      0.30 |       153.00 |   0.002 |

N3                                |      0.71 |       153.00 |   0.005 |

N2                                |      0.24 |       153.00 |   0.002 |

------------------------------------------------------------------------

Elements Suitable for Test Pressure.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

External Pressure Results Summary:

External Pressure Calculation Results:

ASME Code, Section VIII Division 1, 2023

External Pressure Calculations:

|      |    Section |    Outside |  Corroded |    Factor  |     Factor |

From|   To |     Length |   Diameter | Thickness |       A    |        B   |

|      |        mm. |        mm. |       mm. |            |     N./mm² |

-----------------------------------------------------------------------------

10|    20|    No Calc |       1012 |         6 | 0.00082345 |     61.875 |

20|    30|    2075.33 |       1012 |         6 | 0.00029041 |    28.0332 |

30|    40|    No Calc |        ... |        16 |    No Calc |    No Calc |

40|    50|    No Calc |        ... |        16 |    No Calc |    No Calc |

-----------------------------------------------------------------------------

External Pressure Calculations:

|      |   External |   External |        External |    External |

From|   To |  Actual T. | Required T.| Design Pressure |    M.A.W.P. |

|      |        mm. |        mm. |            bars |        bars |

-----------------------------------------------------------------------

10|    20|          6 |    No Calc |             ... |        4.08 |

20|    30|          6 |    No Calc |             ... |        2.22 |

30|    40|         16 |    12.3952 |             ... |     No Calc |

40|    50|         16 |    12.3952 |             ... |     No Calc |

-----------------------------------------------------------------------

Minimum                                                       2.22

External Pressure Calculations:

|      |   Actual Length | Allowable Length | Ring Inertia | Ring Inertia |

From|   To | Bet. Stiffeners |  Bet. Stiffeners |     Required |    Available |

|      |             mm. |              mm. |         cm^4 |         cm^4 |

--------------------------------------------------------------------------------

10|    20|         No Calc |          No Calc |      No Calc |      No Calc |

20|    30|         2075.33 |          No Calc |      No Calc |      No Calc |

30|    40|         No Calc |          No Calc |      No Calc |      No Calc |

40|    50|         No Calc |          No Calc |      No Calc |      No Calc |

--------------------------------------------------------------------------------

Elements Suitable for External Pressure.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Element and Detail Weights:

|    |    Element |    Element |  Corroded |   Corroded | Extra due  |

From| To | Metal Wgt. | ID  Volume |Metal Wgt. |  ID Volume | Misc %     |

|    |        kg. |        cm³ |       kg. |        cm³ |        kg. |

---------------------------------------------------------------------------

10|  20|    88.2606 |     170200 |   88.2606 |     170200 |    8.82606 |

20|  30|     295.61 |    1525516 |    295.61 |    1525516 |     29.561 |

30|  40|    25.2923 |        ... |   25.2923 |        ... |    2.52923 |

40|  50|    128.806 |        ... |   128.806 |        ... |    12.8806 |

---------------------------------------------------------------------------

Total |        537 | 1695716.75 |       537 | 1695716.75 |         53 |

Weight of Details:

|    |  Weight of |  X Offset, | Y Offset, |    Z Offset |

From|Type|     Detail | Dtl. Cent. |Dtl. Cent. |  Dtl. Cent. | Description

|    |        kg. |        mm. |       mm. |         mm. |

---------------------------------------------------------------

10|Liqd|    176.843 |        ... |      -125 |         ... |   Liquid 10

10|Nozl|    5.13765 |        ... |  -372.006 |         ... |   N8

10|Nozl|    5.13765 |        ... |  -338.513 |        -250 |   N5

20|Plat|    33.5987 |        ... |      1800 |         ... |   PLAT:[1 OF 1]

20|Liqd|    1585.06 |        ... |       971 |         ... |   Liquid: 20

20|Nozl|    5.12923 |    575.006 |      1750 |         ... |   N6A

20|Nozl|    5.12923 |    575.006 |       100 |         ... |   N6B

20|Nozl|    7.07625 |        ... |      1750 |     575.006 |   N4A

20|Nozl|    5.12923 |        ... |       100 |     575.006 |   N4B

20|Nozl|    46.0741 |   -575.006 |       814 |         ... |   M1

20|Nozl|    6.05576 |   -406.591 |      1750 |     406.591 |   N1

20|Nozl|    6.07823 |    406.591 |      1750 |     406.591 |   N7

20|Nozl|    6.07823 |    406.591 |       ... |    -406.591 |   N9

20|Legs|     195.43 |        ... |      -425 |         ... |   LEGS

40|Nozl|      19.41 |       -350 |   91.0062 |         ... |   N3

40|Nozl|    6.80982 |        ... |   91.0062 |         350 |   N2

40|Wght|        400 |        ... |       ... |         ... |   WGHT:[1 OF 1]

---------------------------------------------------------------

Total Weight of Each Detail Type:

Platforms                          33.6

Liquid                           1761.9

Nozzles                           123.2

Legs                              195.4

Weights                           400.0

-------------------------------------------

Sum of the Detail Weights        2514.2 kg.

Weight Summation Results: (kg.)

| Fabricated |  Shop Test |   Shipping |    Erected |      Empty |  Operating |

---------------------------------------------------------------------------------------------

Main Elements |      591.8 |      591.8 |      591.8 |      591.8 |      591.8 |      591.8 |

Nozzles |      123.2 |      123.2 |      123.2 |      123.2 |      123.2 |      123.2 |

Legs |      195.4 |      195.4 |      195.4 |      195.4 |      195.4 |      195.4 |

Platforms |        ... |        ... |       33.6 |       33.6 |       33.6 |       33.6 |

Empty Weights |        ... |        ... |        ... |      400.0 |      400.0 |        ... |

Test Weights |        ... |      400.0 |        ... |        ... |        ... |        ... |

Ope Weights |        ... |        ... |        ... |        ... |        ... |      400.0 |

Ope. Liquid |        ... |        ... |        ... |        ... |        ... |     1761.9 |

Test Liquid |        ... |     1694.7 |        ... |        ... |        ... |        ... |

---------------------------------------------------------------------------------------------

Totals |      910.4 |     3005.1 |      944.0 |     1344.0 |     1344.0 |     3105.9 |

Field Installation Options:

Miscellaneous Weight Percent: 10.0 %

Note that the above value for the miscellaneous weight percent has

been applied to the shells/heads/flange/tubesheets/tubes etc. in the

weight calculations for metallic components.

Weight Summary:

Fabricated Wt.  - Bare Weight without Removable Internals                   910.4 kg.

Shop Test Wt.   - Fabricated Weight + Water ( Full )                       3005.1 kg.

Shipping Wt.    - Fab. Weight + removable Intls.+ Shipping App.             944.0 kg.

Erected  Wt.    - Fab. Wt + or - loose items (trays,platforms etc.)        1344.0 kg.

Ope. Wt. no Liq - Fab. Weight + Internals. + Details + Weights             1344.0 kg.

Operating Wt.   - Empty Weight + Operating Liq. Uncorroded                 3105.9 kg.

Field Test Wt.  - Empty Weight + Water (Full)                              3038.7 kg.

Weights used in Vortex Shedding Calculations:

Mass of the Upper 1/3 of the Vertical Vessel (operating)                   1295.3 kg.

Mass of the Upper 1/3 of the Vertical Vessel (filled w/water)              1166.8 kg.

Mass of the Upper 1/3 of the Vertical Vessel (empty)                        767.2 kg.

Field Test weight is computed in the corroded condition.

Surface Areas of Elements:

|    | Outside Surface | Inside Surface |

From| To |            Area |           Area |

|    |             cm² |            cm² |

----------------------------------------------

10|  20|         12785.4 |        12410.6 |

20|  30|         61741.9 |        61009.7 |

30|  40|         2537.24 |        509.189 |

40|  50|         10596.7 |            ... |

----------------------------------------------

Total             87661.3          73929.6 cm²

Total                94.4             79.6 ft²

Element and Detail Weights:

| To |  Total Ele.| Total. Ele.|Total. Ele.|  Total Dtl.| Oper. Wgt. |

From| To |  Empty Wgt.|  Oper. Wgt.|Hydro. Wgt.| Offset Mom.| No Liquid  |

|    |        kg. |        kg. |       kg. |        N-m |        kg. |

---------------------------------------------------------------------------

10|  20|    107.362 |    284.205 |   277.458 |        ... |    107.362 |

20|Legs|    68.8239 |    313.683 |   304.342 |    177.695 |    68.8239 |

Legs|  30|    376.696 |    1716.89 |   1665.76 |        ... |    376.696 |

30|  40|    27.8215 |    27.8215 |   27.8215 |        ... |    27.8215 |

40|  50|    567.907 |    567.907 |   567.907 |     66.644 |    567.907 |

---------------------------------------------------------------------------

Cumulative Vessel Weight

|    | Cumulative Ope | Cumulative | Cumulative  |

From| To | Wgt. No Liquid | Oper. Wgt. | Hydro. Wgt. |

|    |            kg. |        kg. |         kg. |

-------------------------------------------------------

10|  20|            ... |        ... |         ... |

20|Legs|       -107.362 |   -284.205 |    -277.458 |

Legs|  30|        972.424 |    2312.62 |     2261.49 |

30|  40|        595.728 |    595.728 |     595.728 |

40|  50|        567.907 |    567.907 |     567.907 |

-------------------------------------------------------

The cumulative operating weights no liquid in the column above are the cumulative operating

weights minus the operating liquid weight minus any weights absent in the empty condition.

Cumulative Vessel Moment

|    | Cumulative | Cumulative |Cumulative |

From| To | Empty Mom. | Oper. Mom. |Hydro. Mom.|

|    |        N-m |        N-m |       N-m |

-------------------------------------------------

10|  20|        ... |        ... |       ... |

20|Legs|    177.695 |    177.695 |   177.695 |

Legs|  30|     66.644 |     66.644 |    66.644 |

30|  40|     66.644 |     66.644 |    66.644 |

40|  50|     66.644 |     66.644 |    66.644 |

-------------------------------------------------

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Nozzle Flange MAWP Results: (bars & °C)

Nozzle        |    Flange Rating | Des. |       |                | Equiv. | - - - -  Max Pressure |

Description   |   Ope. | Ambient | Temp | Class |    Grade/Group |  Press | UG-44 |   50% |   DNV |

---------------------------------------------------------------------------------------------------

N8            |  13.90 |   15.90 |   85 |   150 |         GR 2.3 |    ... |   ... |   ... |   ... |

N5            |  13.90 |   15.90 |   85 |   150 |         GR 2.3 |    ... |   ... |   ... |   ... |

N6A           |  36.36 |   41.40 |   85 |   300 |         GR 2.3 |    ... |   ... |   ... |   ... |

N6B           |  36.36 |   41.40 |   85 |   300 |         GR 2.3 |    ... |   ... |   ... |   ... |

N4A           |  36.36 |   41.40 |   85 |   300 |         GR 2.3 |    ... |   ... |   ... |   ... |

N4B           |  36.36 |   41.40 |   85 |   300 |         GR 2.3 |    ... |   ... |   ... |   ... |

M1            |  13.90 |   15.90 |   85 |   150 |         GR 2.3 |    ... |   ... |   ... |   ... |

N1            |  13.90 |   15.90 |   85 |   150 |         GR 2.3 |    ... |   ... |   ... |   ... |

N7            |  13.90 |   15.90 |   85 |   150 |         GR 2.3 |    ... |   ... |   ... |   ... |

N9            |  13.90 |   15.90 |   85 |   150 |         GR 2.3 |    ... |   ... |   ... |   ... |

N3            |  36.36 |   41.40 |   85 |   300 |         GR 2.3 |    ... |   ... |   ... |   ... |

N2            |  13.90 |   15.90 |   85 |   150 |         GR 2.3 |    ... |   ... |   ... |   ... |

---------------------------------------------------------------------------------------------------

Min Rating    | 13.900 |  15.900, [for Core Elements]                     |   ... |   ... |   ... |

Flange Pressure Ratings are per ASME B16.5 2020 Metric Edition

Warning:

There are nozzles in this model, but no flange MAWPs were de-rated.  Be

sure this is what you intended.  There is a check box in the nozzle dialog

that instructs PV Elite to perform the de-rating for each nozzle flange.

See ASME VIII-1, UG-44(b) for more information.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

The Natural Frequencies for the vessel have been computed iteratively

by solving a system of matrices.  These matrices describe the mass

and the stiffness of the vessel.  This is the generalized eigenvalue/

eigenvector problem and is referenced in some mathematical texts.

The Natural Frequency for the Vessel (Empty.) is 27.5043 Hz.

The Natural Frequency for the Vessel (Ope...) is 19.6087 Hz.

The Natural Frequency for the Vessel (Filled) is 19.8164 Hz.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Wind Load Calculation Results:

Input Values:

Wind Design Code                               ASCE/SEI 7-16

Wind Load Reduction Scale Factor                       1.000

Basic Wind Speed                     [V]              216.72  Km/hr

Surface Roughness Category                   C: Open Terrain

Type of Surface                            Moderately Smooth

Base Elevation                                           305  mm.

Percent Wind for Hydrotest                              50.0

Using User defined Wind Press. Vs Elev.                    N

Height of Hill or Escarpment     H or Hh                   0  mm.

Distance Upwind of Crest              Lh                   0  mm.

Distance from Crest to the Vessel      x                   0  mm.

Type of Terrain ( Hill, Escarpment )                    Flat

Damping Factor (Beta) for Wind (Ope)                  0.0100

Damping Factor (Beta) for Wind (Empty)                0.0000

Damping Factor (Beta) for Wind (Filled)               0.0000

Site Elevation above Sea Level                          1100  mm.

Wind Analysis Results

The importance factor is not used in the calculation of wind pressure vs. elevation for

the selected wind code.

Static Gust-Effect Factor, Operating Case [G]:

= min(0.85, 0.925((1 + 1.7 * gQ * Izbar * Q )/( 1 + 1.7 * gV * Izbar)))

= min(0.85,0.925((1+1.7*3.4*0.23*0.97)/(1+1.7*3.4*0.23)))

= min(0.85, 0.91 )

= 0.850

Natural Frequency of Vessel (Operating)               19.609 Hz

Natural Frequency of Vessel (Empty)                   27.504 Hz

Natural Frequency of Vessel (Test)                    19.816 Hz

User Entered Importance Factor is                     1.000

Force Coefficient                                [Cf] 0.516

Structure Height to Diameter ratio                    1.985

Height to top of Structure                            2017.350 mm.

This is classified as a rigid structure. Static analysis performed.

Sample Calculation for the First Element:

The ASCE code performs calculations in Imperial Units. The wind pressure is therefore computed

in these units.

Value of [Alpha] and [Zg]:

Exposure Category: C from Table 26.11-1

Alpha = 9.5 : Zg = 274320 mm.

Effective Height [z]:

= Centroid Height + Vessel Base Elevation

= 174 + 305 = 479 mm.

= 1.57 ft. Imperial Units

Velocity Pressure coefficient evaluated at height z [Kz]:

Because z (1.57 ft.) < 15 ft.

= 2.01( 15 / Zg )2/Alpha

= 2.01( 15/900 )2/9.5

= 0.85

Type of Hill: No Hill

Wind Directionality Factor [Kd]:

= 1, per Table 26.6-1

As there is No Hill Present: [Kzt]:

K1 = 0, K2 = 0, K3 = 0

Topographical Factor [Kzt]:

= ( 1 + K1 * K2 * K3 )²

= ( 1 + 0 * 0 * 0 )²

= 1

Velocity Pressure evaluated at height z, Imperial Units [qz]:

= max( 16, 0.00256 * Kz * Kzt * Kd * Ke * V(mph)² )

= max( 16, 0.00256 * 0.85 * 1 * 1 * 1 * 135² )

= 39.4 psf [192 ] Kgs/m²

Force on the first element [F]:

= qz * G * Cf * WindArea

= 39.4 * 0.85 * 0.52 * 3.28

= 56.8 lbs. [252 ] N

Element        Hgt (z)       K1      K2      K3      Kz    Kzt           qz

mm.                                              Kgs/m²

---------------------------------------------------------------------------

BOTTOM DISH      479.0    0.000   0.000   0.000   0.849   1.000     192.394

SHELL           1576.0    0.000   0.000   0.000   0.849   1.000     192.394

BODY FLANGE     2545.2    0.000   0.000   0.000   0.849   1.000     192.394

COVER FLANGE    2564.4    0.000   0.000   0.000   0.849   1.000     192.394

Wind Vibration Calculations

This evaluation is based on work by Kanti Mahajan and Ed Zorilla

Nomenclature

Cf  - Correction factor for natural frequency

D  - Average outer insulated diameter of vessel, mm.

Df  - Damping Factor < 0.75 Unstable,  > 0.95 Stable

Dr  - Average outer insulated of top half of vessel, mm.

f  - Natural frequency of vibration (Hertz)

f1  - Natural frequency of bare vessel based on a unit value of (D/L²)(104)

L  - Total height of structure mm.

Lc  - Total length of conical section(s) of vessel, mm.

tb  - Uncorroded plate thickness at bottom of vessel, mm.

V30 - Design Wind Speed provided by user, Km/hr

Vc  - Critical wind velocity, Km/hr

Vw  - Maximum wind speed at top of structure, Km/hr

W   - Total corroded weight of structure, N

Ws  - Cor. vessel weight excl. weight of parts which do not effect stiff. N

Z  - Maximum amplitude of vibration at top of vessel, mm.

Dl  - Logarithmic decrement ( taken as 0.03 for Welded Structures )

Vp  - Vib. Chance, <= 0.31416E-05 (High); 0.31416E-05 < 0.39270E-05 (Probable)

P30 - wind pressure 30 feet above the base

Check other Conditions and Basic Assumptions:

:: Total Cone Length / Total Length < 0.5

:: 0/2017 = 0

:: [ D / L² ] * 104 < 8 (English Units, ft.)

:: [ 3.33/6.62² ] * 104 = 761 [Geometry Violation]

Compute the vibration possibility. If Vp > 0.39270E-05 no chance. [Vp]:

= W / ( L * Dr²)

= 30457/( 2017 * 1015² )

= 0.14658E-04

Since Vp is > 0.39270E-05 no further vibration analysis is required !

Platform Load Calculations

ID              Wind Area   Elevation    Pressure       Force          Cf

cm²        mm.          Kgs/m²        N

-------------------------------------------------------------------------

PLAT:[1 OF 1]       27.50     2155.00      192.39        5.19        0.10

Wind Loads on Masses/Equipment/Piping

ID              Wind Area   Elevation    Pressure       Force

cm²        mm.          Kgs/m²        N

-------------------------------------------------------------------------

WGHT:[1 OF 1]        0.00     2306.35      192.39        0.00

The Natural Frequency for the Vessel (Ope...) is 19.6087 Hz.

Wind Load Calculation:

|    |       Wind |       Wind |       Wind |       Wind |    Element |

From| To |     Height |   Diameter |       Area |   Pressure |  Wind Load |

|    |        mm. |        mm. |        cm² |     Kgs/m² |          N |

----------------------------------------------------------------------------

10|  20|    478.967 |     1214.4 |     3048.9 |    192.394 |    252.492 |

20|  30|       1576 |     1214.4 |    23583.6 |    192.394 |    1960.15 |

30|  40|    2545.18 |     1215.6 |    194.496 |    192.394 |     16.107 |

40|  50|    2564.35 |       1356 |     216.96 |    192.394 |    17.9673 |

----------------------------------------------------------------------------

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Earthquake Load Calculation Results:

Earthquake Load Calculation:

Input Values:

Seismic Design Code                            ASCE/SEI 7-16

Seismic Load Reduction Scale Factor                    1.000

Importance Factor                                      1.250

Table Value Fa                                         1.400

Table Value Fv                                         2.300

Max. Mapped Res. Acceleration       [Ss]               0.500

Max. Eff. Ground Acceleration        [S]               0.150

Force Modification Factor R                            3.000

Site Class                                                 D

Component Elevation Ratio            z/h               0.000

Amplification Factor                  Ap               0.000

Force Factor                                           0.000

Consider Vertical Acceleration                            No

Minimum Acceleration Multiplier                        0.000

User Value of Sds (used if > 0 )                       0.000

User Value of Sd1 (used if > 0 )                       0.000

Moment Reduction Factor Tau                            1.000

Seismic Analysis Results:

SMS = Fa * Ss = 1.4 * 0.5 = 0.7

SM1 = Fv * S1 = 2.3 * 0.15 = 0.34

SDS = 2/3 * Sms = 2/3 * 0.7 = 0.47

SD1 = 2/3 * Sm1 = 2/3 * 0.34 = 0.23

Check Approximate Fundamental Period from 12.8-7 [Ta]:

= Ct * hnx where Ct = 0.020, x = 0.75 and hn = Structural Height (ft.)

= 0.020( 10.10.75)

= 0.114 seconds

The Coefficient Cu from Table 12.8-1 is : 1.500

Fundamental Period (1/Frequency) [T]:

= ( 1/Natural Frequency ) = ( 1/19.6 )

= 0.051

Check the Value of T which is the smaller of Cu*Ta and T:

= Minimum Value of ( 1.5 * 0.11, 0.051 ) per 12.8.2

= 0.051

As the time period is < 0.06 second, use section 15.4.2.

Base Shear per equation 15.4-5, [V]:

= 0.3 * Sds * W * I

= 0.3 * 0.47 * 28540 * 1.25

= 4994.577 N

Distribute the Base shear force to each element according to the equations

Fx = Cvx * V (eqn. 12.8-11 ) and the vertical distribution factor

Cvx = Wx*hxk/( Sum of Wi*hik ) and k is an exponent which is related

to the period of Vibration.

In this case, the value of k was 1

The Natural Frequency for the Vessel (Ope...) is 19.6087 Hz.

Earthquake Load Calculation:

|    | Earthquake | Earthquake |   Element |    Element |       Element |

From| To |     Height |     Weight | Ope  Load |   Emp Load | Vertical Load |

|    |        mm. |          N |         N |          N |             N |

------------------------------------------------------------------------------

10|  20|         25 |    2786.91 |    10.675 |    3.17779 |           ... |

20|Legs|        350 |    3075.98 |   164.951 |    28.5195 |           ... |

Legs|  30|       1171 |    16835.8 |   3020.63 |    522.255 |           ... |

30|  40|    2006.18 |    272.817 |   83.8583 |    66.0821 |           ... |

40|  50|    2009.35 |    5568.89 |   1714.47 |    1351.04 |           ... |

------------------------------------------------------------------------------

The Earthquake Weight does not include the weight of the saddles.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

The following table is for the Operating Case.

Wind/Earthquake Shear, Bending:

|    | Distance to| Cumulative |Earthquake |       Wind | Earthquake |

From| To |     Support| Wind Shear |     Shear |    Bending |    Bending |

|    |        mm. |          N |         N |        N-m |        N-m |

---------------------------------------------------------------------------

10|  20|    432.033 |        ... |       ... |        ... |        ... |

20|Legs|        150 |    252.492 |    10.675 |    33.3507 |    1.41002 |

Legs|  30|        821 |    2246.72 |   4994.58 |    1262.92 |    5435.23 |

30|  40|    1640.18 |    34.0744 |   1798.33 |    0.61737 |    47.2808 |

40|  50|    1659.35 |    17.9673 |   1714.47 |     0.1438 |    13.7215 |

---------------------------------------------------------------------------

Note:

The Wind Shears/Moments and the Earthquake Shears/Moments calculated

and printed in the Wind/Earthquake Shear and Bending report have

been factored by the input Scalar/Load reductions factors of;

Wind: 1.000;  Earthquake: 1.000.

Note:

Review the Vessel Design Summary for the cumulative shear force

and bending moment on the support.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Wind Deflection Calculations:

The following table is for Wind Deflection; Operating Case.

Wind Load Deflection:

|    | Cumulative |   Centroid |     Elem. End |    Elem. Ang. |

From| To |      Shear | Deflection |    Deflection |     Rotation  |

|    |          N |        mm. |           mm. |               |

---------------------------------------------------------------------

10|  20|        ... |    1.16193 |       1.16193 |     0.0012012 |

20|Legs|    252.492 |    1.16197 |       1.16208 |     0.0012022 |

Legs|  30|    2246.72 |    1.16388 |        1.1665 |     0.0012044 |

30|  40|    34.0744 |    1.16647 |       1.16644 |     0.0012044 |

40|  50|    17.9673 |    1.16647 |        1.1665 |     0.0012044 |

---------------------------------------------------------------------

Critical Wind Velocity for Tower Vibration:

|    |  1st Crit. |  2nd Crit. |

From| To | Wind Speed | Wind Speed |

|    |      Km/hr |      Km/hr |

-------------------------------------

10|  20|    427.475 |    2671.72 |

20|  30|    427.475 |    2671.72 |

30|  40|    427.898 |    2674.36 |

40|  50|    477.319 |    2983.24 |

-------------------------------------

Allowable deflection at the Tower Top (Ope)(6.000"/100ft. Criteria)

Allowable deflection : 10.087    Actual deflection : 1.166 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Longitudinal Stress Constants:

|    |     Metal Area |     Metal Area | Section Modulus | Section Modulus |

From| To |            New |       Corroded |             New |        Corroded |

|    |            cm² |            cm² |            mm.³ |            mm.³ |

---------------------------------------------------------------------------------

10|  20|        189.627 |        189.627 |         4741015 |         4741015 |

20|  30|        189.627 |        189.627 |         4741015 |         4741015 |

30|  40|        189.627 |        189.627 |         4741015 |         4741015 |

40|  50|        189.627 |        189.627 |         4741015 |         4741015 |

---------------------------------------------------------------------------------

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Longitudinal Allowable Stresses:

|    |             |   Hydrotest |             |   Hydrotest |

From| To |     Tensile |     Tensile | Compressive | Compressive |

|    |      N./mm² |      N./mm² |      N./mm² |      N./mm² |

-------------------------------------------------------------------

10|  20|       120.6 |       183.6 |    -76.8303 |    -84.6665 |

20|Legs|     120.549 |     183.597 |    -76.8303 |    -84.6665 |

Legs|  30|     120.549 |     183.597 |    -76.8303 |    -84.6665 |

30|  40|     120.549 |     183.597 |    -76.8303 |    -84.6665 |

40|  50|     120.549 |     183.597 |    -76.8303 |    -84.6665 |

-------------------------------------------------------------------

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Longitudinal Stress Report

Longitudinal Operating and Empty Stresses are computed in the

corroded condition. Stresses due to loads in the hydrostatic test

cases have also been computed in the corroded condition.

Longitudinal Pressure Stresses due to:

|    | Longitudinal Stress | Longitudinal Stress |Longitudinal Stress |

From| To |   Internal Pressure |  External  Pressure | Hydrotest Pressure |

|    |              N./mm² |              N./mm² |             N./mm² |

----------------------------------------------------------------------------

10|  20|             2.16469 |                 ... |             3.2201 |

20|  30|             2.16469 |                 ... |             3.2201 |

30|  40|                 ... |                 ... |                ... |

40|  50|                 ... |                 ... |                ... |

----------------------------------------------------------------------------

Longitudinal Stresses due to Weight Loads for these Conditions:

|    | Wght. Str. | Wght. Str. |Wght. Str. | Wght. Str. | Wght. Str. |

From| To |     Empty  | Operating  |Hydrotest  |  Emp. Mom. |  Opr. Mom. |

|    |     N./mm² |     N./mm² |    N./mm² |     N./mm² |     N./mm² |

---------------------------------------------------------------------------

10|  20|        ... |        ... |       ... |        ... |        ... |

20|Legs|   0.055524 |    0.14698 |   0.14349 |   0.037468 |   0.037468 |

Legs|  30|    -0.5029 |   -0.37627 |   -0.5029 |   0.014052 |   0.014052 |

30|  40|   -0.30809 |   -0.30809 |  -0.30809 |   0.014052 |   0.014052 |

40|  50|    -0.2937 |    -0.2937 |   -0.2937 |   0.014052 |   0.014052 |

---------------------------------------------------------------------------

Longitudinal Stresses due to Weight Loads and Bending for these Conditions:

|    | Wght. Str. |  Bend. Str. |  Bend. Str. |  Bend. Str. | Bend. Str. |

From| To |  Hyd. Mom. |  Oper. Wind |  Oper. Equ. |  Hyd. Wind  |  Hyd. Equ. |

|    |     N./mm² |      N./mm² |      N./mm² |      N./mm² |     N./mm² |

-------------------------------------------------------------------------------

10|  20|        ... |         ... |         ... |         ... |        ... |

20|Legs|   0.037468 |   0.0070323 |  0.00029731 |   0.0035161 |        ... |

Legs|  30|   0.014052 |      0.2663 |     1.14606 |     0.13315 |        ... |

30|  40|   0.014052 |  0.00013018 |   0.0099695 | 0.65089E-04 |        ... |

40|  50|   0.014052 | 0.30321E-04 |   0.0028933 | 0.15161E-04 |        ... |

-------------------------------------------------------------------------------

Longitudinal Stresses due to these Conditions:

|    | Vortex Shedding | Vortex Shedding | Vortex Shedding | Earthquake |

From| To |  Operating Case |     Empty  Case |       Test Case | Empty Case |

|    |          N./mm² |          N./mm² |          N./mm² |     N./mm² |

------------------------------------------------------------------------------

10|  20|             ... |             ... |             ... |        ... |

20|Legs|             ... |             ... |             ... |0.88506E-04 |

Legs|  30|             ... |             ... |             ... |    0.58502 |

30|  40|             ... |             ... |             ... |  0.0078562 |

40|  50|             ... |             ... |             ... |    0.00228 |

------------------------------------------------------------------------------

Longitudinal Stresses due to Applied Axial Forces:

|    | Longitudinal Stress | Longitudinal Stress |

From| To |       Y Forces Wind |    Y Forces Seismic |

|    |              N./mm² |              N./mm² |

-------------------------------------------------------

10|  20|                 ... |                 ... |

20|Legs|                 ... |                 ... |

Legs|  30|                 ... |                 ... |

30|  40|                 ... |                 ... |

40|  50|                 ... |                 ... |

-------------------------------------------------------

Longitudinal Stresses due to User Forces and Moments:

|    |Wind For/Mom | Earthquake For/Mom |  Wind For/Mom | Earthquake For/Mom |

From| To |    Corroded |           Corroded |  No Corrosion |       No Corrosion |

|    |      N./mm² |             N./mm² |        N./mm² |             N./mm² |

-----------------------------------------------------------------------------------

10|  20|         ... |                ... |           ... |                ... |

20|Legs|         ... |                ... |           ... |                ... |

Legs|  30|         ... |                ... |           ... |                ... |

30|  40|         ... |                ... |           ... |                ... |

40|  50|         ... |                ... |           ... |                ... |

-----------------------------------------------------------------------------------

Bending Stresses due to Blast Load:

|    |    Bending |

From| To |     Stress |

|    |     N./mm² |

------------------------

10|  20|        ... |

20|Legs|        ... |

Legs|  30|        ... |

30|  40|        ... |

40|  50|        ... |

------------------------

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Stress Combination Load Cases for Vertical Vessels:

Load Case Definition Key

IP = Longitudinal Stress due to Internal  Pressure

EP = Longitudinal Stress due to External  Pressure

HP = Longitudinal Stress due to Hydrotest Pressure

NP = No Pressure

EW = Longitudinal Stress due to Weight (No Liquid)

OW = Longitudinal Stress due to Weight (Operating)

HW = Longitudinal Stress due to Weight (Hydrotest)

WI = Bending Stress due to Wind Moment (Operating)

EQ = Bending Stress due to Earthquake Moment (Operating)

EE = Bending Stress due to Earthquake Moment (Empty)

HI = Bending Stress due to Wind Moment (Hydrotest)

HE = Bending Stress due to Earthquake Moment (Hydrotest)

WE = Bending Stress due to Wind Moment (Empty) (no CA)

WF = Bending Stress due to Wind Moment (Filled) (no CA)

CW = Longitudinal Stress due to Weight (Empty) (no CA)

VO = Bending Stress due to Vortex Shedding Loads ( Ope )

VE = Bending Stress due to Vortex Shedding Loads ( Emp )

VF = Bending Stress due to Vortex Shedding Loads ( Test No CA. )

FW = Axial Stress due to Vertical Forces for the Wind Case

FS = Axial Stress due to Vertical Forces for the Seismic Case

BW = Bending Stress due to Lat. Forces for the Wind Case, Corroded

BS = Bending Stress due to Lat. Forces for the Seismic Case, Corroded

BN = Bending Stress due to Lat. Forces for the Wind Case, UnCorroded

BU = Bending Stress due to Lat. Forces for the Seismic Case, UnCorroded

BL = Bending Stress due to Blast Loads, Corroded

General Notes:

Case types HI and HE are in the Corroded condition.

Case types WE, WF, and CW are in the Un-Corroded condition.

A blank stress and stress ratio indicates that the corresponding stress comprising

those components that did not contribute to that type of stress.

An asterisk (*) in the final column denotes overstress.

Analysis of Load Case 1 : NP+EW+WI+FW+BW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        120.60      0.00           76.83   0.0000   0.0000

20       0.10        120.55                     76.83   0.0008

20                   120.55     -0.78           76.83            0.0102

Analysis of Load Case 2 : NP+EW+EE+FS+BS

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        120.60      0.00           76.83   0.0000   0.0000

20       0.09        120.55                     76.83   0.0008

20       0.10        120.55     -1.10           76.83   0.0008   0.0143

Analysis of Load Case 3 : NP+OW+WI+FW+BW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        120.60      0.00           76.83   0.0000   0.0000

20       0.19        120.55                     76.83   0.0016

20                   120.55     -0.66           76.83            0.0085

Analysis of Load Case 4 : NP+OW+EQ+FS+BS

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        120.60      0.00           76.83   0.0000   0.0000

20       0.18        120.55                     76.83   0.0015

20       0.78        120.55     -1.54           76.83   0.0065   0.0200

Analysis of Load Case 5 : NP+HW+HI

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        183.60      0.00           84.67   0.0000   0.0000

20       0.18        183.60                     84.67   0.0010

20                   183.60     -0.65           84.67            0.0077

Analysis of Load Case 6 : NP+HW+HE

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        183.60      0.00           84.67   0.0000   0.0000

20       0.18        183.60                     84.67   0.0010

20                   183.60     -0.52           84.67            0.0061

Analysis of Load Case 7 : IP+OW+WI+FW+BW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        120.60                     76.83   0.0179

20       2.36        120.55                     76.83   0.0195

20       2.07        120.55                     76.83   0.0172

Analysis of Load Case 8 : IP+OW+EQ+FS+BS

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        120.60                     76.83   0.0179

20       2.35        120.55                     76.83   0.0195

20       2.95        120.55                     76.83   0.0245

Analysis of Load Case 9 : EP+OW+WI+FW+BW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        120.60      0.00           84.67   0.0000   0.0000

20       0.19        120.55                     84.67   0.0016

20                   120.55     -0.66           84.67            0.0078

Analysis of Load Case 10 : EP+OW+EQ+FS+BS

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        120.60      0.00           84.67   0.0000   0.0000

20       0.18        120.55                     84.67   0.0015

20       0.78        120.55     -1.54           84.67   0.0065   0.0181

Analysis of Load Case 11 : HP+HW+HI

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       3.22        183.60                     84.67   0.0175

20       3.40        183.60                     84.67   0.0185

20       2.86        183.60                     84.67   0.0156

Analysis of Load Case 12 : HP+HW+HE

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       3.22        183.60                     84.67   0.0175

20       3.40        183.60                     84.67   0.0185

20       2.73        183.60                     84.67   0.0149

Analysis of Load Case 13 : IP+WE+EW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        120.60                     76.83   0.0179

20       2.26        120.55                     76.83   0.0187

20       1.68        120.55                     76.83   0.0139

Analysis of Load Case 14 : IP+WF+CW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        120.60                     76.83   0.0179

20       2.31        120.55                     76.83   0.0192

20       1.79        120.55                     76.83   0.0148

Analysis of Load Case 15 : IP+VO+OW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        120.60                     76.83   0.0179

20       2.35        120.55                     76.83   0.0195

20       1.80        120.55                     76.83   0.0150

Analysis of Load Case 16 : IP+VE+EW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        120.60                     76.83   0.0179

20       2.26        120.55                     76.83   0.0187

20       1.68        120.55                     76.83   0.0139

Analysis of Load Case 17 : NP+VO+OW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        120.60      0.00           76.83   0.0000   0.0000

20       0.18        120.55                     76.83   0.0015

20                   120.55     -0.39           76.83            0.0051

Analysis of Load Case 18 : FS+BS+IP+OW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        100.50                     64.03   0.0215

20       2.35        100.46                     64.03   0.0234

20       1.80        100.46                     64.03   0.0179

Analysis of Load Case 19 : FS+BS+EP+OW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       0.00        100.50      0.00           70.56   0.0000   0.0000

20       0.18        100.46                     70.56   0.0018

20                   100.46     -0.39           70.56            0.0055

Analysis of Load Case 20 : BL+IP+OW

From     Tensile    All. Tens.     Comp.      All. Comp.    Tens.    Comp.

Node      Stress     Stress       Stress       Stress       Ratio    Ratio

--------------------------------------------------------------------------

10       2.16        120.60                     76.83   0.0179

20       2.35        120.55                     76.83   0.0195

20       1.80        120.55                     76.83   0.0150

Absolute Maximum of the all of the Stress Ratio's                0.0245

Governing Element: SHELL

Governing Load Case 8 : IP+OW+EQ+FS+BS

|      Long |  Weight + | Eccentric |   Wind or |           |         |

Element |  Pressure |Axial Load |    Moment |Earthquake |  Combined |   Which |

Description |    Stress |    Stress |    Stress |    Stress |    Stress |    Side |

---------------------------------------------------------------------------------------------

BOTTOM DISH |       2.2 |       ... |       ... |       ... |       2.2 | +  Tens |

BOTTOM DISH |       2.2 |       ... |       ... |       ... |       ... | -  Comp |

---------------------------------------------------------------------------------------------

SHELL |       2.2 |       0.1 |       ... |       ... |       2.3 | +  Tens |

SHELL |       2.2 |       0.1 |      -0.0 |      -0.0 |       ... | -  Comp |

---------------------------------------------------------------------------------------------

SHELL |       2.2 |      -0.4 |       ... |       1.1 |       2.9 | +  Tens |

SHELL |       2.2 |      -0.4 |      -0.0 |      -1.1 |       ... | -  Comp |

---------------------------------------------------------------------------------------------

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Shop/Field Installation Options :

Platform(s) installed in the Shop.

Note : The CG is computed from the first Element From Node

Center of Gravity of the Platforms                         1850.000 mm.

Center of Gravity of the Liquid                             905.975 mm.

Center of Gravity of the Nozzles                           1108.947 mm.

Center of Gravity of the Legs                              -375.000 mm.

Center of Gravity of the Added Weights (Operating)         2001.350 mm.

Center of Gravity of the Added Weights (Empty)             2001.350 mm.

Center of Gravity of Bare Shell New and Cold               1118.467 mm.

Center of Gravity of Bare Shell Corroded                   1118.467 mm.

Vessel CG in the Operating Condition                       1025.195 mm.

Vessel CG in the Fabricated (Shop/Empty) Condition         1181.479 mm.

Vessel CG in the Test Condition                            1028.950 mm.

Rigging Analysis Results:

• Lifting weight based on the user selected calculated empty weight

Total Effective Length of Vessel for this analysis          2008.00 mm.

Total vessel weight (No Liquid)                 Twt        13179.65 N

Impact weight multiplication factor             Imp            2.00

Design lifting weight,  DWT = Imp * Twt                    26359.30 N

Elevation of the Tailing Lug (bottom)                          0.00 mm.

Elevation of the Lifting Lug (top   )                       1850.00 mm.

Design Reaction force at the tailing lug                    9525.27 N

Design Reaction force at the lifting lug                   16834.03 N

CG distance during field lifting operation                  1181.48 mm.

CG distance from tailing lug                                1181.48 mm.

CG distance from the nearer lifting lug/trunnion             668.52 mm.

Compressive Allowable Increase Factor                          1.00

Critical Values: (N./mm², mm.)

| Max Stress|   Elevation   |   Allowables           |

-----------------------------------------------------------------

Bending    |      0.67 |     826.80    |      70.56    (UG-23)  |

Shear      |      0.45 |       0.00    |      80.50    (0.7*S)  |

The compressive bending allowable stress is the minimum of the tensile allowable and

the compressive allowable times the specified compressive allowable increase factor.

Forces and Moments at selected elevations (not all analysis points shown):

Distance     Bending Moment   Bending Stress   Shear Force    Shear Stress

mm.                N-m           N./mm²             N          N./mm²

-----------------------------------------------------------------------------

0.00                0.0              0.0        8580.6             0.5

438.40             2097.9              0.4        4434.5             0.2

1603.60             1314.1              0.3        5779.4             0.3

1992.00               15.4              0.0        1921.4             0.1

2001.60                2.5              0.0         768.6             0.0

Unity Check (Actual Stress / Allowable Stress):

Maximum Unity Check is 0.0094 at elevation 827 mm. - Must be <=1

Note: The rigging analysis is performed using a uniformly distributed load.

--- Plot data successfully generated ...----

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Lifting Lug Calculations:

Input Values:

Lifting Lug Material                                    SA-240 316L

Lifting Lug Yield Stress                           Yield     172.00 N./mm²

Total Height of Lifting Lug                            w   150.0000 mm.

Thickness of Lifting Lug                               t    22.0000 mm.

Diameter of Hole in Lifting Lug                       dh    50.0000 mm.

Radius of Semi-Circular Arc of Lifting Lug             r    75.0000 mm.

Height of Lug from bottom to Center of Hole            h    75.0000 mm.

Offset from Vessel OD to Center of Hole              off   130.0000 mm.

Lug Fillet Weld Size                                  tw     8.0000 mm.

Length of weld along side of Lifting Lug              wl   150.0000 mm.

Length of Weld along Bottom of Lifting Lug            wb    20.0000 mm.

Thickness of Collar (if any)                          tc     0.0000 mm.

Diameter of Collar (if any)                           dc     0.0000 mm.

Impact Factor                                     Impfac       2.00

Sling Angle from Horizontal                                    90.0 deg

Number of Lugs in Group                                           2

Reinforcing Pad Circumferential Width                wpb     130.00 mm.

Reinforcing Pad Longitudinal Length                  wpl     270.00 mm.

Reinforcing Pad Thickness                           thkp       8.00 mm.

Reinforcing Pad Fillet Weld Size                     tfp       5.00 mm.

Lifting Lug Orientation to Vessel: Perpendicular

Lift Orientation : From Horizontal to Vertical

Lifting Lug and Weld Stresses at various Angles: N./mm²

--------------- Weld Stresses   ------ Lug Stresses   Maximum

Angle    Primary  Secondary  Allowable   Combined  Allowable     Ratio

----------------------------------------------------------------------

0       58.5        ...       68.8       90.4      113.5     0.851

5       61.0        ...       68.8       92.7      113.5     0.886

10       63.0        ...       68.8       94.2      113.5     0.915

15       64.5        ...       68.8       94.9      113.5     0.938

20       65.6        ...       68.8       94.9      113.5     0.954

25       66.3        ...       68.8       94.1      113.5     0.963

30       66.4        ...       68.8       92.6      113.5     0.966

35       66.1        ...       68.8       90.3      113.5     0.961

40       65.4        ...       68.8       87.2      113.5     0.951

45       64.2        ...       68.8       83.5      113.5     0.933

50       62.6        ...       68.8       79.1      113.5     0.910

55       60.5        ...       68.8       74.0      113.5     0.880

60       58.1        ...       68.8       68.3      113.5     0.844

65       55.3        ...       68.8       61.9      113.5     0.803

70       52.1        ...       68.8       55.0      113.5     0.757

75       48.6        ...       68.8       47.5      113.5     0.707

80       44.9        ...       68.8       39.3      113.5     0.653

85       41.1        ...       68.8       30.5      113.5     0.597

90       37.2        ...       68.8       20.8      113.5     0.541

----------------------------------------------------------------------

Computed Results:

Total vessel weight (No Liquid)                            13179.65 N

Design Reaction force at the tailing lug                    9525.27 N

Design Reaction force at the lifting lug                   16834.03 N

Worst Case Lift Angle for Primary Weld Stresses                  29 deg

Tangential Force at this Angle                               7997.9 N

Axial Force at this Angle                                    4433.3 N

Worst Case Lift Angle for Secondary Weld Stresses                 0 deg

Tangential Force at this Angle                               8417.0 N

Axial Force at this Angle                                       0.0 N

Worst Case Lift Angle for Lug Stresses                           17 deg

Tangential Force at this Angle                               8461.1 N

Axial Force at this Angle                                    2586.8 N

Converting the weld leg dimension (tw) to the weld throat dimension.

Weld Group Inertia Calculations:

Weld Group Inertia about the Circumferential Axis    Ilc      455.248 cm^4

Weld Group Centroid distance in the Long. Direction  Yll       80.656 mm.

Dist. of Weld Group Centroid from Lug bottom       Yll_b       75.000 mm.

Weld Group Inertia about the Longitudinal Axis       Ill       33.652 cm^4

Weld Group Centroid Distance in the Circ. Direction  Ylc       16.656 mm.

Note: The Impact Factor is applied to the Forces acting on the Lug.

Primary Shear Stress in the Welds due to Shear Loads [Ssll]:

= sqrt( Fax2 + Ft2 + Fn2 )/(( 2 * (wl + wb) ) * tw )

= sqrt(44332+79982+02)/((2*(150+20))*5.66)

= 4.76 N./mm²

Shear Stress in the Welds due to Bending Loads [Sblf]:

= (Fn(h-Yll_b))Yll/Ilc + (Fax * off * Yll/Ilc) + (Ft * off * Ylc/Ill)

= (0 (75 -75 ))80.7/455 +

(4433 * 130 * 80.7/455 ) +

(7998 * 130 * 16.7/33.7 )

= 61.68 N./mm²

Total Shear Stress for Combined Loads [St]:

= Ssll + Sblf

= 4.76 + 61.7

= 66.43 N./mm²

Allowable Shear Stress for Combined Loads [Sta]:

= 0.4 * Yield * Occfac (AISC Shear Allowable)

= 0.4 * 172 * 1

= 68.80 N./mm²

Shear Stress in Lug above Hole [Shs]:

= sqrt( Pl2 + Fax2 ) / Sha

= sqrt( 131802 + 44332 )/22

= 6.32 N./mm²

Allowable Shear Stress in Lug above Hole [Sas]:

= 0.4 * Yield * Occfac

= 0.4 * 172 * 1

= 68.80 N./mm²

Pin Hole Bearing Width [Pbw]:

= Lug Thickness + 2 * Collar Thickness

= 22 + 2 * 0

= 22.000 mm.

Pin Hole Bearing Stress [Pbs]:

= sqrt( Pl2 + Faxial2 ) / ( Pbw * dh )

= sqrt( 131802 + 44332 )/( 22 * 50 )

= 12.64 N./mm²

Allowable Bearing Stress [Pba]:

= min( 0.75 * Yield * Occfac, 0.9 * Yield ) AISC Bearing All.

= min( 0.75 * 172 * 1, 155 )

= 129.00 N./mm²

Bending Stress at the Base of the Lug [Fbs]:

= Ft * off/(w * t2/6) + Fax * off/(w2 * t/6)

= 8461 * 130/(150 * 222/6) +

2587 * 130/(1502 * 22/6)

= 92.92 N./mm²

Tensile Stress at the Base of the Lug [Fa]:

= Fn / (w * t)

= 0/(150 * 22 )

= 0.00 N./mm²

Total Combined Stress at the Base of the Lug:

= Fbs + Fa

= 92.9 + 0

= 92.92 N./mm²

Lug Allowable Stress for Bending and Tension:

= min( 0.66 * Yield * Occfac, 0.75 * Yield )

= min( 0.66 * 172 * 1, 129 )

= 113.52 N./mm²

Required Shackle Pin Diameter [Spd]:

= sqrt((2(Fn2 + Fax2)0.5/( pi * Sta)))

= sqrt(2(131802 + 44332)0.5/( pi * 68.8 ))

= 11.3438 mm.

WRC 107/537 Stress Analysis for the Lifting Lug to Shell Junction in

the new and Cold Condition (no corrosion applied).

Note: The full lug load is applied in both the circumferential and

longitudinal directions simultaneously.

The ratio Beta1/Beta2 for rectangular attachments was outside the range of values in tables 7 & 8

and was set to be an acceptable (maximum or minimum) value. This generally results in one

dimension being altered so that the aspect ratio does not exceed 4 to 1.

Attachment dimensions (C11P, C22P etc.) may be modified accounting for attachment

welds where applicable.

Input Echo, WRC107/537 Item     1,       Description: Lift Lug

Diameter Basis for Vessel                 Vbasis      ID

Cylindrical or Spherical Vessel           Cylsph      Cylindrical

Internal Corrosion Allowance                 Cas      0.0000  mm.

Vessel Diameter                               Dv    1000.000  mm.

Vessel Thickness                              Tv       6.000  mm.

Note: Lifting lug calculations are performed at ambient temperature conditions.

Attachment Type                             Type      Rectangular

Parameter C11                                C11       22.00  mm.

Parameter C22                                C22       88.00  mm.

Thickness of Reinforcing Pad                Tpad       8.000  mm.

Pad Parameter C11P                          C11p     140.000  mm.

Pad Parameter C22P                          C22p     275.000  mm.

Design Internal Pressure                      Dp       0.000  bars

Include Pressure Thrust                                   No

External Forces and Moments in WRC 107/537 Convention:

Radial Load             (SUS)                  P         0.0  N

Longitudinal Shear      (SUS)                 Vl     13179.6  N

Circumferential Shear   (SUS)                 Vc     13179.6  N

Circumferential Moment  (SUS)                 Mc      1714.0  N-m

Longitudinal Moment     (SUS)                 Ml         0.0  N-m

Torsional Moment        (SUS)                 Mt         0.0  N-m

Use Interactive Control                           No

WRC107 Version                           Version  March  1979

Include Pressure Stress Indices per Div. 2                No

Compute Pressure Stress per WRC-368                       No

Local Loads applied at end of Nozzle/Attachment           No

WRC Bulletin 537 provides equations for the dimensionless curves found in bulletin 107.  As noted

in the foreword to bulletin 537, "537 is equivalent to WRC 107".  Where 107 is printed in the

results below, "537" can be interchanged with "107".

WRC 107/537 has dimensional limitations that can be found in the bulletin's Foreword

and which can also be inferred by review of the non-dimensional curve limits.

WRC 107 Stress Calculation for SUStained loads:

Radial Load                                   P          0.0  N

Circumferential Shear                        VC      13179.6  N

Longitudinal Shear                           VL      13179.6  N

Circumferential Moment                       MC       1714.0  N-m

Longitudinal Moment                          ML          0.0  N-m

Torsional Moment                             MT          0.0  N-m

Dimensionless Parameters used :   Gamma = 36.21

Dimensionless Loads for Cylindrical Shells at Attachment Junction:

-------------------------------------------------------------------

Curves read for 1979              Beta  Figure    Value   Location

-------------------------------------------------------------------

N(PHI) / ( P/Rm )                0.064   4C        6.684   (A,B)

N(PHI) / ( P/Rm )                0.064   3C        6.440   (C,D)

M(PHI) / ( P )                   0.038   2C1       0.175   (A,B)

M(PHI) / ( P )                   0.038   1C        0.221   (C,D)

N(PHI) / ( MC/(Rm**2 * Beta) )   0.034   3A        0.277   (A,B,C,D)

M(PHI) / ( MC/(Rm    * Beta) )   0.044   1A        0.104   (A,B,C,D)

N(PHI) / ( ML/(Rm**2 * Beta) )   0.055   3B        1.960   (A,B,C,D)

M(PHI) / ( ML/(Rm    * Beta) )   0.048   1B        0.059   (A,B,C,D)

N(x)   / ( P/Rm )                0.052   3C        6.720   (A,B)

N(x)   / ( P/Rm )                0.052   4C        6.792   (C,D)

M(x)   / ( P )                   0.054   1C1       0.189   (A,B)

M(x)   / ( P )                   0.054   2C        0.145   (C,D)

N(x)   / ( MC/(Rm**2 * Beta) )   0.034   4A        0.342   (A,B,C,D)

M(x)   / ( MC/(Rm    * Beta) )   0.059   2A        0.061   (A,B,C,D)

N(x)   / ( ML/(Rm**2 * Beta) )   0.055   4B        0.540   (A,B,C,D)

M(x)   / ( ML/(Rm    * Beta) )   0.066   2B        0.090   (A,B,C,D)

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|     -1.0|     -1.0|      1.0|      1.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|   -246.3|    246.3|    246.3|   -246.3|

Circ. Memb. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|      0.0|      0.0|      0.0|      0.0|   -247.3|    245.4|    247.3|   -245.4|

------------------------------------------------------------------------------------------------

Long. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|     -2.2|     -2.2|      2.2|      2.2|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|   -107.3|    107.3|    107.3|   -107.3|

Long. Memb. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|      0.0|      0.0|      0.0|      0.0|   -109.6|    105.1|    109.6|   -105.1|

------------------------------------------------------------------------------------------------

Shear VC |     21.4|     21.4|    -21.4|    -21.4|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -5.3|     -5.3|      5.3|      5.3|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|     21.4|     21.4|    -21.4|    -21.4|     -5.3|     -5.3|      5.3|      5.3|

------------------------------------------------------------------------------------------------

Str. Int. |     42.8|     42.8|     42.8|     42.8|    247.5|    245.6|    247.5|    245.6|

Dimensionless Parameters used :   Gamma = 83.83

Dimensionless Loads for Cylindrical Shells at Pad Edge:

-------------------------------------------------------------------

Curves read for 1979              Beta  Figure    Value   Location

-------------------------------------------------------------------

N(PHI) / ( P/Rm )                0.256   4C        8.093   (A,B)

N(PHI) / ( P/Rm )                0.256   3C        3.332   (C,D)

M(PHI) / ( P )                   0.180   2C1       0.024   (A,B)

M(PHI) / ( P )                   0.180   1C  !     0.067   (C,D)

N(PHI) / ( MC/(Rm**2 * Beta) )   0.174   3A        3.173   (A,B,C,D)

M(PHI) / ( MC/(Rm    * Beta) )   0.184   1A        0.064   (A,B,C,D)

N(PHI) / ( ML/(Rm**2 * Beta) )   0.218   3B        6.112   (A,B,C,D)

M(PHI) / ( ML/(Rm    * Beta) )   0.197   1B        0.015   (A,B,C,D)

N(x)   / ( P/Rm )                0.221   3C        4.172   (A,B)

N(x)   / ( P/Rm )                0.221   4C        9.087   (C,D)

M(x)   / ( P )                   0.227   1C1       0.038   (A,B)

M(x)   / ( P )                   0.227   2C  !     0.036   (C,D)

N(x)   / ( MC/(Rm**2 * Beta) )   0.174   4A        7.066   (A,B,C,D)

M(x)   / ( MC/(Rm    * Beta) )   0.222   2A        0.025   (A,B,C,D)

N(x)   / ( ML/(Rm**2 * Beta) )   0.218   4B        3.003   (A,B,C,D)

M(x)   / ( ML/(Rm    * Beta) )   0.235   2B        0.014   (A,B,C,D)

Note - The ! mark next to the figure name denotes curve value exceeded.

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Edge of Reinforcing Pad (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|    -11.0|    -11.0|     11.0|     11.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|   -199.1|    199.1|    199.1|   -199.1|

Circ. Memb. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|      0.0|      0.0|      0.0|      0.0|   -210.1|    188.1|    210.1|   -188.1|

------------------------------------------------------------------------------------------------

Long. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|    -34.7|    -34.7|     34.7|     34.7|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|    -62.7|     62.7|     62.7|    -62.7|

Long. Memb. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|      0.0|      0.0|      0.0|      0.0|    -97.5|     28.0|     97.5|    -28.0|

------------------------------------------------------------------------------------------------

Shear VC |      7.8|      7.8|     -7.8|     -7.8|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -4.0|     -4.0|      4.0|      4.0|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      7.8|      7.8|     -7.8|     -7.8|     -4.0|     -4.0|      4.0|      4.0|

------------------------------------------------------------------------------------------------

Str. Int. |     15.7|     15.7|     15.7|     15.7|    210.2|    188.2|    210.2|    188.2|

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Pl (SUS) |      0.0|      0.0|      0.0|      0.0|     -1.0|     -1.0|      1.0|      1.0|

Circ. Q  (SUS) |      0.0|      0.0|      0.0|      0.0|   -246.3|    246.3|    246.3|   -246.3|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Pl (SUS) |      0.0|      0.0|      0.0|      0.0|     -2.2|     -2.2|      2.2|      2.2|

Long. Q  (SUS) |      0.0|      0.0|      0.0|      0.0|   -107.3|    107.3|    107.3|   -107.3|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |     21.4|     21.4|    -21.4|    -21.4|     -5.3|     -5.3|      5.3|      5.3|

Shear Q  (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Pm (SUS)       |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |     42.8|     42.8|     42.8|     42.8|     10.8|     10.8|     10.8|     10.8|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|     42.8|     42.8|     42.8|     42.8|    247.5|    245.6|    247.5|    245.6|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          0.00            115.00      |      Passed     |

Pm+Pl (SUS)    |         42.79            172.50      |      Passed     |

Pm+Pl+Q (TOTAL)|        247.49            344.99      |      Passed     |

The Pm+Pl+Q allowable above was based on the rules of ASME VIII-2 Part 4, 4.1.6.3 (a)&(b) or

per ASME VIII-2, 5.5.6.1(1)&(2) Part 5 as applicable.

The Pm+Pl+Q allowable was based on a temperature range cycling from ambient to design temperature.

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Reinforcing Pad Edge (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Pl (SUS) |      0.0|      0.0|      0.0|      0.0|    -11.0|    -11.0|     11.0|     11.0|

Circ. Q  (SUS) |      0.0|      0.0|      0.0|      0.0|   -199.1|    199.1|    199.1|   -199.1|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Pl (SUS) |      0.0|      0.0|      0.0|      0.0|    -34.7|    -34.7|     34.7|     34.7|

Long. Q  (SUS) |      0.0|      0.0|      0.0|      0.0|    -62.7|     62.7|     62.7|    -62.7|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |      7.8|      7.8|     -7.8|     -7.8|     -4.0|     -4.0|      4.0|      4.0|

Shear Q  (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Pm (SUS)       |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |     15.7|     15.7|     15.7|     15.7|     35.4|     35.4|     35.4|     35.4|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|     15.7|     15.7|     15.7|     15.7|    210.2|    188.2|    210.2|    188.2|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          0.00            115.00      |      Passed     |

Pm+Pl (SUS)    |         35.40            172.50      |      Passed     |

Pm+Pl+Q (TOTAL)|        210.19            344.99      |      Passed     |

The Pm+Pl+Q allowable above was based on the rules of ASME VIII-2 Part 4, 4.1.6.3 (a)&(b) or

per ASME VIII-2, 5.5.6.1(1)&(2) Part 5 as applicable.

The Pm+Pl+Q allowable was based on a temperature range cycling from ambient to design temperature.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

RESULTS FOR LEGS :  Operating Case   Description:  LEGS

Legs attached to: SHELL

Section Properties :  I Beam HE100A

European Structural Steel Data

Overall Leg Length                                  1450.000  mm.

Effective Leg Length                        Leglen  1200.000  mm.

Distance Leg Up Side of Vessel                       300.000  mm.

Number of Legs                                Nleg     4

Cross Sectional Area for HE100A               Aleg    21.200  cm²

Section Inertia    ( strong axis )                   348.998  cm^4

Section Inertia    ( weak   axis )                   133.999  cm^4

Section Modulus    ( strong axis )                 72800.297  mm.³

Section Modulus    ( weak   axis )                 26800.107  mm.³

Radius of Gyration ( strong axis )                    40.600  mm.

Radius of Gyration ( weak   axis )                    25.100  mm.

Leg Orientation - Weak Axis

Overturning Moment at top of Legs                     5435.2  N-m

Total Weight Load at top of Legs                 W   28714.7  N

Total Shear force at top of Legs                      4994.6  N

Additional force in Leg due to Bracing        Fadd       0.0  N

Occasional Load Factor                      Occfac     1.333

Effective Leg End Condition Factor               k     1.000

Note: The Legs are Not Cross Braced

The Leg Shear Force includes Wind, Seismic, and Applied Load Effects.

Pad Width along Circumference                 C11P   200.000  mm.

Pad Length along Vessel Axis                  C22P   200.000  mm.

Pad Thickness                                 Tpad     6.000  mm.

Maximum Shear at top of one Leg [Vleg]:

= ( max( Wind, Seismic, Blast ) + applied forces )( Imax / Itot )

= ( 4995 )( 349/966 )

= 1806.74 N

Axial Compression, Leg furthest from the Neutral Axis [Sma]:

= W/Nleg + (Mleg/(Nlegm*Rn))/Aleg

= 28716/4 + (5433061/( 2 * 560 ))/2120

= 5.68 N./mm²

Axial Compression, Leg closest to the Neutral Axis [Sva]:

= ( W / Nleg ) / Aleg

= ( 28715/4 )/21.2

= 3.39 N./mm²

Allowable Compressive Stress for the Selected Leg (KL/r < Cc ) [Sa]:

= Occfac( 1-(kl/r)²/(2*Cc²))Fy/( 5/3+3(Kl/r)/(8*Cc)-(Kl/r³)/(8*Cc³)

= 1.33(1-(47.8)²/(2*127²))248/(5/3+3(47.8)/(8*127)-(47.8³)/(8*127³)

= 170.74 N./mm²

Bending at the Bottom of the Leg closest to the N.A. [S]:

= ( Vleg * Leglen / Smdwa )

= ( 1807 * 1200/26800 )

= 80.91 N./mm²

Allowable Bending Stress[Sb]:

= ( 0.6 * Fy * Occfac )

= ( 0.6 * 248 * 1.33 )

= 198.53 N./mm²

AISC Unity Check [Sc]( must be <= to 1 and > 0 ) :

= (Sma/Sa)+(0.85*S)/((1-Sma/Spex)*Sb)

= (5.68/171 )+( 0.85 *80.9 )/(( 1 -5.68/611 ) *199 )

= 0.3829

WRC 107 Stress Analysis for Leg to Shell Junction, Ope Condition

Rectangular Attachment Parameter             C11      96.000  mm.

Rectangular Attachment Parameter             C22     180.950  mm.

Input Echo, WRC107/537 Item     1,       Description: LEGS

Diameter Basis for Vessel                 Vbasis      ID

Cylindrical or Spherical Vessel           Cylsph      Cylindrical

Internal Corrosion Allowance                 Cas      0.0000  mm.

Vessel Diameter                               Dv    1000.000  mm.

Vessel Thickness                              Tv       6.000  mm.

Design Temperature                            T1        85.0  C

Vessel Material                                  SA-240 316L

Vessel UNS Number                                     S31603

Vessel Cold S.I. Allowable                   Smc      115.00  N./mm²

Vessel Hot  S.I. Allowable                   Smh      100.46  N./mm²

Attachment Type                             Type      Rectangular

Parameter C11                                C11       96.00  mm.

Parameter C22                                C22      180.95  mm.

Thickness of Reinforcing Pad                Tpad       6.000  mm.

Pad Parameter C11P                          C11p     200.000  mm.

Pad Parameter C22P                          C22p     200.000  mm.

Design Internal Pressure                      Dp       0.522  bars

Include Pressure Thrust                                   No

Vessel Centerline Direction Cosine            Vx       0.000

Vessel Centerline Direction Cosine            Vy       1.000

Vessel Centerline Direction Cosine            Vz       0.000

Nozzle Centerline Direction Cosine            Nx       1.000

Nozzle Centerline Direction Cosine            Ny       0.000

Nozzle Centerline Direction Cosine            Nz       0.000

Global Force  (SUS)                           Fx         0.0  N

Global Force  (SUS)                           Fy      7178.7  N

Global Force  (SUS)                           Fz         0.0  N

Global Moment (SUS)                           Mx         0.0  N-m

Global Moment (SUS)                           My         0.0  N-m

Global Moment (SUS)                           Mz       385.8  N-m

Internal Pressure (SUS)                        P        0.52  bars

Include Pressure Thrust                                   No

Global Force  (OCC)                           Fx      1806.7  N

Global Force  (OCC)                           Fy      4853.3  N

Global Force  (OCC)                           Fz         0.0  N

Global Moment (OCC)                           Mx         0.0  N-m

Global Moment (OCC)                           My         0.0  N-m

Global Moment (OCC)                           Mz      1345.3  N-m

Occasional Internal Pressure (OCC)          Pvar        0.00  bars

Use Interactive Control                           No

WRC107 Version                           Version  March  1979

Include Pressure Stress Indices per Div. 2                No

Compute Pressure Stress per WRC-368                       No

Local Loads applied at end of Nozzle/Attachment           No

WRC Bulletin 537 provides equations for the dimensionless curves found in bulletin 107.  As noted

in the foreword to bulletin 537, "537 is equivalent to WRC 107".  Where 107 is printed in the

results below, "537" can be interchanged with "107".

WRC 107/537 has dimensional limitations that can be found in the bulletin's Foreword

and which can also be inferred by review of the non-dimensional curve limits.

WRC 107 Stress Calculation for SUStained loads:

Radial Load                                   P          0.0  N

Circumferential Shear                        VC          0.0  N

Longitudinal Shear                           VL       7178.7  N

Circumferential Moment                       MC          0.0  N-m

Longitudinal Moment                          ML       -385.8  N-m

Torsional Moment                             MT          0.0  N-m

Dimensionless Parameters used :   Gamma = 42.17

Dimensionless Loads for Cylindrical Shells at Attachment Junction:

-------------------------------------------------------------------

Curves read for 1979              Beta  Figure    Value   Location

-------------------------------------------------------------------

N(PHI) / ( P/Rm )                0.169   4C        6.256   (A,B)

N(PHI) / ( P/Rm )                0.169   3C        4.432   (C,D)

M(PHI) / ( P )                   0.120   2C1       0.070   (A,B)

M(PHI) / ( P )                   0.120   1C        0.103   (C,D)

N(PHI) / ( MC/(Rm**2 * Beta) )   0.117   3A        1.472   (A,B,C,D)

M(PHI) / ( MC/(Rm    * Beta) )   0.127   1A        0.089   (A,B,C,D)

N(PHI) / ( ML/(Rm**2 * Beta) )   0.145   3B        4.670   (A,B,C,D)

M(PHI) / ( ML/(Rm    * Beta) )   0.136   1B        0.037   (A,B,C,D)

N(x)   / ( P/Rm )                0.147   3C        5.061   (A,B)

N(x)   / ( P/Rm )                0.147   4C        6.635   (C,D)

M(x)   / ( P )                   0.151   1C1       0.089   (A,B)

M(x)   / ( P )                   0.151   2C        0.052   (C,D)

N(x)   / ( MC/(Rm**2 * Beta) )   0.117   4A        2.209   (A,B,C,D)

M(x)   / ( MC/(Rm    * Beta) )   0.154   2A        0.042   (A,B,C,D)

N(x)   / ( ML/(Rm**2 * Beta) )   0.145   4B        1.567   (A,B,C,D)

M(x)   / ( ML/(Rm    * Beta) )   0.159   2B        0.047   (A,B,C,D)

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |      3.5|      3.5|     -3.5|     -3.5|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      8.6|     -8.6|     -8.6|      8.6|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|     12.1|     -5.2|    -12.1|      5.2|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Long. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |      1.6|      1.6|     -1.6|     -1.6|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |      9.4|     -9.4|     -9.4|      9.4|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|     11.1|     -7.8|    -11.1|      7.8|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -1.7|     -1.7|      1.7|      1.7|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -1.7|     -1.7|      1.7|      1.7|

------------------------------------------------------------------------------------------------

Str. Int. |     12.1|      7.8|     12.1|      7.8|      3.3|      3.3|      3.3|      3.3|

Dimensionless Parameters used :   Gamma = 83.83

Dimensionless Loads for Cylindrical Shells at Pad Edge:

-------------------------------------------------------------------

Curves read for 1979              Beta  Figure    Value   Location

-------------------------------------------------------------------

N(PHI) / ( P/Rm )                0.199   4C        9.751   (A,B)

N(PHI) / ( P/Rm )                0.199   3C        4.857   (C,D)

M(PHI) / ( P )                   0.199   2C1       0.020   (A,B)

M(PHI) / ( P )                   0.199   1C  !     0.067   (C,D)

N(PHI) / ( MC/(Rm**2 * Beta) )   0.199   3A        2.954   (A,B,C,D)

M(PHI) / ( MC/(Rm    * Beta) )   0.199   1A        0.062   (A,B,C,D)

N(PHI) / ( ML/(Rm**2 * Beta) )   0.199   3B        6.692   (A,B,C,D)

M(PHI) / ( ML/(Rm    * Beta) )   0.199   1B        0.014   (A,B,C,D)

N(x)   / ( P/Rm )                0.199   3C        4.857   (A,B)

N(x)   / ( P/Rm )                0.199   4C        9.751   (C,D)

M(x)   / ( P )                   0.199   1C1       0.047   (A,B)

M(x)   / ( P )                   0.199   2C  !     0.036   (C,D)

N(x)   / ( MC/(Rm**2 * Beta) )   0.199   4A        7.366   (A,B,C,D)

M(x)   / ( MC/(Rm    * Beta) )   0.199   2A        0.027   (A,B,C,D)

N(x)   / ( ML/(Rm**2 * Beta) )   0.199   4B        3.124   (A,B,C,D)

M(x)   / ( ML/(Rm    * Beta) )   0.199   2B        0.019   (A,B,C,D)

Note - The ! mark next to the figure name denotes curve value exceeded.

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Edge of Reinforcing Pad (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |      8.6|      8.6|     -8.6|     -8.6|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      9.3|     -9.3|     -9.3|      9.3|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|     17.8|     -0.7|    -17.8|      0.7|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Long. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |      4.0|      4.0|     -4.0|     -4.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |     12.0|    -12.0|    -12.0|     12.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|     16.0|     -8.0|    -16.0|      8.0|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -3.0|     -3.0|      3.0|      3.0|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -3.0|     -3.0|      3.0|      3.0|

------------------------------------------------------------------------------------------------

Str. Int. |     17.8|      8.0|     17.8|      8.0|      6.0|      6.0|      6.0|      6.0|

WRC 107 Stress Calculation for OCCasional loads:

Radial Load                                   P       1806.7  N

Circumferential Shear                        VC          0.0  N

Longitudinal Shear                           VL       4853.3  N

Circumferential Moment                       MC          0.0  N-m

Longitudinal Moment                          ML      -1345.3  N-m

Torsional Moment                             MT          0.0  N-m

Dimensionless Parameters used :   Gamma = 42.17

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |     -1.9|     -1.9|     -1.9|     -1.9|     -1.3|     -1.3|     -1.3|     -1.3|

Circ. Bend. P  |     -5.3|      5.3|     -5.3|      5.3|     -7.8|      7.8|     -7.8|      7.8|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |     12.1|     12.1|    -12.1|    -12.1|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |     30.1|    -30.1|    -30.1|     30.1|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|     35.1|    -14.6|    -49.4|     21.4|     -9.1|      6.5|     -9.1|      6.5|

------------------------------------------------------------------------------------------------

Long. Memb. P  |     -1.5|     -1.5|     -1.5|     -1.5|     -2.0|     -2.0|     -2.0|     -2.0|

Long. Bend. P  |     -6.7|      6.7|     -6.7|      6.7|     -3.9|      3.9|     -3.9|      3.9|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |      5.8|      5.8|     -5.8|     -5.8|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |     32.9|    -32.9|    -32.9|     32.9|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|     30.4|    -21.9|    -46.8|     32.3|     -5.9|      2.0|     -5.9|      2.0|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -1.1|     -1.1|      1.1|      1.1|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -1.1|     -1.1|      1.1|      1.1|

------------------------------------------------------------------------------------------------

Str. Int. |     35.1|     21.9|     49.4|     32.3|      9.4|      6.7|      9.4|      6.7|

Dimensionless Parameters used :   Gamma = 83.83

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Edge of Reinforcing Pad (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |     -5.8|     -5.8|     -5.8|     -5.8|     -2.9|     -2.9|     -2.9|     -2.9|

Circ. Bend. P  |     -6.1|      6.1|     -6.1|      6.1|    -20.3|     20.3|    -20.3|     20.3|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |     29.8|     29.8|    -29.8|    -29.8|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |     32.4|    -32.4|    -32.4|     32.4|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|     50.3|     -2.3|    -74.1|      2.8|    -23.2|     17.4|    -23.2|     17.4|

------------------------------------------------------------------------------------------------

Long. Memb. P  |     -2.9|     -2.9|     -2.9|     -2.9|     -5.8|     -5.8|     -5.8|     -5.8|

Long. Bend. P  |    -14.0|     14.0|    -14.0|     14.0|    -10.8|     10.8|    -10.8|     10.8|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |     13.9|     13.9|    -13.9|    -13.9|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |     41.8|    -41.8|    -41.8|     41.8|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|     38.8|    -16.8|    -72.7|     39.0|    -16.7|      5.0|    -16.7|      5.0|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -2.0|     -2.0|      2.0|      2.0|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -2.0|     -2.0|      2.0|      2.0|

------------------------------------------------------------------------------------------------

Str. Int. |     50.3|     16.8|     74.1|     39.0|     23.8|     17.7|     23.8|     17.7|

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Pm (SUS) |      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|

Circ. Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Pm(TOTAL)|      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|

Circ. Pl (SUS) |      3.5|      3.5|     -3.5|     -3.5|      0.0|      0.0|      0.0|      0.0|

Circ. Pl (OCC) |     10.3|     10.3|    -14.0|    -14.0|     -1.3|     -1.3|     -1.3|     -1.3|

Circ. Pl(TOTAL)|     13.8|     13.8|    -17.5|    -17.5|     -1.3|     -1.3|     -1.3|     -1.3|

Circ. Q  (SUS) |      8.6|     -8.6|     -8.6|      8.6|      0.0|      0.0|      0.0|      0.0|

Circ. Q  (OCC) |     24.9|    -24.9|    -35.4|     35.4|     -7.8|      7.8|     -7.8|      7.8|

Circ. Q (TOTAL)|     33.5|    -33.5|    -44.1|     44.1|     -7.8|      7.8|     -7.8|      7.8|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|

Long. Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Pm(TOTAL)|      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|

Long. Pl (SUS) |      1.6|      1.6|     -1.6|     -1.6|      0.0|      0.0|      0.0|      0.0|

Long. Pl (OCC) |      4.2|      4.2|     -7.3|     -7.3|     -2.0|     -2.0|     -2.0|     -2.0|

Long. Pl(TOTAL)|      5.9|      5.9|     -8.9|     -8.9|     -2.0|     -2.0|     -2.0|     -2.0|

Long. Q  (SUS) |      9.4|     -9.4|     -9.4|      9.4|      0.0|      0.0|      0.0|      0.0|

Long. Q  (OCC) |     26.2|    -26.2|    -39.6|     39.6|     -3.9|      3.9|     -3.9|      3.9|

Long. Q (TOTAL)|     35.6|    -35.6|    -49.0|     49.0|     -3.9|      3.9|     -3.9|      3.9|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm(TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |      0.0|      0.0|      0.0|      0.0|     -1.7|     -1.7|      1.7|      1.7|

Shear Pl (OCC) |      0.0|      0.0|      0.0|      0.0|     -1.1|     -1.1|      1.1|      1.1|

Shear Pl(TOTAL)|      0.0|      0.0|      0.0|      0.0|     -2.8|     -2.8|      2.8|      2.8|

Shear Q  (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q  (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q (TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Pm (SUS)       |      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|

------------------------------------------------------------------------------------------------

Pm (SUS+OCC)   |      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |      5.6|      5.7|      1.3|      1.3|      3.5|      3.5|      3.5|      3.5|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS+OCC)|     15.9|     16.0|     15.3|     15.3|      5.8|      5.8|      5.8|      5.8|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|     49.4|     28.6|     59.4|     41.2|      8.9|      9.8|      8.9|      9.8|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          2.20            100.46      |      Passed     |

Pm (SUS+OCC)   |          2.20            120.55      |      Passed     |

Pm+Pl (SUS)    |          5.68            150.69      |      Passed     |

Pm+Pl (SUS+OCC)|         15.96            180.82      |      Passed     |

Pm+Pl+Q (TOTAL)|         59.40            301.37      |      Passed     |

The Pm+Pl+Q allowable above was based on the rules of ASME VIII-2 Part 4, 4.1.6.3 (a)&(b) or

per ASME VIII-2, 5.5.6.1(1)&(2) Part 5 as applicable.

The Pm+Pl+Q allowable was based on a temperature range cycling from ambient to design temperature.

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Reinforcing Pad Edge (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Pm (SUS) |      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|

Circ. Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Pm(TOTAL)|      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|

Circ. Pl (SUS) |      8.6|      8.6|     -8.6|     -8.6|      0.0|      0.0|      0.0|      0.0|

Circ. Pl (OCC) |     24.0|     24.0|    -35.7|    -35.7|     -2.9|     -2.9|     -2.9|     -2.9|

Circ. Pl(TOTAL)|     32.5|     32.5|    -44.2|    -44.2|     -2.9|     -2.9|     -2.9|     -2.9|

Circ. Q  (SUS) |      9.3|     -9.3|     -9.3|      9.3|      0.0|      0.0|      0.0|      0.0|

Circ. Q  (OCC) |     26.3|    -26.3|    -38.4|     38.4|    -20.3|     20.3|    -20.3|     20.3|

Circ. Q (TOTAL)|     35.6|    -35.6|    -47.7|     47.7|    -20.3|     20.3|    -20.3|     20.3|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|

Long. Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Pm(TOTAL)|      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|

Long. Pl (SUS) |      4.0|      4.0|     -4.0|     -4.0|      0.0|      0.0|      0.0|      0.0|

Long. Pl (OCC) |     11.0|     11.0|    -16.8|    -16.8|     -5.8|     -5.8|     -5.8|     -5.8|

Long. Pl(TOTAL)|     15.0|     15.0|    -20.8|    -20.8|     -5.8|     -5.8|     -5.8|     -5.8|

Long. Q  (SUS) |     12.0|    -12.0|    -12.0|     12.0|      0.0|      0.0|      0.0|      0.0|

Long. Q  (OCC) |     27.8|    -27.8|    -55.9|     55.9|    -10.8|     10.8|    -10.8|     10.8|

Long. Q (TOTAL)|     39.8|    -39.8|    -67.9|     67.9|    -10.8|     10.8|    -10.8|     10.8|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm(TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |      0.0|      0.0|      0.0|      0.0|     -3.0|     -3.0|      3.0|      3.0|

Shear Pl (OCC) |      0.0|      0.0|      0.0|      0.0|     -2.0|     -2.0|      2.0|      2.0|

Shear Pl(TOTAL)|      0.0|      0.0|      0.0|      0.0|     -5.0|     -5.0|      5.0|      5.0|

Shear Q  (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q  (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q (TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Pm (SUS)       |      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|

------------------------------------------------------------------------------------------------

Pm (SUS+OCC)   |      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |     12.9|     12.9|      4.2|      4.2|      6.4|      6.5|      6.4|      6.5|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS+OCC)|     36.9|     36.9|     39.9|     39.8|     11.2|     11.3|     11.2|     11.3|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|     72.4|     24.0|     87.6|     49.2|     22.2|     23.3|     22.2|     23.3|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          4.38            100.46      |      Passed     |

Pm (SUS+OCC)   |          4.38            120.55      |      Passed     |

Pm+Pl (SUS)    |         12.93            150.69      |      Passed     |

Pm+Pl (SUS+OCC)|         39.89            180.82      |      Passed     |

Pm+Pl+Q (TOTAL)|         87.60            301.37      |      Passed     |

The Pm+Pl+Q allowable above was based on the rules of ASME VIII-2 Part 4, 4.1.6.3 (a)&(b) or

per ASME VIII-2, 5.5.6.1(1)&(2) Part 5 as applicable.

The Pm+Pl+Q allowable was based on a temperature range cycling from ambient to design temperature.

Bolting Size Requirement for Leg Baseplates :

Baseplate Material                                     S275JR

Baseplate Allowable Stress                    SBA     114.00 N./mm²

Baseplate Length                                B   300.0000 mm.

Baseplate Width                                 D   280.0000 mm.

Baseplate Thickness                          BTHK    30.0000 mm.

Leg Dimension Along Baseplate Length            d    95.9998 mm.

Leg Dimension Along Baseplate Width             b    99.9998 mm.

Dist. from the Leg Edge to Bolt Hole Center     z    35.0000 mm.

Bolt Material                                       SA 325

Bolt Allowable Tensile Stress                STBA     120.00 N./mm²

Bolt Allowable Shear Stress               SBShear      80.00 N./mm²

Anchor Bolt Nominal Diameter                  BOD    24.0000 mm.

Number of Anchor Bolts in Tension per Leg      NB          2

Total Number of Anchors Bolt per Leg          NBT          4

Bolt Corrosion Allowance                      bca     3.0000 mm.

Ultimate 28-day Concrete Strength         FCPRIME     20.685 N./mm²

Leg Baseplate and Bolting Analysis, including Moments

Shear Stress in a Single Bolt [taub]:

= Shear Force/( Bolt Area * Number of Bolts )

= 4995/( 1.53 * 16 )

= 2.0 N./mm².  Must be less than 80.0 N./mm².

I-Beam Leg

Base Plate Available Area [AA]:

= B * D

= 300 * 280

= 840.000 cm²

Clearance Between The Bolt And The Leg Edge [BCL]:

= z - BOD / 2

= 35 - 24/2

= 28.022 mm.

Moment at Baseplate [MOMENT]:

= Vleg * Lleg

= 1807 * 1450

= 2620.841 N-m

Axial Load on the baseplate [P]:

= 3294.912 N  , based on governing load combination.

Eccentricity [e]:

= MOMENT * Conv_Factor / P

= 2621 * 1000/3295

= 795.098 mm., > D/6  [Plate Uplift Condition]

Clearance [a]:

= (D - d)/2

= (280 - 96 )/2

= 92.000 mm.

Modular Ratio of Steel/Concrete [n]:

= ES/EC

= 203402/21526

= 9.449

Dimension [F]:

= 0.5 * d + z

= 0.5 * 96 + 35

= 83.000 mm.

Dimension [K1]:

= 3(e - 0.5 * D)

= 3(795 - 0.5 * 280 )

= 1965.294 mm.

Value [K2]:

= 6 * n * Ast/B(F + e)

= 6 * 9.45 * 3.06/300 (83 + 795 )

= 507.623 cm²

Distance [K3]:

= -K2( 0.5 * D + F )

= -508 ( 0.5 * 280 + 83 )

= -113199.797 mm.³

Solving For The Effective Bearing Length Using Iteration [Y]:

:: Y³ + K1 * Y² + K2 * Y + K3 = 0

:: Y³ + 1965 * Y² + 508 * Y - 113200 = 0

= 63.231 mm.

Term [NUM]:

= D/2 - Y/3 - e

= 280/2 - 63.2/3 - 795

= -676.175 mm.

Term [DENOM]:

= D/2 - Y/3 + F

= 280/2 - 63.2/3 + 83

= 201.923 mm.

Total Bolt Tension Force [T]:

= - P * NUM/DENOM

= - 3295 * -676/202

= 11033.593 N

Overturning Moment Due To Bolt In Tension [Mt]:

= T(0.5 * D + F - Y)

= 11034 (0.5 * 280 + 83 - 63.2 )

= 1763.545 N-m

Bearing Pressure [FC]:

= 2(P + T)/(Y * B)

= 2(3295 + 11034 )/(63.2 * 300 )

= 15.11 bars [ <= FCPRIME ( 206.84)]

Equivalent Bearing Pressure [f1]:

= FC(Y - a)/Y

= 15.1 (63.2 - 92 )/63.2

= -6.874 bars

Overturning Moment Due To Bearing Pressure [Mc]:

= ( a² * B/6 ) * ( f1 + 2 * FC )

= ( 92² * 300/6 ) * ( -6.87 + 2 * 15.1 )

= 988.179 N-m

The Baseplate Required Thickness [TREQ]:

= sqrt( 6 * max( Mt, Mc ) / ( B * 1.5 * SBA ) )

= sqrt( 6 * 1764/( 300 * 171 ) )

= 14.360 mm.

Required bolt area per D. Moss [ABREQM]:

= T/STBA

= 11034/120

= 0.9195 cm² [ < Ast ( 3.06) --> PASSED ]

Distance from Top of Legs to Vessel CG [CD_DIST]:

= 675 mm.

Total Overturning Moment at Baseplate [Mbb]:

= ( Mleg / max( CD_DIST, minDist ) ) * ( CD_DIST + Lleg )

= ( 1263/max( 675, 38.1 ) ) * ( 675 + 1450 )

= 3975.084 N-m

Required Total Bolt Area per Leg per H. Bednar [ABREQB]:

= ( 1/( Nleg * STBA ) ) * ( ( 4 * Mbb/( Rn * 2 ) ) - W )

= ( 1/( 4 * 120 )) * ( ( 4 * 3975/( 1119 ) ) - 13180 )

= 0.0212 cm²

Available Total Bolt Corr. Area per Leg [ABAVL]:

= As * NBT

= 1.53 * 4

= 6.1180 cm² [ > ABREQB ( 0.02) --> PASSED]

Baseplate & Bolting Summary of Results: (mm., cm²)

Actual    Required   Pass/Fail

-------------------------------------------------------------------------

Baseplate Thickness                        30.000      14.360      Pass

Bolt Root Area (Bednar)                      6.12        0.02      Pass

Bolt Root Area (D. Moss)                     3.06        0.92      Pass

The required thickness calculation is performed based on:

* Strong axis orientation of the beam leg

* Even number of bolts installed only on the B dimension sides

Review notes about nozzle loadings and supports in the Warnings report.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

RESULTS FOR LEGS :  HydroTest Case   Description:  LEGS

Legs attached to: SHELL

Section Properties :  I Beam HE100A

European Structural Steel Data

Overall Leg Length                                  1450.000  mm.

Effective Leg Length                        Leglen  1200.000  mm.

Distance Leg Up Side of Vessel                       300.000  mm.

Number of Legs                                Nleg     4

Cross Sectional Area for HE100A               Aleg    21.200  cm²

Section Inertia    ( strong axis )                   348.998  cm^4

Section Inertia    ( weak   axis )                   133.999  cm^4

Section Modulus    ( strong axis )                 72800.297  mm.³

Section Modulus    ( weak   axis )                 26800.107  mm.³

Radius of Gyration ( strong axis )                    40.600  mm.

Radius of Gyration ( weak   axis )                    25.100  mm.

Leg Orientation - Weak Axis

Overturning Moment at top of Legs                      631.5  N-m

Total Weight Load at top of Legs                 W   27726.1  N

Total Shear force at top of Legs                      1123.4  N

Additional force in Leg due to Bracing        Fadd       0.0  N

Occasional Load Factor                      Occfac     1.333

Effective Leg End Condition Factor               k     1.000

Note: The Legs are Not Cross Braced

The Leg Shear Force includes Wind, Seismic, and Applied Load Effects.

Pad Width along Circumference                 C11P   200.000  mm.

Pad Length along Vessel Axis                  C22P   200.000  mm.

Pad Thickness                                 Tpad     6.000  mm.

Maximum Shear at top of one Leg [Vleg]:

= ( max( Wind, Seismic, Blast ) + applied forces )( Imax / Itot )

= ( 1123 )( 349/966 )

= 406.37 N

Axial Compression, Leg furthest from the Neutral Axis [Sma]:

= W/Nleg + (Mleg/(Nlegm*Rn))/Aleg

= 27727/4 + (631210/( 2 * 560 ))/2120

= 3.54 N./mm²

Axial Compression, Leg closest to the Neutral Axis [Sva]:

= ( W / Nleg ) / Aleg

= ( 27726/4 )/21.2

= 3.27 N./mm²

Allowable Compressive Stress for the Selected Leg (KL/r < Cc ) [Sa]:

= Occfac( 1-(kl/r)²/(2*Cc²))Fy/( 5/3+3(Kl/r)/(8*Cc)-(Kl/r³)/(8*Cc³)

= 1.33(1-(47.8)²/(2*127²))248/(5/3+3(47.8)/(8*127)-(47.8³)/(8*127³)

= 170.74 N./mm²

Bending at the Bottom of the Leg closest to the N.A. [S]:

= ( Vleg * Leglen / Smdwa )

= ( 406 * 1200/26800 )

= 18.20 N./mm²

Allowable Bending Stress[Sb]:

= ( 0.6 * Fy * Occfac )

= ( 0.6 * 248 * 1.33 )

= 198.53 N./mm²

AISC Unity Check [Sc]( must be <= to 1 and > 0 ) :

= (Sma/Sa)+(0.85*S)/((1-Sma/Spex)*Sb)

= (3.54/171 )+( 0.85 *18.2 )/(( 1 -3.54/611 ) *199 )

= 0.0991

WRC 107 Stress Analysis for Leg to Shell Junction, Test Condition

Rectangular Attachment Parameter             C11      96.000  mm.

Rectangular Attachment Parameter             C22     180.950  mm.

Input Echo, WRC107/537 Item     1,       Description: LEGS

Diameter Basis for Vessel                 Vbasis      ID

Cylindrical or Spherical Vessel           Cylsph      Cylindrical

Internal Corrosion Allowance                 Cas      0.0000  mm.

Vessel Diameter                               Dv    1000.000  mm.

Vessel Thickness                              Tv       6.000  mm.

Design Temperature                            T1        85.0  C

Vessel Material                                  SA-240 316L

Vessel UNS Number                                     S31603

Vessel Cold S.I. Allowable                   Smc      153.00  N./mm²

Vessel Hot  S.I. Allowable                   Smh      153.00  N./mm²

Attachment Type                             Type      Rectangular

Parameter C11                                C11       96.00  mm.

Parameter C22                                C22      180.95  mm.

Thickness of Reinforcing Pad                Tpad       6.000  mm.

Pad Parameter C11P                          C11p     200.000  mm.

Pad Parameter C22P                          C22p     200.000  mm.

Design Internal Pressure                      Dp       0.522  bars

Include Pressure Thrust                                   No

Vessel Centerline Direction Cosine            Vx       0.000

Vessel Centerline Direction Cosine            Vy       1.000

Vessel Centerline Direction Cosine            Vz       0.000

Nozzle Centerline Direction Cosine            Nx       1.000

Nozzle Centerline Direction Cosine            Ny       0.000

Nozzle Centerline Direction Cosine            Nz       0.000

Global Force  (SUS)                           Fx         0.0  N

Global Force  (SUS)                           Fy      6931.5  N

Global Force  (SUS)                           Fz         0.0  N

Global Moment (SUS)                           Mx         0.0  N-m

Global Moment (SUS)                           My         0.0  N-m

Global Moment (SUS)                           Mz       372.5  N-m

Internal Pressure (SUS)                        P        0.52  bars

Include Pressure Thrust                                   No

Global Force  (OCC)                           Fx       406.4  N

Global Force  (OCC)                           Fy       563.9  N

Global Force  (OCC)                           Fz         0.0  N

Global Moment (OCC)                           Mx         0.0  N-m

Global Moment (OCC)                           My         0.0  N-m

Global Moment (OCC)                           Mz       274.2  N-m

Occasional Internal Pressure (OCC)          Pvar        0.42  bars

Use Interactive Control                           No

WRC107 Version                           Version  March  1979

Include Pressure Stress Indices per Div. 2                No

Compute Pressure Stress per WRC-368                       No

Local Loads applied at end of Nozzle/Attachment           No

WRC Bulletin 537 provides equations for the dimensionless curves found in bulletin 107.  As noted

in the foreword to bulletin 537, "537 is equivalent to WRC 107".  Where 107 is printed in the

results below, "537" can be interchanged with "107".

WRC 107/537 has dimensional limitations that can be found in the bulletin's Foreword

and which can also be inferred by review of the non-dimensional curve limits.

WRC 107 Stress Calculation for SUStained loads:

Radial Load                                   P          0.0  N

Circumferential Shear                        VC          0.0  N

Longitudinal Shear                           VL       6931.5  N

Circumferential Moment                       MC          0.0  N-m

Longitudinal Moment                          ML       -372.5  N-m

Torsional Moment                             MT          0.0  N-m

Dimensionless Parameters used :   Gamma = 42.17

Dimensionless Loads for Cylindrical Shells at Attachment Junction:

-------------------------------------------------------------------

Curves read for 1979              Beta  Figure    Value   Location

-------------------------------------------------------------------

N(PHI) / ( P/Rm )                0.169   4C        6.256   (A,B)

N(PHI) / ( P/Rm )                0.169   3C        4.432   (C,D)

M(PHI) / ( P )                   0.120   2C1       0.070   (A,B)

M(PHI) / ( P )                   0.120   1C        0.103   (C,D)

N(PHI) / ( MC/(Rm**2 * Beta) )   0.117   3A        1.472   (A,B,C,D)

M(PHI) / ( MC/(Rm    * Beta) )   0.127   1A        0.089   (A,B,C,D)

N(PHI) / ( ML/(Rm**2 * Beta) )   0.145   3B        4.670   (A,B,C,D)

M(PHI) / ( ML/(Rm    * Beta) )   0.136   1B        0.037   (A,B,C,D)

N(x)   / ( P/Rm )                0.147   3C        5.061   (A,B)

N(x)   / ( P/Rm )                0.147   4C        6.635   (C,D)

M(x)   / ( P )                   0.151   1C1       0.089   (A,B)

M(x)   / ( P )                   0.151   2C        0.052   (C,D)

N(x)   / ( MC/(Rm**2 * Beta) )   0.117   4A        2.209   (A,B,C,D)

M(x)   / ( MC/(Rm    * Beta) )   0.154   2A        0.042   (A,B,C,D)

N(x)   / ( ML/(Rm**2 * Beta) )   0.145   4B        1.567   (A,B,C,D)

M(x)   / ( ML/(Rm    * Beta) )   0.159   2B        0.047   (A,B,C,D)

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |      3.4|      3.4|     -3.4|     -3.4|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      8.3|     -8.3|     -8.3|      8.3|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|     11.7|     -5.0|    -11.7|      5.0|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Long. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |      1.6|      1.6|     -1.6|     -1.6|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |      9.1|     -9.1|     -9.1|      9.1|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|     10.7|     -7.5|    -10.7|      7.5|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -1.6|     -1.6|      1.6|      1.6|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -1.6|     -1.6|      1.6|      1.6|

------------------------------------------------------------------------------------------------

Str. Int. |     11.7|      7.5|     11.7|      7.5|      3.2|      3.2|      3.2|      3.2|

Dimensionless Parameters used :   Gamma = 83.83

Dimensionless Loads for Cylindrical Shells at Pad Edge:

-------------------------------------------------------------------

Curves read for 1979              Beta  Figure    Value   Location

-------------------------------------------------------------------

N(PHI) / ( P/Rm )                0.199   4C        9.751   (A,B)

N(PHI) / ( P/Rm )                0.199   3C        4.857   (C,D)

M(PHI) / ( P )                   0.199   2C1       0.020   (A,B)

M(PHI) / ( P )                   0.199   1C  !     0.067   (C,D)

N(PHI) / ( MC/(Rm**2 * Beta) )   0.199   3A        2.954   (A,B,C,D)

M(PHI) / ( MC/(Rm    * Beta) )   0.199   1A        0.062   (A,B,C,D)

N(PHI) / ( ML/(Rm**2 * Beta) )   0.199   3B        6.692   (A,B,C,D)

M(PHI) / ( ML/(Rm    * Beta) )   0.199   1B        0.014   (A,B,C,D)

N(x)   / ( P/Rm )                0.199   3C        4.857   (A,B)

N(x)   / ( P/Rm )                0.199   4C        9.751   (C,D)

M(x)   / ( P )                   0.199   1C1       0.047   (A,B)

M(x)   / ( P )                   0.199   2C  !     0.036   (C,D)

N(x)   / ( MC/(Rm**2 * Beta) )   0.199   4A        7.366   (A,B,C,D)

M(x)   / ( MC/(Rm    * Beta) )   0.199   2A        0.027   (A,B,C,D)

N(x)   / ( ML/(Rm**2 * Beta) )   0.199   4B        3.124   (A,B,C,D)

M(x)   / ( ML/(Rm    * Beta) )   0.199   2B        0.019   (A,B,C,D)

Note - The ! mark next to the figure name denotes curve value exceeded.

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Edge of Reinforcing Pad (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |      8.3|      8.3|     -8.3|     -8.3|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      9.0|     -9.0|     -9.0|      9.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|     17.2|     -0.7|    -17.2|      0.7|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Long. Memb. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. P  |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |      3.9|      3.9|     -3.9|     -3.9|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |     11.6|    -11.6|    -11.6|     11.6|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|     15.4|     -7.7|    -15.4|      7.7|      0.0|      0.0|      0.0|      0.0|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -2.9|     -2.9|      2.9|      2.9|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -2.9|     -2.9|      2.9|      2.9|

------------------------------------------------------------------------------------------------

Str. Int. |     17.2|      7.7|     17.2|      7.7|      5.8|      5.8|      5.8|      5.8|

WRC 107 Stress Calculation for OCCasional loads:

Radial Load                                   P        406.4  N

Circumferential Shear                        VC          0.0  N

Longitudinal Shear                           VL        563.9  N

Circumferential Moment                       MC          0.0  N-m

Longitudinal Moment                          ML       -274.2  N-m

Torsional Moment                             MT          0.0  N-m

Dimensionless Parameters used :   Gamma = 42.17

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |     -0.4|     -0.4|     -0.4|     -0.4|     -0.3|     -0.3|     -0.3|     -0.3|

Circ. Bend. P  |     -1.2|      1.2|     -1.2|      1.2|     -1.7|      1.7|     -1.7|      1.7|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |      2.5|      2.5|     -2.5|     -2.5|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      6.1|     -6.1|     -6.1|      6.1|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|      7.0|     -2.9|    -10.2|      4.4|     -2.0|      1.5|     -2.0|      1.5|

------------------------------------------------------------------------------------------------

Long. Memb. P  |     -0.3|     -0.3|     -0.3|     -0.3|     -0.4|     -0.4|     -0.4|     -0.4|

Long. Bend. P  |     -1.5|      1.5|     -1.5|      1.5|     -0.9|      0.9|     -0.9|      0.9|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |      1.2|      1.2|     -1.2|     -1.2|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |      6.7|     -6.7|     -6.7|      6.7|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|      6.0|     -4.4|     -9.7|      6.7|     -1.3|      0.4|     -1.3|      0.4|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -0.1|     -0.1|      0.1|      0.1|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -0.1|     -0.1|      0.1|      0.1|

------------------------------------------------------------------------------------------------

Str. Int. |      7.0|      4.4|     10.2|      6.7|      2.1|      1.5|      2.1|      1.5|

Dimensionless Parameters used :   Gamma = 83.83

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Edge of Reinforcing Pad (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Memb. P  |     -1.3|     -1.3|     -1.3|     -1.3|     -0.7|     -0.7|     -0.7|     -0.7|

Circ. Bend. P  |     -1.4|      1.4|     -1.4|      1.4|     -4.6|      4.6|     -4.6|      4.6|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Circ. Memb. ML |      6.1|      6.1|     -6.1|     -6.1|      0.0|      0.0|      0.0|      0.0|

Circ. Bend. ML |      6.6|     -6.6|     -6.6|      6.6|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Circ. Str.|     10.0|     -0.5|    -15.4|      0.6|     -5.2|      3.9|     -5.2|      3.9|

------------------------------------------------------------------------------------------------

Long. Memb. P  |     -0.7|     -0.7|     -0.7|     -0.7|     -1.3|     -1.3|     -1.3|     -1.3|

Long. Bend. P  |     -3.2|      3.2|     -3.2|      3.2|     -2.4|      2.4|     -2.4|      2.4|

Long. Memb. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Bend. MC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Long. Memb. ML |      2.8|      2.8|     -2.8|     -2.8|      0.0|      0.0|      0.0|      0.0|

Long. Bend. ML |      8.5|     -8.5|     -8.5|      8.5|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Long. Str.|      7.6|     -3.2|    -15.2|      8.2|     -3.7|      1.1|     -3.7|      1.1|

------------------------------------------------------------------------------------------------

Shear VC |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -0.2|     -0.2|      0.2|      0.2|

Shear MT |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Shear|      0.0|      0.0|      0.0|      0.0|     -0.2|     -0.2|      0.2|      0.2|

------------------------------------------------------------------------------------------------

Str. Int. |     10.0|      3.2|     15.4|      8.2|      5.3|      3.9|      5.3|      3.9|

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Pm (SUS) |      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|

Circ. Pm (OCC) |      1.7|      1.8|      1.7|      1.8|      1.7|      1.8|      1.7|      1.8|

Circ. Pm(TOTAL)|      3.9|      4.0|      3.9|      4.0|      3.9|      4.0|      3.9|      4.0|

Circ. Pl (SUS) |      3.4|      3.4|     -3.4|     -3.4|      0.0|      0.0|      0.0|      0.0|

Circ. Pl (OCC) |      2.1|      2.1|     -2.9|     -2.9|     -0.3|     -0.3|     -0.3|     -0.3|

Circ. Pl(TOTAL)|      5.4|      5.4|     -6.3|     -6.3|     -0.3|     -0.3|     -0.3|     -0.3|

Circ. Q  (SUS) |      8.3|     -8.3|     -8.3|      8.3|      0.0|      0.0|      0.0|      0.0|

Circ. Q  (OCC) |      5.0|     -5.0|     -7.3|      7.3|     -1.7|      1.7|     -1.7|      1.7|

Circ. Q (TOTAL)|     13.3|    -13.3|    -15.7|     15.7|     -1.7|      1.7|     -1.7|      1.7|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|      1.1|

Long. Pm (OCC) |      0.9|      0.9|      0.9|      0.9|      0.9|      0.9|      0.9|      0.9|

Long. Pm(TOTAL)|      1.9|      1.9|      1.9|      1.9|      1.9|      1.9|      1.9|      1.9|

Long. Pl (SUS) |      1.6|      1.6|     -1.6|     -1.6|      0.0|      0.0|      0.0|      0.0|

Long. Pl (OCC) |      0.8|      0.8|     -1.5|     -1.5|     -0.4|     -0.4|     -0.4|     -0.4|

Long. Pl(TOTAL)|      2.4|      2.4|     -3.1|     -3.1|     -0.4|     -0.4|     -0.4|     -0.4|

Long. Q  (SUS) |      9.1|     -9.1|     -9.1|      9.1|      0.0|      0.0|      0.0|      0.0|

Long. Q  (OCC) |      5.2|     -5.2|     -8.2|      8.2|     -0.9|      0.9|     -0.9|      0.9|

Long. Q (TOTAL)|     14.3|    -14.3|    -17.3|     17.3|     -0.9|      0.9|     -0.9|      0.9|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm(TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |      0.0|      0.0|      0.0|      0.0|     -1.6|     -1.6|      1.6|      1.6|

Shear Pl (OCC) |      0.0|      0.0|      0.0|      0.0|     -0.1|     -0.1|      0.1|      0.1|

Shear Pl(TOTAL)|      0.0|      0.0|      0.0|      0.0|     -1.7|     -1.7|      1.7|      1.7|

Shear Q  (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q  (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q (TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Pm (SUS)       |      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|      2.1|      2.2|

------------------------------------------------------------------------------------------------

Pm (SUS+OCC)   |      3.9|      4.0|      3.9|      4.0|      3.9|      4.0|      3.9|      4.0|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |      5.5|      5.6|      1.2|      1.2|      3.4|      3.4|      3.4|      3.4|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS+OCC)|      9.3|      9.4|      2.4|      2.3|      4.5|      4.6|      4.5|      4.6|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|     22.6|      9.9|     18.5|     16.1|      3.7|      6.2|      3.7|      6.2|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          2.20            153.00      |      Passed     |

Pm (SUS+OCC)   |          3.96            183.60      |      Passed     |

Pm+Pl (SUS)    |          5.56            229.50      |      Passed     |

Pm+Pl (SUS+OCC)|          9.38            275.40      |      Passed     |

Pm+Pl+Q (TOTAL)|         22.59            458.99      |      Passed     |

The Pm+Pl+Q allowable above was based on the rules of ASME VIII-2 Part 4, 4.1.6.3 (a)&(b) or

per ASME VIII-2, 5.5.6.1(1)&(2) Part 5 as applicable.

The Pm+Pl+Q allowable was based on a temperature range cycling from ambient to design temperature.

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Reinforcing Pad Edge (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Circ. Pm (SUS) |      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|

Circ. Pm (OCC) |      3.5|      3.5|      3.5|      3.5|      3.5|      3.5|      3.5|      3.5|

Circ. Pm(TOTAL)|      7.8|      7.9|      7.8|      7.9|      7.8|      7.9|      7.8|      7.9|

Circ. Pl (SUS) |      8.3|      8.3|     -8.3|     -8.3|      0.0|      0.0|      0.0|      0.0|

Circ. Pl (OCC) |      4.8|      4.8|     -7.4|     -7.4|     -0.7|     -0.7|     -0.7|     -0.7|

Circ. Pl(TOTAL)|     13.0|     13.0|    -15.6|    -15.6|     -0.7|     -0.7|     -0.7|     -0.7|

Circ. Q  (SUS) |      9.0|     -9.0|     -9.0|      9.0|      0.0|      0.0|      0.0|      0.0|

Circ. Q  (OCC) |      5.2|     -5.2|     -8.0|      8.0|     -4.6|      4.6|     -4.6|      4.6|

Circ. Q (TOTAL)|     14.2|    -14.2|    -16.9|     16.9|     -4.6|      4.6|     -4.6|      4.6|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|      2.2|

Long. Pm (OCC) |      1.7|      1.7|      1.7|      1.7|      1.7|      1.7|      1.7|      1.7|

Long. Pm(TOTAL)|      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|

Long. Pl (SUS) |      3.9|      3.9|     -3.9|     -3.9|      0.0|      0.0|      0.0|      0.0|

Long. Pl (OCC) |      2.2|      2.2|     -3.5|     -3.5|     -1.3|     -1.3|     -1.3|     -1.3|

Long. Pl(TOTAL)|      6.0|      6.0|     -7.3|     -7.3|     -1.3|     -1.3|     -1.3|     -1.3|

Long. Q  (SUS) |     11.6|    -11.6|    -11.6|     11.6|      0.0|      0.0|      0.0|      0.0|

Long. Q  (OCC) |      5.4|     -5.4|    -11.7|     11.7|     -2.4|      2.4|     -2.4|      2.4|

Long. Q (TOTAL)|     17.0|    -17.0|    -23.3|     23.3|     -2.4|      2.4|     -2.4|      2.4|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pm(TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |      0.0|      0.0|      0.0|      0.0|     -2.9|     -2.9|      2.9|      2.9|

Shear Pl (OCC) |      0.0|      0.0|      0.0|      0.0|     -0.2|     -0.2|      0.2|      0.2|

Shear Pl(TOTAL)|      0.0|      0.0|      0.0|      0.0|     -3.1|     -3.1|      3.1|      3.1|

Shear Q  (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q  (OCC) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Q (TOTAL)|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Pm (SUS)       |      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|      4.3|      4.4|

------------------------------------------------------------------------------------------------

Pm (SUS+OCC)   |      7.8|      7.9|      7.8|      7.9|      7.8|      7.9|      7.8|      7.9|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |     12.6|     12.6|      3.9|      3.9|      6.3|      6.4|      6.3|      6.4|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS+OCC)|     20.8|     20.9|      7.9|      7.8|      8.7|      8.8|      8.7|      8.8|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|     35.0|     13.7|     26.7|     19.8|      6.7|     13.0|      6.7|     13.0|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          4.38            153.00      |      Passed     |

Pm (SUS+OCC)   |          7.87            183.60      |      Passed     |

Pm+Pl (SUS)    |         12.63            229.50      |      Passed     |

Pm+Pl (SUS+OCC)|         20.90            275.40      |      Passed     |

Pm+Pl+Q (TOTAL)|         34.99            458.99      |      Passed     |

The Pm+Pl+Q allowable above was based on the rules of ASME VIII-2 Part 4, 4.1.6.3 (a)&(b) or

per ASME VIII-2, 5.5.6.1(1)&(2) Part 5 as applicable.

The Pm+Pl+Q allowable was based on a temperature range cycling from ambient to design temperature.

Bolting Size Requirement for Leg Baseplates :

Baseplate Material                                     S275JR

Baseplate Allowable Stress                    SBA     114.00 N./mm²

Baseplate Length                                B   300.0000 mm.

Baseplate Width                                 D   280.0000 mm.

Baseplate Thickness                          BTHK    30.0000 mm.

Leg Dimension Along Baseplate Length            d    95.9998 mm.

Leg Dimension Along Baseplate Width             b    99.9998 mm.

Dist. from the Leg Edge to Bolt Hole Center     z    35.0000 mm.

Bolt Material                                       SA 325

Bolt Allowable Tensile Stress                STBA     120.00 N./mm²

Bolt Allowable Shear Stress               SBShear      80.00 N./mm²

Anchor Bolt Nominal Diameter                  BOD    24.0000 mm.

Number of Anchor Bolts in Tension per Leg      NB          2

Total Number of Anchors Bolt per Leg          NBT          4

Bolt Corrosion Allowance                      bca     3.0000 mm.

Ultimate 28-day Concrete Strength         FCPRIME     20.685 N./mm²

Leg Baseplate and Bolting Analysis, including Moments

Shear Stress in a Single Bolt [taub]:

= Shear Force/( Bolt Area * Number of Bolts )

= 4995/( 1.53 * 16 )

= 2.0 N./mm².  Must be less than 80.0 N./mm².

I-Beam Leg

Base Plate Available Area [AA]:

= B * D

= 300 * 280

= 840.000 cm²

Clearance Between The Bolt And The Leg Edge [BCL]:

= z - BOD / 2

= 35 - 24/2

= 28.022 mm.

Moment at Baseplate [MOMENT]:

= Vleg * Lleg

= 406 * 1450

= 589.468 N-m

Axial Load on the baseplate [P]:

= 3294.912 N  , based on governing load combination.

Eccentricity [e]:

= MOMENT * Conv_Factor / P

= 589 * 1000/3295

= 178.830 mm., > D/6  [Plate Uplift Condition]

Clearance [a]:

= (D - d)/2

= (280 - 96 )/2

= 92.000 mm.

Modular Ratio of Steel/Concrete [n]:

= ES/EC

= 203402/21526

= 9.449

Dimension [F]:

= 0.5 * d + z

= 0.5 * 96 + 35

= 83.000 mm.

Dimension [K1]:

= 3(e - 0.5 * D)

= 3(179 - 0.5 * 280 )

= 116.490 mm.

Value [K2]:

= 6 * n * Ast/B(F + e)

= 6 * 9.45 * 3.06/300 (83 + 179 )

= 151.362 cm²

Distance [K3]:

= -K2( 0.5 * D + F )

= -151 ( 0.5 * 280 + 83 )

= -33753.758 mm.³

Solving For The Effective Bearing Length Using Iteration [Y]:

:: Y³ + K1 * Y² + K2 * Y + K3 = 0

:: Y³ + 116 * Y² + 151 * Y - 33754 = 0

= 95.445 mm.

Term [NUM]:

= D/2 - Y/3 - e

= 280/2 - 95.4/3 - 179

= -70.645 mm.

Term [DENOM]:

= D/2 - Y/3 + F

= 280/2 - 95.4/3 + 83

= 191.185 mm.

Total Bolt Tension Force [T]:

= - P * NUM/DENOM

= - 3295 * -70.6/191

= 1217.512 N

Overturning Moment Due To Bolt In Tension [Mt]:

= T(0.5 * D + F - Y)

= 1218 (0.5 * 280 + 83 - 95.4 )

= 155.362 N-m

Bearing Pressure [FC]:

= 2(P + T)/(Y * B)

= 2(3295 + 1218 )/(95.4 * 300 )

= 3.15 bars [ <= FCPRIME ( 206.84)]

Equivalent Bearing Pressure [f1]:

= FC(Y - a)/Y

= 3.15 (95.4 - 92 )/95.4

= 0.114 bars

Overturning Moment Due To Bearing Pressure [Mc]:

= ( a² * B/6 ) * ( f1 + 2 * FC )

= ( 92² * 300/6 ) * ( 0.11 + 2 * 3.15 )

= 271.697 N-m

The Baseplate Required Thickness [TREQ]:

= sqrt( 6 * max( Mt, Mc ) / ( B * 1.5 * SBA ) )

= sqrt( 6 * 272/( 300 * 171 ) )

= 5.636 mm.

Required bolt area per D. Moss [ABREQM]:

= T/STBA

= 1218/120

= 0.1015 cm² [ < Ast ( 3.06) --> PASSED ]

Distance from Top of Legs to Vessel CG [CD_DIST]:

= 679 mm.

Total Overturning Moment at Baseplate [Mbb]:

= ( Mleg / max( CD_DIST, minDist ) ) * ( CD_DIST + Lleg )

= ( 631/max( 679, 38.1 ) ) * ( 679 + 1450 )

= 1980.042 N-m

Required Total Bolt Area per Leg per H. Bednar [ABREQB]:

= ( 1/( Nleg * STBA ) ) * ( ( 4 * Mbb/( Rn * 2 ) ) - W )

= ( 1/( 4 * 120 )) * ( ( 4 * 1980/( 1119 ) ) - 13180 )

= -0.1272 cm² --> (No tension in bolts)

Baseplate & Bolting Summary of Results: (mm., cm²)

Actual    Required   Pass/Fail

-------------------------------------------------------------------------

Baseplate Thickness                        30.000       5.636      Pass

Bolt Root Area (D. Moss)                     3.06        0.10      Pass

The required thickness calculation is performed based on:

* Strong axis orientation of the beam leg

* Even number of bolts installed only on the B dimension sides

Review notes about nozzle loadings and supports in the Warnings report.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N8                                From:   10

Pressure for Reinforcement Calculations        P       0.750  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.50  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Elliptical Head             D     1000.00  mm.

Aspect Ratio of Elliptical Head               Ar        2.00

Head Finished (Minimum) Thickness              t      6.0000  mm.

Head Internal Corrosion Allowance              c      0.0000  mm.

Head External Corrosion Allowance             co      0.0000  mm.

Distance from Head Centerline                 L1      0.0000  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                            0.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      8.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Flange Class                                             150

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N8

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Elliptical Head, tr [Int. Press]

= P*D*K1/(2*Sv*E-0.2*P) per Appendix 1-4(c)

= 0.75*1000*0.9/(2*100*1-0.2*0.75)

= 0.3361 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.75*30.2/(100*1+0.4*0.75)

= 0.0225 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N8.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0225 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.3734 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.023, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

Stresses on Nozzle due to External and Pressure Loads per the ASME B31.3 Piping Code,

(see 319.4.4 and 302.3.5):

--------------------------------------------------------

Sustained  :     21.4,  Allowable :    100.5 N./mm²  Passed

Expansion  :      0.0,  Allowable :    248.0 N./mm²  Passed

Occasional :      0.1,  Allowable :    133.7 N./mm²  Passed

Shear      :     16.8,  Allowable :     70.3 N./mm²  Passed

The number of cycles on this nozzle was assumed to be 7000 or less for the determination of

the expansion stress allowable.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N8

Intermediate Calc. for nozzle/shell Welds   Tmin      8.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          5.6000 = 0.7 * tmin.  5.6560 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.5029 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.5029 mm.

Input Echo, WRC107/537 Item     1,       Description: N8            :

Diameter Basis for Vessel                 Vbasis      ID

Cylindrical or Spherical Vessel           Cylsph      Spherical

Internal Corrosion Allowance                 Cas      0.0000  mm.

Vessel Diameter                               Dv    1800.000  mm.

Vessel Thickness                              Tv       6.000  mm.

Design Temperature                            T1        85.0  C

Vessel Material                                  SA-240 316L

Vessel UNS Number                                     S31603

Vessel Cold S.I. Allowable                   Smc      115.00  N./mm²

Vessel Hot  S.I. Allowable                   Smh      100.50  N./mm²

Attachment Type                             Type      Round

WRC107 Attachment Classification          Holsol      Hollow

Diameter Basis for Nozzle                 Nbasis      OD

Corrosion Allowance for Nozzle               Can      0.0000  mm.

Nozzle Diameter                               Dn      60.325  mm.

Nozzle Thickness                              Tn       8.738  mm.

Nozzle Material                                SA-312 TP316L

Nozzle UNS Number                                     S31603

Nozzle Cold S.I. Allowable                  SNmc      115.00  N./mm²

Nozzle Hot  S.I. Allowable                  SNmh      100.50  N./mm²

Design Internal Pressure                      Dp       0.750  bars

Include Pressure Thrust                                   No

External Forces and Moments in WRC 107/537 Convention:

Radial Load             (SUS)                  P     -1000.0  N

Longitudinal Shear      (SUS)           (Vl)  V1      1225.0  N

Circumferential Shear   (SUS)           (Vc)  V2      1225.0  N

Circumferential Moment  (SUS)           (Mc)  M1       250.0  N-m

Longitudinal Moment     (SUS)           (Ml)  M2       250.0  N-m

Torsional Moment        (SUS)                 Mt       350.0  N-m

Use Interactive Control                           No

WRC107 Version                           Version  March  1979

Include Pressure Stress Indices per Div. 2                No

Compute Pressure Stress per WRC-368                       No

Local Loads applied at end of Nozzle/Attachment           No

WRC Bulletin 537 provides equations for the dimensionless curves found in bulletin 107.  As noted

in the foreword to bulletin 537, "537 is equivalent to WRC 107".  Where 107 is printed in the

results below, "537" can be interchanged with "107".

WRC 107/537 has dimensional limitations that can be found in the bulletin's Foreword

and which can also be inferred by review of the non-dimensional curve limits.

Stress Attenuation Diameter (for Insert Plates) per WRC 297:

= NozzleOD + 2 * 1.65 * sqrt( Rmean( t - ca ) )

= 60.325 + 2 * 1.65 * sqrt( 903.0 ( 6.0 - 0 ) )

= 303.228 mm.

WRC 107 Stress Calculation for SUStained loads:

Radial Load                                   P      -1000.0  N

Circumferential Shear                  (VC)  V2       1225.0  N

Longitudinal Shear                     (VL)  V1       1225.0  N

Circumferential Moment                 (MC)  M1        250.0  N-m

Longitudinal Moment                    (ML)  M2        250.0  N-m

Torsional Moment                             MT        350.0  N-m

Dimensionless Param: U =   0.41 TAU =   5.00 (  2.95) RHO =   0.69

Dimensionless Loads for Spherical Shells at Attachment Junction:

------------------------------------------------------------

Curves read for 1979            Figure     Value   Location

------------------------------------------------------------

N(x) * T / P                     SP 1    0.08524   (A,B,C,D)

M(x)     / P                     SP 1    0.10837   (A,B,C,D)

N(x) * T * SQRT(Rm * T ) / MC    SM 1    0.13874   (A,B,C,D)

M(x)     * SQRT(Rm * T ) / MC    SM 1    0.30529   (A,B,C,D)

N(x) * T * SQRT(Rm * T ) / ML    SM 1    0.13874   (A,B,C,D)

M(x)     * SQRT(Rm * T ) / ML    SM 1    0.30529   (A,B,C,D)

N(y) * T / P                     SP 1    0.14151   (A,B,C,D)

M(y)     / P                     SP 1    0.03726   (A,B,C,D)

N(y) * T * SQRT(Rm * T ) / MC    SM 1    0.08279   (A,B,C,D)

M(y)     * SQRT(Rm * T ) / MC    SM 1    0.11593   (A,B,C,D)

N(y) * T * SQRT(Rm * T ) / ML    SM 1    0.08279   (A,B,C,D)

M(y)     * SQRT(Rm * T ) / ML    SM 1    0.11593   (A,B,C,D)

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Rad.  Memb. P  |      2.4|      2.4|      2.4|      2.4|      2.4|      2.4|      2.4|      2.4|

Rad.  Bend. P  |     18.1|    -18.1|     18.1|    -18.1|     18.1|    -18.1|     18.1|    -18.1|

Rad.  Memb. MC |      0.0|      0.0|      0.0|      0.0|    -13.1|    -13.1|     13.1|     13.1|

Rad.  Memb. MC |      0.0|      0.0|      0.0|      0.0|   -172.8|    172.8|    172.8|   -172.8|

Rad.  Memb. ML |    -13.1|    -13.1|     13.1|     13.1|      0.0|      0.0|      0.0|      0.0|

Rad.  Bend. ML |   -172.8|    172.8|    172.8|   -172.8|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Rad.  Str.|   -165.4|    144.0|    206.3|   -175.4|   -165.4|    144.0|    206.3|   -175.4|

------------------------------------------------------------------------------------------------

Tang. Memb. P  |      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|

Tang. Bend. P  |      6.2|     -6.2|      6.2|     -6.2|      6.2|     -6.2|      6.2|     -6.2|

Tang. Memb. MC |      0.0|      0.0|      0.0|      0.0|     -7.8|     -7.8|      7.8|      7.8|

Tang. Bend. MC |      0.0|      0.0|      0.0|      0.0|    -65.6|     65.6|     65.6|    -65.6|

Tang. Memb. ML |     -7.8|     -7.8|      7.8|      7.8|      0.0|      0.0|      0.0|      0.0|

Tang. Bend. ML |    -65.6|     65.6|     65.6|    -65.6|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Tang. Str.|    -63.3|     55.5|     83.6|    -60.1|    -63.3|     55.5|     83.6|    -60.1|

------------------------------------------------------------------------------------------------

Shear VC |      2.2|      2.2|     -2.2|     -2.2|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -2.2|     -2.2|      2.2|      2.2|

Shear MT |     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|

|                                                                               |

Tot. Shear|     12.4|     12.4|      8.0|      8.0|      8.0|      8.0|     12.4|     12.4|

------------------------------------------------------------------------------------------------

Str. Int. |    166.9|    145.7|    206.8|    175.9|    166.0|    144.7|    207.5|    176.7|

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Rad.  Pm (SUS) |      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|

Rad.  Pl (SUS) |    -10.7|    -10.7|     15.5|     15.5|    -10.7|    -10.7|     15.5|     15.5|

Rad.  Q  (SUS) |   -154.7|    154.7|    190.8|   -190.8|   -154.7|    154.7|    190.8|   -190.8|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|

Long. Pl (SUS) |     -3.9|     -3.9|     11.7|     11.7|     -3.9|     -3.9|     11.7|     11.7|

Long. Q  (SUS) |    -59.4|     59.4|     71.8|    -71.8|    -59.4|     59.4|     71.8|    -71.8|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |      2.2|      2.2|     -2.2|     -2.2|     -2.2|     -2.2|      2.2|      2.2|

Shear Q  (SUS) |     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|

Pm (SUS)       |      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |      8.1|      8.1|     22.1|     22.1|      8.1|      8.1|     22.1|     22.1|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|    161.3|    151.3|    212.4|    170.3|    160.4|    150.3|    213.1|    171.1|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          5.63            100.50      |      Passed     |

Pm+Pl (SUS)    |         22.07            150.75      |      Passed     |

Pm+Pl+Q (TOTAL)|        213.14            301.50      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N5                                From:   10

Pressure for Reinforcement Calculations        P       0.747  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.50  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Elliptical Head             D     1000.00  mm.

Aspect Ratio of Elliptical Head               Ar        2.00

Head Finished (Minimum) Thickness              t      6.0000  mm.

Head Internal Corrosion Allowance              c      0.0000  mm.

Head External Corrosion Allowance             co      0.0000  mm.

Distance from Head Centerline                 L1    250.0000  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                            0.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      8.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Flange Class                                             150

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Note : Checking Nozzle in the Meridional direction.

Reinforcement CALCULATION, Description: N5

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Elliptical Head, tr [Int. Press]

= P*D*K1/(2*Sv*E-0.2*P) per Appendix 1-4(c)

= 0.75*1000*0.9/(2*100*1-0.2*0.75)

= 0.3346 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.75*30.2/(100*1+0.4*0.75)

= 0.0224 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    89.2165  mm.

Parallel to Vessel Wall, opening length              d    44.6083  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N5.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N5

Intermediate Calc. for nozzle/shell Welds   Tmin      8.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          5.6000 = 0.7 * tmin.  5.6560 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

Note : Checking Nozzle in the Latitudinal direction.

Reinforcement CALCULATION, Description: N5

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Elliptical Head, tr [Int. Press]

= P*D*K1/(2*Sv*E-0.2*P) per Appendix 1-4(c)

= 0.75*1000*0.9/(2*100*1-0.2*0.75)

= 0.3346 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.75*30.2/(100*1+0.4*0.75)

= 0.0224 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N5.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0224 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.3717 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.022, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

Stresses on Nozzle due to External and Pressure Loads per the ASME B31.3 Piping Code,

(see 319.4.4 and 302.3.5):

--------------------------------------------------------

Sustained  :     21.4,  Allowable :    100.5 N./mm²  Passed

Expansion  :      0.0,  Allowable :    248.0 N./mm²  Passed

Occasional :      0.1,  Allowable :    133.7 N./mm²  Passed

Shear      :     16.8,  Allowable :     70.3 N./mm²  Passed

The number of cycles on this nozzle was assumed to be 7000 or less for the determination of

the expansion stress allowable.

Weld Size Calculations, Description: N5

Intermediate Calc. for nozzle/shell Welds   Tmin      8.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          5.6000 = 0.7 * tmin.  5.6560 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 9.2439 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 165.4852 mm.

Warning:

The equations for nozzle local stress analysis pertain to radial nozzles only

and this nozzle is not specified as radial.  Please check the specific

analysis code for all assumptions and limitations.

Input Echo, WRC107/537 Item     1,       Description: N5            :

Diameter Basis for Vessel                 Vbasis      ID

Cylindrical or Spherical Vessel           Cylsph      Spherical

Internal Corrosion Allowance                 Cas      0.0000  mm.

Vessel Diameter                               Dv    1800.000  mm.

Vessel Thickness                              Tv       6.000  mm.

Design Temperature                            T1        85.0  C

Vessel Material                                  SA-240 316L

Vessel UNS Number                                     S31603

Vessel Cold S.I. Allowable                   Smc      115.00  N./mm²

Vessel Hot  S.I. Allowable                   Smh      100.50  N./mm²

Attachment Type                             Type      Round

WRC107 Attachment Classification          Holsol      Hollow

Diameter Basis for Nozzle                 Nbasis      OD

Corrosion Allowance for Nozzle               Can      0.0000  mm.

Nozzle Diameter                               Dn      60.325  mm.

Nozzle Thickness                              Tn       8.738  mm.

Nozzle Material                                SA-312 TP316L

Nozzle UNS Number                                     S31603

Nozzle Cold S.I. Allowable                  SNmc      115.00  N./mm²

Nozzle Hot  S.I. Allowable                  SNmh      100.50  N./mm²

Design Internal Pressure                      Dp       0.747  bars

Include Pressure Thrust                                   No

External Forces and Moments in WRC 107/537 Convention:

Radial Load             (SUS)                  P     -1000.0  N

Longitudinal Shear      (SUS)           (Vl)  V1      1225.0  N

Circumferential Shear   (SUS)           (Vc)  V2      1225.0  N

Circumferential Moment  (SUS)           (Mc)  M1       250.0  N-m

Longitudinal Moment     (SUS)           (Ml)  M2       250.0  N-m

Torsional Moment        (SUS)                 Mt       350.0  N-m

Use Interactive Control                           No

WRC107 Version                           Version  March  1979

Include Pressure Stress Indices per Div. 2                No

Compute Pressure Stress per WRC-368                       No

Local Loads applied at end of Nozzle/Attachment           No

WRC Bulletin 537 provides equations for the dimensionless curves found in bulletin 107.  As noted

in the foreword to bulletin 537, "537 is equivalent to WRC 107".  Where 107 is printed in the

results below, "537" can be interchanged with "107".

WRC 107/537 has dimensional limitations that can be found in the bulletin's Foreword

and which can also be inferred by review of the non-dimensional curve limits.

Stress Attenuation Diameter (for Insert Plates) per WRC 297:

= NozzleOD + 2 * 1.65 * sqrt( Rmean( t - ca ) )

= 60.325 + 2 * 1.65 * sqrt( 903.0 ( 6.0 - 0 ) )

= 303.228 mm.

WRC 107 Stress Calculation for SUStained loads:

Radial Load                                   P      -1000.0  N

Circumferential Shear                  (VC)  V2       1225.0  N

Longitudinal Shear                     (VL)  V1       1225.0  N

Circumferential Moment                 (MC)  M1        250.0  N-m

Longitudinal Moment                    (ML)  M2        250.0  N-m

Torsional Moment                             MT        350.0  N-m

Dimensionless Param: U =   0.41 TAU =   5.00 (  2.95) RHO =   0.69

Dimensionless Loads for Spherical Shells at Attachment Junction:

------------------------------------------------------------

Curves read for 1979            Figure     Value   Location

------------------------------------------------------------

N(x) * T / P                     SP 1    0.08524   (A,B,C,D)

M(x)     / P                     SP 1    0.10837   (A,B,C,D)

N(x) * T * SQRT(Rm * T ) / MC    SM 1    0.13874   (A,B,C,D)

M(x)     * SQRT(Rm * T ) / MC    SM 1    0.30529   (A,B,C,D)

N(x) * T * SQRT(Rm * T ) / ML    SM 1    0.13874   (A,B,C,D)

M(x)     * SQRT(Rm * T ) / ML    SM 1    0.30529   (A,B,C,D)

N(y) * T / P                     SP 1    0.14151   (A,B,C,D)

M(y)     / P                     SP 1    0.03726   (A,B,C,D)

N(y) * T * SQRT(Rm * T ) / MC    SM 1    0.08279   (A,B,C,D)

M(y)     * SQRT(Rm * T ) / MC    SM 1    0.11593   (A,B,C,D)

N(y) * T * SQRT(Rm * T ) / ML    SM 1    0.08279   (A,B,C,D)

M(y)     * SQRT(Rm * T ) / ML    SM 1    0.11593   (A,B,C,D)

Stress Concentration Factors: Kn = 1.00,   Kb = 1.00

Stresses in the Vessel at the Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Rad.  Memb. P  |      2.4|      2.4|      2.4|      2.4|      2.4|      2.4|      2.4|      2.4|

Rad.  Bend. P  |     18.1|    -18.1|     18.1|    -18.1|     18.1|    -18.1|     18.1|    -18.1|

Rad.  Memb. MC |      0.0|      0.0|      0.0|      0.0|    -13.1|    -13.1|     13.1|     13.1|

Rad.  Memb. MC |      0.0|      0.0|      0.0|      0.0|   -172.8|    172.8|    172.8|   -172.8|

Rad.  Memb. ML |    -13.1|    -13.1|     13.1|     13.1|      0.0|      0.0|      0.0|      0.0|

Rad.  Bend. ML |   -172.8|    172.8|    172.8|   -172.8|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Rad.  Str.|   -165.4|    144.0|    206.3|   -175.4|   -165.4|    144.0|    206.3|   -175.4|

------------------------------------------------------------------------------------------------

Tang. Memb. P  |      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|      3.9|

Tang. Bend. P  |      6.2|     -6.2|      6.2|     -6.2|      6.2|     -6.2|      6.2|     -6.2|

Tang. Memb. MC |      0.0|      0.0|      0.0|      0.0|     -7.8|     -7.8|      7.8|      7.8|

Tang. Bend. MC |      0.0|      0.0|      0.0|      0.0|    -65.6|     65.6|     65.6|    -65.6|

Tang. Memb. ML |     -7.8|     -7.8|      7.8|      7.8|      0.0|      0.0|      0.0|      0.0|

Tang. Bend. ML |    -65.6|     65.6|     65.6|    -65.6|      0.0|      0.0|      0.0|      0.0|

|                                                                               |

Tot. Tang. Str.|    -63.3|     55.5|     83.6|    -60.1|    -63.3|     55.5|     83.6|    -60.1|

------------------------------------------------------------------------------------------------

Shear VC |      2.2|      2.2|     -2.2|     -2.2|      0.0|      0.0|      0.0|      0.0|

Shear VL |      0.0|      0.0|      0.0|      0.0|     -2.2|     -2.2|      2.2|      2.2|

Shear MT |     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|

|                                                                               |

Tot. Shear|     12.4|     12.4|      8.0|      8.0|      8.0|      8.0|     12.4|     12.4|

------------------------------------------------------------------------------------------------

Str. Int. |    166.9|    145.7|    206.8|    175.9|    166.0|    144.7|    207.5|    176.7|

------------------------------------------------------------------------------------------------

WRC 107/537 Stress Summations:

Vessel Stress Summation at Attachment Junction (N./mm²)

|                    Stress Intensity Values at                                 |

Type of        |                                                                               |

Stress    Load |       Au        Al        Bu        Bl        Cu        Cl        Du        Dl|

------------------------------------------------------------------------------------------------

Rad.  Pm (SUS) |      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|

Rad.  Pl (SUS) |    -10.7|    -10.7|     15.5|     15.5|    -10.7|    -10.7|     15.5|     15.5|

Rad.  Q  (SUS) |   -154.7|    154.7|    190.8|   -190.8|   -154.7|    154.7|    190.8|   -190.8|

------------------------------------------------------------------------------------------------

Long. Pm (SUS) |      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|

Long. Pl (SUS) |     -3.9|     -3.9|     11.7|     11.7|     -3.9|     -3.9|     11.7|     11.7|

Long. Q  (SUS) |    -59.4|     59.4|     71.8|    -71.8|    -59.4|     59.4|     71.8|    -71.8|

------------------------------------------------------------------------------------------------

Shear Pm (SUS) |      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|      0.0|

Shear Pl (SUS) |      2.2|      2.2|     -2.2|     -2.2|     -2.2|     -2.2|      2.2|      2.2|

Shear Q  (SUS) |     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|     10.2|

Pm (SUS)       |      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|      5.6|

------------------------------------------------------------------------------------------------

Pm+Pl (SUS)    |      8.1|      8.1|     22.0|     22.0|      8.1|      8.1|     22.0|     22.0|

------------------------------------------------------------------------------------------------

Pm+Pl+Q (Total)|    161.3|    151.3|    212.4|    170.3|    160.4|    150.3|    213.1|    171.1|

Vessel Stress Summation Comparison (N./mm²):

Type of        |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.    |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          5.60            100.50      |      Passed     |

Pm+Pl (SUS)    |         22.04            150.75      |      Passed     |

Pm+Pl+Q (TOTAL)|        213.11            301.50      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N6A                               From:   20

Pressure for Reinforcement Calculations        P       0.542  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                    1800.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                           90.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Flange Class                                             300

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N6A

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.54*500/(100*1-0.6*0.54)

= 0.2697 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.54*30.2/(100*1+0.4*0.54)

= 0.0163 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N6A.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0163 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.2697 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.016, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N6A

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.9106 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.9106 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N6B                               From:   20

Pressure for Reinforcement Calculations        P       0.710  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                     150.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                           90.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Flange Class                                             300

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N6B

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.71*500/(100*1-0.6*0.71)

= 0.3534 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.71*30.2/(100*1+0.4*0.71)

= 0.0213 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N6B.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0213 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.3534 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.021, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N6B

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.9106 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.9106 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N4A                               From:   20

Pressure for Reinforcement Calculations        P       0.542  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                    1800.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                          180.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Flange Class                                             300

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N4A

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.54*500/(100*1-0.6*0.54)

= 0.2697 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.54*30.2/(100*1+0.4*0.54)

= 0.0163 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N4A.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0163 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.2697 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.016, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N4A

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.9106 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.9106 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N4B                               From:   20

Pressure for Reinforcement Calculations        P       0.710  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                     150.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                          180.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Flange Class                                             300

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N4B

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.71*500/(100*1-0.6*0.71)

= 0.3534 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.71*30.2/(100*1+0.4*0.71)

= 0.0213 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N4B.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0213 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.3534 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.021, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N4B

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.9106 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.9106 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: M1                                From:   20

Pressure for Reinforcement Calculations        P       0.637  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                     864.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                          270.00  deg

Diameter                                            600.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        10S

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Pad Material                                     SA-240 316L

Pad Allowable Stress at Temperature           Sp      100.50  N./mm²

Pad Allowable Stress At Ambient              Spa      115.00  N./mm²

Diameter of Pad along vessel surface          Dp   1050.0000  mm.

Thickness of Pad                              te      6.0000  mm.

Weld leg size between Pad and Shell           Wp      6.0000  mm.

Groove weld depth between Pad and Nozzle    Wgpn      6.0000  mm.

Reinforcing Pad Width                               220.2000  mm.

This is a Manway or Access Opening.

Flange Class                                             150

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: M1

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation          609.600  mm.

Actual Thickness Used in Calculation                   6.350  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.64*500/(100*1-0.6*0.64)

= 0.3172 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.64*305/(100*1+0.4*0.64)

= 0.1931 mm.

UG-40 Modified by Appendix 1-7, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl  1193.7999  mm.

Parallel to Vessel Wall, opening length              d   596.9000  mm.

Normal to Vessel Wall (Thickness Limit), pad side Tlwp    15.0000  mm.

Weld Strength Reduction Factor [fr1]:

= min( 1, Sn/Sv )

= min( 1, 100/100 )

= 1.000

Weld Strength Reduction Factor [fr2]:

= min( 1, Sn/Sv )

= min( 1, 100/100 )

= 1.000

Weld Strength Reduction Factor [fr4]:

= min( 1, Sp/Sv )

= min( 1, 100/100 )

= 1.000

Weld Strength Reduction Factor [fr3]:

= min( fr2, fr4 )

= min( 1, 1 )

= 1.000

Results of Nozzle Reinforcement Area Calculations: (cm²)

AREA AVAILABLE, A1 to A5     |      MAWP|  External|     Mapnc|

---------------------------------------------------------------

Area Required             Ar |     1.893|        NA|        NA|

Area in Shell             A1 |    33.921|        NA|        NA|

Area in Nozzle Wall       A2 |     1.847|        NA|        NA|

Area in Inward Nozzle     A3 |     0.000|        NA|        NA|

Area in Welds    A41+A42+A43 |     0.720|        NA|        NA|

Area in Element           A5 |    26.424|        NA|        NA|

---------------------------------------------------------------

TOTAL AREA AVAILABLE    Atot |    62.912|        NA|        NA|

The MAWP Case Governs the Analysis.

Nozzle Angle Used in Area Calculations                 90.00  degs.

The area available without a pad is Sufficient.

The area available with the given pad is Sufficient.

Area Required [A]:

= ( d * tr*F + 2 * tn * tr*F * (1-fr1) ) UG-37(c)

= (597*0.32*1+2*6.35*0.32*1*(1-1))

= 1.893 cm²

Reinforcement Areas per Figure UG-37.1

Area Available in Shell [A1]:

= d( E1*t - F*tr ) - 2 * tn( E1*t - F*tr ) * ( 1 - fr1 )

= 597 ( 1 * 6 - 1 * 0.32 ) - 2 * 6.35

( 1 * 6 - 1 * 0.32 ) * ( 1 - 1 )

= 33.921 cm²

Area Available in Nozzle Wall Projecting Outward [A2]:

= ( 2 * Tlwp ) * ( tn - trn ) * fr2

= ( 2 * 15 ) * ( 6.35 - 0.19 ) * 1

= 1.847 cm²

Area Available in Welds [A41 + A42 + A43]:

= Wo² * fr3 + (Wi-can/0.707)² * fr2 + Wp² * fr4

= 6² * 1 + (0 )² * 1 + 6² * 1

= 0.720 cm²

Area Available in Element [A5]:

= (min(Dp,DL)-(Nozzle OD))*(min(tp,Tlwp,te)) * fr4

= ( 1050 - 610 ) * 6 * 1

= 26.424 cm²

UG-45 Minimum Nozzle Neck Thickness Requirement:

Wall Thickness for Internal/External pressures      ta = 0.1931 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= ta

= 0.1931 mm.

Available Nozzle Neck Thickness = 0.875 * 6.350 = 5.556 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

UG-40 Modified by Appendix 1-7, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl   895.3500  mm.

Parallel to Vessel Wall, opening length              d   447.6750  mm.

Normal to Vessel Wall (Thickness Limit), pad side Tlwp    15.0000  mm.

The Pad diameter is greater than the Diameter Limit. The excess will not be considered.

Reinforcement Areas for Large Nozzle (Per App 1-7(a)): (cm²  )

AREA AVAILABLE, A1 to A6              |    No Pad |  With Pad |

---------------------------------------------------------------

Area Required [Internal Pressure]  AR |     1.262 |     1.262 |

Area Available in Shell            A1 |    16.960 |    16.960 |

Area Available in Nozzle Wall      A2 |     1.847 |     1.847 |

Area Available in Inward Nozzle    A3 |     0.000 |     0.000 |

Area Available in Welds            A4 |     0.360 |     0.360 |

Area Available in Pad              A5 |     0.000 |    17.145 |

Area Available in Hub              A6 |     0.000 |     0.000 |

TOTAL AREA AVAILABLE             Atot |    19.167 |    36.312 |

The area available without a pad is Sufficient.

The area available with the given pad is Sufficient.

The Nozzle does not fall within the scope of the diameter requirements

of App. 1-7(b). Therefore, the Membrane and Bending stress checks

are not required.

Weld Size Calculations, Description: M1

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Intermediate Calc. for pad/shell Welds   TminPad      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Pad Weld             3.0000 = 0.5*TminPad  4.2420 = 0.7 * Wp mm.

Weld Strength and Weld Loads per UG-41.1, Sketch (a) or (b)

Weld Load [W]:

= max( 0, (A-A1+2*tn*fr1*(E1*t-tr))Sv)

= max( 0, (1.89 - 33.9 + 2 * 6.35 * 1 *

(1 * 6 - 0.32 ) )100 )

= max( 0, -314465.50) N

Note: F is always set to 1.0 throughout the calculation.

Weld Load [W1]:

= (A2+A5+A4-(Wi-Can/.707)²*fr2)*Sv

= ( 1.85 + 26.4 + 0.72 - 0 * 1 ) * 100

= 291212.50 N

Weld Load [W2]:

= (A2 + A3 + A4 + (2 * tn * t * fr1)) * Sv

= ( 1.85 + 0 + 0.36 + ( 0.76 ) ) * 100

= 29823.91 N

Weld Load [W3]:

= (A2+A3+A4+A5+(2*tn*t*fr1))*S

= ( 1.85 + 0 + 0.72 + 26.4 + ( 0.76 ) ) * 100

= 298866.72 N

Strength of Connection Elements for Failure Path Analysis

Shear, Outward Nozzle Weld [Sonw]:

= (pi/2) * Dlo * Wo * 0.49 * Snw

= ( 3.14/2.0 ) * 610 * 6 * 0.49 * 100

= 282905. N

Shear, Pad Element Weld [Spew]:

= (pi/2) * DP * WP * 0.49 * SEW

= ( 3.14/2.0 ) * 1050 * 6 * 0.49 * 100

= 487082. N

Shear, Nozzle Wall [Snw]:

= (pi *( Dlr + Dlo )/4 ) * ( Thk - Can ) * 0.7 * Sn

= (3.14 * 302 ) * ( 6.35 - 0 ) * 0.7 * 100

= 423271. N

Tension, Pad Groove Weld [Tpgw]:

= ( pi/2) * Dlo * Wgpn * 0.74 * Seg

=  (3.14/2 ) * 610 * 6 * 0.74 * 100

= 427245. N

Tension, Shell Groove Weld [Tngw]:

= (pi/2) * Dlo * ( T - cas ) * 0.74 * Sng

= ( 3.14/2.0 ) * 610 * ( 6 - 0 ) * 0.74 * 100

= 427245. N

See UG-41 and UW-15 for clarification of allowable stresses.

Strength of Failure Paths:

Failure Path [Path 1-1]:

= ( SPEW + SNW ) = ( 487082 + 423271 ) = 910353 N

Failure Path [Path 2-2]:

= ( Sonw + Tpgw + Tngw + Sinw )

= ( 282905 + 427245 + 427245 + 0 ) = 1137395 N

Failure Path [Path 3-3]:

= ( Spew + Tngw + Sinw )

= ( 487082 + 427245 + 0 ) = 914327 N

Summary of Failure Path Calculations:

Path 1-1 = 910353  N  , must exceed W = 0 N          or W1 = 291212 N

Path 2-2 = 1137395 N  , must exceed W = 0 N          or W2 = 29823 N

Path 3-3 = 914327  N  , must exceed W = 0 N          or W3 = 298866 N

Maximum Allowable Pressure for this Nozzle at this Location:

Converged Maximum Allowable Pressure in the Operating case: 0.52 bars

Note: The MAWP of this junction was limited by the parent Shell/Head.

The Drop for this Nozzle is : 103.6454 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 259.6454 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N1                                From:   20

Pressure for Reinforcement Calculations        P       0.542  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                    1800.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                          225.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Pad Material                                     SA-240 316L

Pad Allowable Stress at Temperature           Sp      100.50  N./mm²

Pad Allowable Stress At Ambient              Spa      115.00  N./mm²

Diameter of Pad along vessel surface          Dp    162.0000  mm.

Thickness of Pad                              te      6.0000  mm.

Weld leg size between Pad and Shell           Wp      6.0000  mm.

Groove weld depth between Pad and Nozzle    Wgpn      6.0000  mm.

Reinforcing Pad Width                                50.8375  mm.

Flange Class                                             150

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N1

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.54*500/(100*1-0.6*0.54)

= 0.2697 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.54*30.2/(100*1+0.4*0.54)

= 0.0163 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), pad side Tlwp    15.0000  mm.

The Pad diameter is greater than the Diameter Limit. The excess will not be considered.

Taking a UG-36(c)(3)(a) exemption for nozzle: N1.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0163 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.2697 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.016, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

Stresses on Nozzle due to External and Pressure Loads per the ASME B31.3 Piping Code,

(see 319.4.4 and 302.3.5):

--------------------------------------------------------

Sustained  :     21.3,  Allowable :    100.5 N./mm²  Passed

Expansion  :      0.0,  Allowable :    248.0 N./mm²  Passed

Occasional :      0.1,  Allowable :    133.7 N./mm²  Passed

Shear      :     16.8,  Allowable :     70.3 N./mm²  Passed

The number of cycles on this nozzle was assumed to be 7000 or less for the determination of

the expansion stress allowable.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N1

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Intermediate Calc. for pad/shell Welds   TminPad      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Pad Weld             3.0000 = 0.5*TminPad  4.2420 = 0.7 * Wp mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.9106 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.9106 mm.

Input Echo, WRC297 Item     1,            Description: N1            :

Diameter Basis for Cylindrical Shell                  ID

Shell Corrosion Allowance                             0.0000  mm.

Shell Diameter                                      1000.000  mm.

Shell Thickness                                       6.0000  mm.

Shell Stress Concentration Factor                      1.000

Vessel Material                                  SA-240 316L

Vessel Cold S.I. Allowable                   Smc      115.00  N./mm²

Vessel Hot  S.I. Allowable                   Smh      100.46  N./mm²

Diameter Basis for Nozzle                             OD

Nozzle Corrosion Allowance                            0.0000  mm.

Nozzle Diameter                                       60.325  mm.

Nozzle Thickness                                      8.7376  mm.

Nozzle Stress Concentration Factor                     1.000

Nozzle Material                                SA-312 TP316L

Nozzle Cold S.I. Allowable                  SNmc      115.00  N./mm²

Nozzle Hot  S.I. Allowable                  SNmh      100.50  N./mm²

Diameter of Reinforcing Pad                          162.000  mm.

Thickness of Reinforcing Pad                          6.0000  mm.

Note:

External Forces and Moments in WRC 107/537 Convention:

These loads are assumed to be SUStained loads.

Design Internal Pressure                      Dp        0.54  bars

Radial Load                                    P    -1000.00  N

Circumferential Shear                         Vc     1225.00  N

Longitudinal Shear                            Vl     1225.00  N

Circumferential Moment                        Mc      250.00  N-m

Longitudinal Moment                           Ml      250.00  N-m

Torsional Moment                              Mt      350.00  N-m

Include Axial Pressure Thrust                             No

Include Pressure Stress Indices per Div. 2                No

Local Loads applied at end of Nozzle/Attachment           No

Warning - The ratio, Dn/Tn (6.904) is < 10

check the limitations of WRC bulletin 297.

Stress Computations at the Edge of the Nozzle:

Stress Attenuation Diameter (for Insert Plates) per WRC 297:

= NozzleOD + 2 * 1.65 * sqrt( Rmean( t - ca ) )

= 60.325 + 2 * 1.65 * sqrt( 506.0( 6.0 - 0.0 ) )

= 242.155 mm.

WRC 297 Curve Access Parameters:

Vessel Mean Diameter      (D) =   1012.000 mm.

Nozzle Outside Diameter   (d) =     60.325 mm.

Vessel Thickness used     (T) =     12.000 mm.

Nozzle Thickness used     (t) =      8.738 mm.

T / t                         =      1.373

d / t                         =     10.000

Lambda = [(d/D) * sqrt(D/T)]  = 0.547

Nr/P    =      0.178

Mr/P    =      0.234

M0/P    =      0.098

N0/P    =      0.163

MrD/Mc  =      0.265

NrDL/Mc =      0.165

M0d/Mc  =      0.137

N0DL/Mc =     -0.096

MrD/Ml  =      0.236

NrDL/Ml =      0.155

M0D/Ml  =      0.120

N0DL/Ml =      0.020

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses Normal to          Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         4       -4        4       -4

Outplane Membrane (Mc)         0        0        0        0

Outplane Bending  (Mc)         0        0        0        0

Outplane Membrane (ML)         0        0        0        0

Outplane Bending  (ML)       -20       20       20      -20

Normal Pressure Stress         2        2        2        2

----------------------- -------- -------- -------- --------

Outplane Stress Summary      -13       19       28      -20

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses parallel to        Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          1        1        1        1

Inplane Bending  (P )          9       -9        9       -9

Inplane Membrane (Mc)          0        0        0        0

Inplane Bending  (Mc)          0        0        0        0

Inplane Membrane (ML)         -4       -4        4        4

Inplane Bending  (ML)        -40       40       40      -40

Inplane Pressure Stress        1        1        1        1

----------------------- -------- -------- -------- --------

Inplane Stress Summary       -33       28       57      -43

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Shear stress normal to      Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          1        1       -1       -1

Outplane  Shear  (Vl)          0        0        0        0

Outplane  Shear  (Mt)          5        5        5        5

----------------------- -------- -------- -------- --------

Shear Stress Summary           6        6        4        4

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stress Intensities          Top      Top   Bottom   Bottom

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        34       31       57       44

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses Normal to         Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         4       -4        4       -4

Outplane Membrane (Mc)         2        2       -2       -2

Outplane Bending  (Mc)       -23       23       23      -23

Outplane Membrane (ML)         0        0        0        0

Outplane Bending  (ML)         0        0        0        0

Normal Pressure Stress         1        1        1        1

----------------------- -------- -------- -------- --------

Outplane Stress Summary      -14       24       27      -28

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses parallel to       Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          1        1        1        1

Inplane Bending  (P )          9       -9        9       -9

Inplane Membrane (Mc)         -4       -4        4        4

Inplane Bending  (Mc)        -45       45       45      -45

Inplane Membrane (ML)          0        0        0        0

Inplane Bending  (ML)          0        0        0        0

Inplane Pressure Stress        2        2        2        2

----------------------- -------- -------- -------- --------

Inplane Stress Summary       -37       34       63      -47

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Shear stress normal to     Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          0        0        0        0

Outplane  Shear  (Vl)         -1       -1        1        1

Torsional Shear  (Mt)          5        5        5        5

----------------------- -------- -------- -------- --------

Shear Stress Summary           4        4        6        6

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stress Intensities         Left     Left    Right    Right

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        37       36       64       49

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses in the             Top      Top   Bottom   Bottom

hoop direction)          Outside   Inside  Outside   Inside

-----------------------------------------------------------

Hoop Membrane (P )             1        1        1        1

Hoop Bending  (P )             0        0        0        0

Hoop Membrane (Mc)             0        0        0        0

Hoop Bending  (Mc)             0        0        0        0

Hoop Membrane (ML)             0        0        0        0

Hoop Bending  (ML)             0        0        0        0

Hoop Pressure Stress           0        0        0        0

----------------------- -------- -------- -------- --------

Hoop Stress Summary            0        0        1        1

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses Normal to          Top      Top   Bottom   Bottom

pipe cross-section)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Axial Membrane (P )            0        0        0        0

Axial Bending  (P )           11      -11       11      -11

Axial Membrane (Mc)            0        0        0        0

Axial Bending  (Mc)            0        0        0        0

Axial Membrane (ML)          -15      -15       15       15

Axial Bending  (ML)          -51       51       51      -51

Axial Pressure Stress          0        0        0        0

----------------------- -------- -------- -------- --------

Axial Stress Summary         -54       25       79      -46

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Shear stress)           Outside   Inside  Outside   Inside

-----------------------------------------------------------

Shear due to (Vc)              1        1       -1       -1

Shear due to (Vl)              0        0        0        0

Shear due to Torsion           6        6        6        6

----------------------- -------- -------- -------- --------

Shear Stress Summary           8        8        5        5

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stress Intensities      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        58       29       79       49

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses in the            Left     Left    Right    Right

hoop direction)          Outside   Inside  Outside   Inside

-----------------------------------------------------------

Hoop Membrane (P )             1        1        1        1

Hoop Bending  (P )             0        0        0        0

Hoop Membrane (Mc)             2        2       -2       -2

Hoop Bending  (Mc)             0        0        0        0

Hoop Membrane (ML)             0        0        0        0

Hoop Bending  (ML)             0        0        0        0

Hoop Pressure Stress           0        0        0        0

----------------------- -------- -------- -------- --------

Hoop Stress Summary            4        4       -1       -1

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses Normal to         Left     Left    Right    Right

pipe cross-section)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Axial Membrane (P )            0        0        0        0

Axial Bending  (P )           11      -11       11      -11

Axial Membrane (Mc)          -15      -15       15       15

Axial Bending  (Mc)          -59       59       59      -59

Axial Membrane (ML)            0        0        0        0

Axial Bending  (ML)            0        0        0        0

Axial Pressure Stress          0        0        0        0

----------------------- -------- -------- -------- --------

Axial Stress Summary         -62       33       87      -54

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Shear stress)           Outside   Inside  Outside   Inside

-----------------------------------------------------------

Shear due to (Vc)              0        0        0        0

Shear due to (Vl)             -1       -1        1        1

Shear due to Torsion           6        6        6        6

----------------------- -------- -------- -------- --------

Shear Stress Summary           5        5        8        8

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stress Intensities      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        67       34       90       55

Stress Computations at the Edge of the Reinforcing Pad:

Stress Attenuation Diameter (for Insert Plates) per WRC 297:

= NozzleOD + 2 * 1.65 * sqrt( Rmean( t - ca ) )

= 60.325 + 2 * 1.65 * sqrt( 503.0( 6.0 - 0.0 ) )

= 241.615 mm.

WRC 297 Curve Access Parameters:

Vessel Mean Diameter      (D) =   1006.000 mm.

Nozzle Outside Diameter   (d) =    162.000 mm.

Vessel Thickness used     (T) =      6.000 mm.

Nozzle Thickness used     (t) =     59.575 mm.

T / t                         =      0.101

d / t                         =     10.000

Lambda = [(d/D) * sqrt(D/T)]  = 2.085

Note: Re-pad thickness is added to nozzle thickness to simulate the Re-pad.

Nr/P    =      0.083

Mr/P    =      0.142

M0/P    =      0.042

N0/P    =      0.052

MrD/Mc  =      0.289

NrDL/Mc =      0.112

M0d/Mc  =      0.088

N0DL/Mc =      0.061

MrD/Ml  =      0.107

NrDL/Ml =      0.110

M0D/Ml  =      0.032

N0DL/Ml =      0.070

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses Normal to          Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         6       -6        6       -6

Outplane Membrane (Mc)         0        0        0        0

Outplane Bending  (Mc)         0        0        0        0

Outplane Membrane (ML)        -2       -2        2        2

Outplane Bending  (ML)        -8        8        8       -8

Normal Pressure Stress         4        4        4        4

----------------------- -------- -------- -------- --------

Outplane Stress Summary        1        4       24       -6

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses parallel to        Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          2        2        2        2

Inplane Bending  (P )         23      -23       23      -23

Inplane Membrane (Mc)          0        0        0        0

Inplane Bending  (Mc)          0        0        0        0

Inplane Membrane (ML)         -4       -4        4        4

Inplane Bending  (ML)        -27       27       27      -27

Inplane Pressure Stress        2        2        2        2

----------------------- -------- -------- -------- --------

Inplane Stress Summary        -3        3       60      -41

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Shear stress normal to      Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          0        0        0        0

Outplane  Shear  (Vl)          0        0        0        0

Outplane  Shear  (Mt)          1        1        1        1

----------------------- -------- -------- -------- --------

Shear Stress Summary           2        2        0        0

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stress Intensities          Top      Top   Bottom   Bottom

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress         7        6       60       41

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses Normal to         Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         6       -6        6       -6

Outplane Membrane (Mc)        -2       -2        2        2

Outplane Bending  (Mc)       -22       22       22      -22

Outplane Membrane (ML)         0        0        0        0

Outplane Bending  (ML)         0        0        0        0

Normal Pressure Stress         2        2        2        2

----------------------- -------- -------- -------- --------

Outplane Stress Summary      -14       16       35      -23

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses parallel to       Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          2        2        2        2

Inplane Bending  (P )         23      -23       23      -23

Inplane Membrane (Mc)         -4       -4        4        4

Inplane Bending  (Mc)        -74       74       74      -74

Inplane Membrane (ML)          0        0        0        0

Inplane Bending  (ML)          0        0        0        0

Inplane Pressure Stress        4        4        4        4

----------------------- -------- -------- -------- --------

Inplane Stress Summary       -48       52      109      -86

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Shear stress normal to     Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          0        0        0        0

Outplane  Shear  (Vl)          0        0        0        0

Torsional Shear  (Mt)          1        1        1        1

----------------------- -------- -------- -------- --------

Shear Stress Summary           0        0        2        2

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stress Intensities         Left     Left    Right    Right

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        48       52      109       86

WRC 297 Stress Summations per ASME Sec. VIII Div. 2:

Vessel Stress Summation at Vessel-Nozzle Junction  (N./mm²):

Type of     |              Stress Values at                          |

Stress Int.  |                                                        |

Location    |    Au     Al     Bu     Bl     Cu     Cl     Du     Dl |

-------------------------------------------------------------------------

Circ. Pm (SUS) |      2      2      2      2      2      2      2      2|

Circ. Pl (SUS) |      1      1      1      1     -3     -3      5      5|

Circ. Q  (SUS) |    -16     16     24    -24    -36     36     54    -54|

-------------------------------------------------------------------------

Long. Pm (SUS) |      1      1      1      1      1      1      1      1|

Long. Pl (SUS) |     -3     -3      5      5      3      3     -1     -1|

Long. Q  (SUS) |    -31     31     49    -49    -19     19     27    -27|

-------------------------------------------------------------------------

Shear Pm (SUS) |      0      0      0      0      0      0      0      0|

Shear Pl (SUS) |      1      1     -1     -1     -1     -1      1      1|

Shear Q  (SUS) |      5      5      5      5      5      5      5      5|

Pm (SUS)       |    2.0    2.0    2.0    2.0    2.0    2.0    2.0    2.0|

-------------------------------------------------------------------------

Pm+Pl (SUS)    |    5.4    5.4    6.3    6.3    5.4    5.4    7.3    7.3|

-------------------------------------------------------------------------

Pm+Pl+Q (Total)|   34.7   31.8   55.6   43.7   37.7   36.2   62.0   48.7|

Vessel Stress Summation Comparison  (N./mm²):

Type of     |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.  |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          2.00            100.46      |      Passed     |

Pm+Pl (SUS)    |          7.28            150.69      |      Passed     |

Pm+Pl+Q (TOTAL)|         62.03            301.37      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

WRC 297 Stress Summations per ASME Sec. VIII Div. 2:

Vessel Stress Summation at Reinforcing Pad Edge  (N./mm²):

Type of     |              Stress Values at                          |

Stress Int.  |                                                        |

Location    |    Au     Al     Bu     Bl     Cu     Cl     Du     Dl |

-------------------------------------------------------------------------

Circ. Pm (SUS) |      4      4      4      4      4      4      4      4|

Circ. Pl (SUS) |     -1     -1      3      3     -2     -2      6      6|

Circ. Q  (SUS) |     -2      2     14    -14    -51     51     97    -97|

-------------------------------------------------------------------------

Long. Pm (SUS) |      2      2      2      2      2      2      2      2|

Long. Pl (SUS) |     -2     -2      6      6     -1     -1      3      3|

Long. Q  (SUS) |     -4      4     50    -50    -16     16     28    -28|

-------------------------------------------------------------------------

Shear Pm (SUS) |      0      0      0      0      0      0      0      0|

Shear Pl (SUS) |      0      0      0      0      0      0      0      0|

Shear Q  (SUS) |      1      1      1      1      1      1      1      1|

Pm (SUS)       |    4.0    4.0    4.0    4.0    4.0    4.0    4.0    4.0|

-------------------------------------------------------------------------

Pm+Pl (SUS)    |    3.0    3.0    8.0    8.0    2.0    2.0   10.0   10.0|

-------------------------------------------------------------------------

Pm+Pl+Q (Total)|    5.4    5.6   58.0   42.0   49.0   53.0  107.0   87.0|

Vessel Stress Summation Comparison  (N./mm²):

Type of     |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.  |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          4.00            100.46      |      Passed     |

Pm+Pl (SUS)    |         10.00            150.69      |      Passed     |

Pm+Pl+Q (TOTAL)|        107.01            301.37      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

WRC 297 Stress Summations per ASME Sec. VIII Div. 2:

Nozzle Stress Summation at Vessel-Nozzle Junction  (N./mm²):

Type of     |              Stress Values at                          |

Stress Int.  |                                                        |

Location    |    Au     Al     Bu     Bl     Cu     Cl     Du     Dl |

-------------------------------------------------------------------------

Circ. Pm (SUS) |      0      0      0      0      0      0      0      0|

Circ. Pl (SUS) |      1      1      1      1      3      3     -1     -1|

Circ. Q  (SUS) |      0      0      0      0      0      0      0      0|

-------------------------------------------------------------------------

Long. Pm (SUS) |      0      0      0      0      0      0      0      0|

Long. Pl (SUS) |    -15    -15     15     15    -15    -15     15     15|

Long. Q  (SUS) |    -40     40     62    -62    -48     48     70    -70|

-------------------------------------------------------------------------

Shear Pm (SUS) |      0      0      0      0      0      0      0      0|

Shear Pl (SUS) |      1      1     -1     -1     -1     -1      1      1|

Shear Q  (SUS) |      6      6      6      6      6      6      6      6|

Pm (SUS)       |    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0|

-------------------------------------------------------------------------

Pm+Pl (SUS)    |   16.1   16.1   15.1   15.1   18.1   18.1   16.1   16.1|

-------------------------------------------------------------------------

Pm+Pl+Q (Total)|   57.7   27.8   77.3   49.0   66.8   33.8   87.1   55.9|

Nozzle Stress Summation Comparison  (N./mm²):

Type of     |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.  |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          0.00            100.50      |      Passed     |

Pm+Pl (SUS)    |         18.11            150.75      |      Passed     |

Pm+Pl+Q (TOTAL)|         87.13            301.50      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N7                                From:   20

Pressure for Reinforcement Calculations        P       0.542  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                    1800.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                          135.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Pad Material                                     SA-240 316L

Pad Allowable Stress at Temperature           Sp      100.46  N./mm²

Pad Allowable Stress At Ambient              Spa      115.00  N./mm²

Diameter of Pad along vessel surface          Dp    162.0000  mm.

Thickness of Pad                              te      6.0000  mm.

Weld leg size between Pad and Shell           Wp      6.0000  mm.

Groove weld depth between Pad and Nozzle    Wgpn      6.0000  mm.

Reinforcing Pad Width                                50.8375  mm.

Flange Class                                             150

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N7

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.54*500/(100*1-0.6*0.54)

= 0.2697 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.54*30.2/(100*1+0.4*0.54)

= 0.0163 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), pad side Tlwp    15.0000  mm.

The Pad diameter is greater than the Diameter Limit. The excess will not be considered.

Taking a UG-36(c)(3)(a) exemption for nozzle: N7.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0163 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.2697 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.016, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

Stresses on Nozzle due to External and Pressure Loads per the ASME B31.3 Piping Code,

(see 319.4.4 and 302.3.5):

--------------------------------------------------------

Sustained  :     21.3,  Allowable :    100.5 N./mm²  Passed

Expansion  :      0.0,  Allowable :    248.0 N./mm²  Passed

Occasional :      0.1,  Allowable :    133.7 N./mm²  Passed

Shear      :     16.7,  Allowable :     70.3 N./mm²  Passed

The number of cycles on this nozzle was assumed to be 7000 or less for the determination of

the expansion stress allowable.

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N7

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Intermediate Calc. for pad/shell Welds   TminPad      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Pad Weld             3.0000 = 0.5*TminPad  4.2420 = 0.7 * Wp mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.9106 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.9106 mm.

Input Echo, WRC297 Item     1,            Description: N7            :

Diameter Basis for Cylindrical Shell                  ID

Shell Corrosion Allowance                             0.0000  mm.

Shell Diameter                                      1000.000  mm.

Shell Thickness                                       6.0000  mm.

Shell Stress Concentration Factor                      1.000

Vessel Material                                  SA-240 316L

Vessel Cold S.I. Allowable                   Smc      115.00  N./mm²

Vessel Hot  S.I. Allowable                   Smh      100.46  N./mm²

Diameter Basis for Nozzle                             OD

Nozzle Corrosion Allowance                            0.0000  mm.

Nozzle Diameter                                       60.325  mm.

Nozzle Thickness                                      8.7376  mm.

Nozzle Stress Concentration Factor                     1.000

Nozzle Material                                SA-312 TP316L

Nozzle Cold S.I. Allowable                  SNmc      115.00  N./mm²

Nozzle Hot  S.I. Allowable                  SNmh      100.50  N./mm²

Diameter of Reinforcing Pad                          162.000  mm.

Thickness of Reinforcing Pad                          6.0000  mm.

Note:

External Forces and Moments in WRC 107/537 Convention:

These loads are assumed to be SUStained loads.

Design Internal Pressure                      Dp        0.54  bars

Radial Load                                    P    -1000.00  N

Circumferential Shear                         Vc     1225.00  N

Longitudinal Shear                            Vl     1125.00  N

Circumferential Moment                        Mc      250.00  N-m

Longitudinal Moment                           Ml      250.00  N-m

Torsional Moment                              Mt      350.00  N-m

Include Axial Pressure Thrust                             No

Include Pressure Stress Indices per Div. 2                No

Local Loads applied at end of Nozzle/Attachment           No

Warning - The ratio, Dn/Tn (6.904) is < 10

check the limitations of WRC bulletin 297.

Stress Computations at the Edge of the Nozzle:

Stress Attenuation Diameter (for Insert Plates) per WRC 297:

= NozzleOD + 2 * 1.65 * sqrt( Rmean( t - ca ) )

= 60.325 + 2 * 1.65 * sqrt( 506.0( 6.0 - 0.0 ) )

= 242.155 mm.

WRC 297 Curve Access Parameters:

Vessel Mean Diameter      (D) =   1012.000 mm.

Nozzle Outside Diameter   (d) =     60.325 mm.

Vessel Thickness used     (T) =     12.000 mm.

Nozzle Thickness used     (t) =      8.738 mm.

T / t                         =      1.373

d / t                         =     10.000

Lambda = [(d/D) * sqrt(D/T)]  = 0.547

Nr/P    =      0.178

Mr/P    =      0.234

M0/P    =      0.098

N0/P    =      0.163

MrD/Mc  =      0.265

NrDL/Mc =      0.165

M0d/Mc  =      0.137

N0DL/Mc =     -0.096

MrD/Ml  =      0.236

NrDL/Ml =      0.155

M0D/Ml  =      0.120

N0DL/Ml =      0.020

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses Normal to          Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         4       -4        4       -4

Outplane Membrane (Mc)         0        0        0        0

Outplane Bending  (Mc)         0        0        0        0

Outplane Membrane (ML)         0        0        0        0

Outplane Bending  (ML)       -20       20       20      -20

Normal Pressure Stress         2        2        2        2

----------------------- -------- -------- -------- --------

Outplane Stress Summary      -13       19       28      -20

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses parallel to        Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          1        1        1        1

Inplane Bending  (P )          9       -9        9       -9

Inplane Membrane (Mc)          0        0        0        0

Inplane Bending  (Mc)          0        0        0        0

Inplane Membrane (ML)         -4       -4        4        4

Inplane Bending  (ML)        -40       40       40      -40

Inplane Pressure Stress        1        1        1        1

----------------------- -------- -------- -------- --------

Inplane Stress Summary       -33       28       57      -43

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Shear stress normal to      Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          1        1       -1       -1

Outplane  Shear  (Vl)          0        0        0        0

Outplane  Shear  (Mt)          5        5        5        5

----------------------- -------- -------- -------- --------

Shear Stress Summary           6        6        4        4

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stress Intensities          Top      Top   Bottom   Bottom

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        34       31       57       44

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses Normal to         Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         4       -4        4       -4

Outplane Membrane (Mc)         2        2       -2       -2

Outplane Bending  (Mc)       -23       23       23      -23

Outplane Membrane (ML)         0        0        0        0

Outplane Bending  (ML)         0        0        0        0

Normal Pressure Stress         1        1        1        1

----------------------- -------- -------- -------- --------

Outplane Stress Summary      -14       24       27      -28

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses parallel to       Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          1        1        1        1

Inplane Bending  (P )          9       -9        9       -9

Inplane Membrane (Mc)         -4       -4        4        4

Inplane Bending  (Mc)        -45       45       45      -45

Inplane Membrane (ML)          0        0        0        0

Inplane Bending  (ML)          0        0        0        0

Inplane Pressure Stress        2        2        2        2

----------------------- -------- -------- -------- --------

Inplane Stress Summary       -37       34       63      -47

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Shear stress normal to     Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          0        0        0        0

Outplane  Shear  (Vl)          0        0        0        0

Torsional Shear  (Mt)          5        5        5        5

----------------------- -------- -------- -------- --------

Shear Stress Summary           4        4        6        6

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stress Intensities         Left     Left    Right    Right

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        38       36       64       49

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses in the             Top      Top   Bottom   Bottom

hoop direction)          Outside   Inside  Outside   Inside

-----------------------------------------------------------

Hoop Membrane (P )             1        1        1        1

Hoop Bending  (P )             0        0        0        0

Hoop Membrane (Mc)             0        0        0        0

Hoop Bending  (Mc)             0        0        0        0

Hoop Membrane (ML)             0        0        0        0

Hoop Bending  (ML)             0        0        0        0

Hoop Pressure Stress           0        0        0        0

----------------------- -------- -------- -------- --------

Hoop Stress Summary            0        0        1        1

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses Normal to          Top      Top   Bottom   Bottom

pipe cross-section)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Axial Membrane (P )            0        0        0        0

Axial Bending  (P )           11      -11       11      -11

Axial Membrane (Mc)            0        0        0        0

Axial Bending  (Mc)            0        0        0        0

Axial Membrane (ML)          -15      -15       15       15

Axial Bending  (ML)          -51       51       51      -51

Axial Pressure Stress          0        0        0        0

----------------------- -------- -------- -------- --------

Axial Stress Summary         -54       25       79      -46

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Shear stress)           Outside   Inside  Outside   Inside

-----------------------------------------------------------

Shear due to (Vc)              1        1       -1       -1

Shear due to (Vl)              0        0        0        0

Shear due to Torsion           6        6        6        6

----------------------- -------- -------- -------- --------

Shear Stress Summary           8        8        5        5

Nozzle Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stress Intensities      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        58       29       79       49

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses in the            Left     Left    Right    Right

hoop direction)          Outside   Inside  Outside   Inside

-----------------------------------------------------------

Hoop Membrane (P )             1        1        1        1

Hoop Bending  (P )             0        0        0        0

Hoop Membrane (Mc)             2        2       -2       -2

Hoop Bending  (Mc)             0        0        0        0

Hoop Membrane (ML)             0        0        0        0

Hoop Bending  (ML)             0        0        0        0

Hoop Pressure Stress           0        0        0        0

----------------------- -------- -------- -------- --------

Hoop Stress Summary            4        4       -1       -1

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses Normal to         Left     Left    Right    Right

pipe cross-section)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Axial Membrane (P )            0        0        0        0

Axial Bending  (P )           11      -11       11      -11

Axial Membrane (Mc)          -15      -15       15       15

Axial Bending  (Mc)          -59       59       59      -59

Axial Membrane (ML)            0        0        0        0

Axial Bending  (ML)            0        0        0        0

Axial Pressure Stress          0        0        0        0

----------------------- -------- -------- -------- --------

Axial Stress Summary         -62       33       87      -54

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Shear stress)           Outside   Inside  Outside   Inside

-----------------------------------------------------------

Shear due to (Vc)              0        0        0        0

Shear due to (Vl)             -1       -1        1        1

Shear due to Torsion           6        6        6        6

----------------------- -------- -------- -------- --------

Shear Stress Summary           5        5        8        8

Nozzle Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stress Intensities      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        67       34       90       55

Stress Computations at the Edge of the Reinforcing Pad:

Stress Attenuation Diameter (for Insert Plates) per WRC 297:

= NozzleOD + 2 * 1.65 * sqrt( Rmean( t - ca ) )

= 60.325 + 2 * 1.65 * sqrt( 503.0( 6.0 - 0.0 ) )

= 241.615 mm.

WRC 297 Curve Access Parameters:

Vessel Mean Diameter      (D) =   1006.000 mm.

Nozzle Outside Diameter   (d) =    162.000 mm.

Vessel Thickness used     (T) =      6.000 mm.

Nozzle Thickness used     (t) =     59.575 mm.

T / t                         =      0.101

d / t                         =     10.000

Lambda = [(d/D) * sqrt(D/T)]  = 2.085

Note: Re-pad thickness is added to nozzle thickness to simulate the Re-pad.

Nr/P    =      0.083

Mr/P    =      0.142

M0/P    =      0.042

N0/P    =      0.052

MrD/Mc  =      0.289

NrDL/Mc =      0.112

M0d/Mc  =      0.088

N0DL/Mc =      0.061

MrD/Ml  =      0.107

NrDL/Ml =      0.110

M0D/Ml  =      0.032

N0DL/Ml =      0.070

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses Normal to          Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         6       -6        6       -6

Outplane Membrane (Mc)         0        0        0        0

Outplane Bending  (Mc)         0        0        0        0

Outplane Membrane (ML)        -2       -2        2        2

Outplane Bending  (ML)        -8        8        8       -8

Normal Pressure Stress         4        4        4        4

----------------------- -------- -------- -------- --------

Outplane Stress Summary        1        4       24       -6

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stresses parallel to        Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          2        2        2        2

Inplane Bending  (P )         23      -23       23      -23

Inplane Membrane (Mc)          0        0        0        0

Inplane Bending  (Mc)          0        0        0        0

Inplane Membrane (ML)         -4       -4        4        4

Inplane Bending  (ML)        -27       27       27      -27

Inplane Pressure Stress        2        2        2        2

----------------------- -------- -------- -------- --------

Inplane Stress Summary        -3        3       60      -41

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Shear stress normal to      Top      Top   Bottom   Bottom

longitudinal plane)      Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          0        0        0        0

Outplane  Shear  (Vl)          0        0        0        0

Outplane  Shear  (Mt)          1        1        1        1

----------------------- -------- -------- -------- --------

Shear Stress Summary           2        2        0        0

Vessel Stresses  (N./mm²):

LONGITUDINAL PLANE            Au       Al       Bu       Bl

(Stress Intensities          Top      Top   Bottom   Bottom

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress         7        6       60       41

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses Normal to         Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane Membrane (P )         1        1        1        1

Outplane Bending  (P )         6       -6        6       -6

Outplane Membrane (Mc)        -2       -2        2        2

Outplane Bending  (Mc)       -22       22       22      -22

Outplane Membrane (ML)         0        0        0        0

Outplane Bending  (ML)         0        0        0        0

Normal Pressure Stress         2        2        2        2

----------------------- -------- -------- -------- --------

Outplane Stress Summary      -14       16       35      -23

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stresses parallel to       Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Inplane Membrane (P )          2        2        2        2

Inplane Bending  (P )         23      -23       23      -23

Inplane Membrane (Mc)         -4       -4        4        4

Inplane Bending  (Mc)        -74       74       74      -74

Inplane Membrane (ML)          0        0        0        0

Inplane Bending  (ML)          0        0        0        0

Inplane Pressure Stress        4        4        4        4

----------------------- -------- -------- -------- --------

Inplane Stress Summary       -48       52      109      -86

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Shear stress normal to     Left     Left    Right    Right

circumferential plane)   Outside   Inside  Outside   Inside

-----------------------------------------------------------

Outplane  Shear  (Vc)          0        0        0        0

Outplane  Shear  (Vl)          0        0        0        0

Torsional Shear  (Mt)          1        1        1        1

----------------------- -------- -------- -------- --------

Shear Stress Summary           0        0        2        2

Vessel Stresses  (N./mm²):

CIRCUMFERENTIAL PLANE         Cu       Cl       Du       Dl

(Stress Intensities         Left     Left    Right    Right

Outside   Inside  Outside   Inside

-----------------------------------------------------------

Two * Max Shear Stress        48       52      109       86

WRC 297 Stress Summations per ASME Sec. VIII Div. 2:

Vessel Stress Summation at Vessel-Nozzle Junction  (N./mm²):

Type of     |              Stress Values at                          |

Stress Int.  |                                                        |

Location    |    Au     Al     Bu     Bl     Cu     Cl     Du     Dl |

-------------------------------------------------------------------------

Circ. Pm (SUS) |      2      2      2      2      2      2      2      2|

Circ. Pl (SUS) |      1      1      1      1     -3     -3      5      5|

Circ. Q  (SUS) |    -16     16     24    -24    -36     36     54    -54|

-------------------------------------------------------------------------

Long. Pm (SUS) |      1      1      1      1      1      1      1      1|

Long. Pl (SUS) |     -3     -3      5      5      3      3     -1     -1|

Long. Q  (SUS) |    -31     31     49    -49    -19     19     27    -27|

-------------------------------------------------------------------------

Shear Pm (SUS) |      0      0      0      0      0      0      0      0|

Shear Pl (SUS) |      1      1     -1     -1      0      0      0      0|

Shear Q  (SUS) |      5      5      5      5      5      5      5      5|

Pm (SUS)       |    2.0    2.0    2.0    2.0    2.0    2.0    2.0    2.0|

-------------------------------------------------------------------------

Pm+Pl (SUS)    |    5.4    5.4    6.3    6.3    5.0    5.0    7.0    7.0|

-------------------------------------------------------------------------

Pm+Pl+Q (Total)|   34.7   31.8   55.6   43.7   38.1   36.8   61.7   48.2|

Vessel Stress Summation Comparison  (N./mm²):

Type of     |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.  |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          2.00            100.46      |      Passed     |

Pm+Pl (SUS)    |          7.00            150.69      |      Passed     |

Pm+Pl+Q (TOTAL)|         61.72            301.37      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

WRC 297 Stress Summations per ASME Sec. VIII Div. 2:

Vessel Stress Summation at Reinforcing Pad Edge  (N./mm²):

Type of     |              Stress Values at                          |

Stress Int.  |                                                        |

Location    |    Au     Al     Bu     Bl     Cu     Cl     Du     Dl |

-------------------------------------------------------------------------

Circ. Pm (SUS) |      4      4      4      4      4      4      4      4|

Circ. Pl (SUS) |     -1     -1      3      3     -2     -2      6      6|

Circ. Q  (SUS) |     -2      2     14    -14    -51     51     97    -97|

-------------------------------------------------------------------------

Long. Pm (SUS) |      2      2      2      2      2      2      2      2|

Long. Pl (SUS) |     -2     -2      6      6     -1     -1      3      3|

Long. Q  (SUS) |     -4      4     50    -50    -16     16     28    -28|

-------------------------------------------------------------------------

Shear Pm (SUS) |      0      0      0      0      0      0      0      0|

Shear Pl (SUS) |      0      0      0      0      0      0      0      0|

Shear Q  (SUS) |      1      1      1      1      1      1      1      1|

Pm (SUS)       |    4.0    4.0    4.0    4.0    4.0    4.0    4.0    4.0|

-------------------------------------------------------------------------

Pm+Pl (SUS)    |    3.0    3.0    8.0    8.0    2.0    2.0   10.0   10.0|

-------------------------------------------------------------------------

Pm+Pl+Q (Total)|    5.4    5.6   58.0   42.0   49.0   53.0  107.0   87.0|

Vessel Stress Summation Comparison  (N./mm²):

Type of     |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.  |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          4.00            100.46      |      Passed     |

Pm+Pl (SUS)    |         10.00            150.69      |      Passed     |

Pm+Pl+Q (TOTAL)|        107.01            301.37      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

WRC 297 Stress Summations per ASME Sec. VIII Div. 2:

Nozzle Stress Summation at Vessel-Nozzle Junction  (N./mm²):

Type of     |              Stress Values at                          |

Stress Int.  |                                                        |

Location    |    Au     Al     Bu     Bl     Cu     Cl     Du     Dl |

-------------------------------------------------------------------------

Circ. Pm (SUS) |      0      0      0      0      0      0      0      0|

Circ. Pl (SUS) |      1      1      1      1      3      3     -1     -1|

Circ. Q  (SUS) |      0      0      0      0      0      0      0      0|

-------------------------------------------------------------------------

Long. Pm (SUS) |      0      0      0      0      0      0      0      0|

Long. Pl (SUS) |    -15    -15     15     15    -15    -15     15     15|

Long. Q  (SUS) |    -40     40     62    -62    -48     48     70    -70|

-------------------------------------------------------------------------

Shear Pm (SUS) |      0      0      0      0      0      0      0      0|

Shear Pl (SUS) |      1      1     -1     -1     -1     -1      1      1|

Shear Q  (SUS) |      6      6      6      6      6      6      6      6|

Pm (SUS)       |    0.0    0.0    0.0    0.0    0.0    0.0    0.0    0.0|

-------------------------------------------------------------------------

Pm+Pl (SUS)    |   16.1   16.1   15.1   15.1   18.1   18.1   16.1   16.1|

-------------------------------------------------------------------------

Pm+Pl+Q (Total)|   57.7   27.8   77.3   49.0   66.8   33.8   87.1   55.9|

Nozzle Stress Summation Comparison  (N./mm²):

Type of     |     Max. S.I.     S.I. Allowable     |      Result     |

Stress Int.  |                                      |                 |

-------------------------------------------------------------------------

Pm (SUS)       |          0.00            100.50      |      Passed     |

Pm+Pl (SUS)    |         18.11            150.75      |      Passed     |

Pm+Pl+Q (TOTAL)|         87.13            301.50      |      Passed     |

Because only sustained loads were specified, the Pm+Pl+Q allowable was 3 * Smh.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N9                                From:   20

Pressure for Reinforcement Calculations        P       0.720  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Inside Diameter of Cylindrical Shell           D     1000.00  mm.

Shell Finished (Minimum) Thickness             t      6.0000  mm.

Shell Internal Corrosion Allowance             c      0.0000  mm.

Shell External Corrosion Allowance            co      0.0000  mm.

Distance from Bottom/Left Tangent                      50.00  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                           45.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv      6.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Flange Class                                             150

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N9

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Cylindrical Shell, tr [Int. Press]

= P*R/(Sv*E-0.6*P) per UG-27 (c)(1)

= 0.72*500/(100*1-0.6*0.72)

= 0.3585 mm.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.72*30.2/(100*1+0.4*0.72)

= 0.0216 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    85.6996  mm.

Parallel to Vessel Wall, opening length              d    42.8498  mm.

Normal to Vessel Wall (Thickness Limit), no pad   Tlnp    15.0000  mm.

Taking a UG-36(c)(3)(a) exemption for nozzle: N9.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0216 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 0.3585 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 1.5000 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 1.5, 1.5 ) ]

= 1.5000 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.022, 1.5 )

= 1.5000 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N9

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Drop for this Nozzle is : 0.9106 mm.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 156.9106 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N3                                From:   40

Pressure for Reinforcement Calculations        P       0.522  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Outside Diameter of Bolted Blind Flange        D     1130.00  mm.

Head Finished (Minimum) Thickness              t     16.0000  mm.

Head Internal Corrosion Allowance              c      0.0000  mm.

Head External Corrosion Allowance             co      0.0000  mm.

Distance from Head Centerline                 L1    350.0000  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.46  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                          270.00  deg

Diameter                                            100.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        40S

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv     16.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Pad Material                                     SA-240 316L

Pad Allowable Stress at Temperature           Sp      100.50  N./mm²

Pad Allowable Stress At Ambient              Spa      115.00  N./mm²

Diameter of Pad along vessel surface          Dp    214.3000  mm.

Thickness of Pad                              te      6.0000  mm.

Weld leg size between Pad and Shell           Wp      6.0000  mm.

Groove weld depth between Pad and Nozzle    Wgpn      6.0000  mm.

Reinforcing Pad Width                                50.0000  mm.

Flange Class                                             300

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N3

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation          114.300  mm.

Actual Thickness Used in Calculation                   6.020  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.52*57.1/(100*1+0.4*0.52)

= 0.0297 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl   204.5208  mm.

Parallel to Vessel Wall, opening length              d   102.2604  mm.

Normal to Vessel Wall (Thickness Limit), pad side Tlwp    21.0495  mm.

The Pad diameter is greater than the Diameter Limit. The excess will not be considered.

Weld Strength Reduction Factor [fr1]:

= min( 1, Sn/Sv )

= min( 1, 100/100 )

= 1.000

Weld Strength Reduction Factor [fr2]:

= min( 1, Sn/Sv )

= min( 1, 100/100 )

= 1.000

Weld Strength Reduction Factor [fr4]:

= min( 1, Sp/Sv )

= min( 1, 100/100 )

= 1.000

Weld Strength Reduction Factor [fr3]:

= min( fr2, fr4 )

= min( 1, 1 )

= 1.000

Results of Nozzle Reinforcement Area Calculations: (cm²)

AREA AVAILABLE, A1 to A5     |      MAWP|  External|     Mapnc|

---------------------------------------------------------------

Area Required             Ar |     6.189|     6.779|        NA|

Area in Shell             A1 |     3.983|     2.803|        NA|

Area in Nozzle Wall       A2 |     2.522|     2.534|        NA|

Area in Inward Nozzle     A3 |     0.000|     0.000|        NA|

Area in Welds    A41+A42+A43 |     0.360|     0.360|        NA|

Area in Element           A5 |     5.413|     5.413|        NA|

---------------------------------------------------------------

TOTAL AREA AVAILABLE    Atot |    12.278|    11.111|        NA|

The External Pressure Case Governs the Analysis.

Nozzle Angle Used in Area Calculations                 90.00  degs.

The area available without a pad is Insufficient.

The area available with the given pad is Sufficient.

SELECTION OF POSSIBLE REINFORCING PADS:   Diameter  Thickness

Based on given Pad Thickness:            132.3302     6.0000  mm.

Based on given Pad Diameter:             214.3000     1.1991  mm.

Based on Shell or Nozzle Thickness:      132.2709     6.0198  mm.

Area Required [A]:

= 0.5 * d * t + t * tn(1-fr1) per UG-39(b)(1)

= 0.5*102*13.3+13.3*6.02(1-1)

= 6.779 cm²

Reinforcement Areas per Figure UG-37.1

Area Available in Shell [A1]:

= d( E1*t - F*tr ) - 2 * tn( E1*t - F*tr ) * ( 1 - fr1 )

= 102 ( 1 * 16 - 1 * 13.3 ) - 2 * 6.02

( 1 * 16 - 1 * 13.3 ) * ( 1 - 1 )

= 2.803 cm²

Area Available in Nozzle Wall Projecting Outward [A2]:

= ( 2 * Tlwp ) * ( tn - trn ) * fr2

= ( 2 * 21 ) * ( 6.02 - 0 ) * 1

= 2.534 cm²

Area Available in Welds [A41 + A42 + A43]:

= Wo² * fr3 + (Wi-can/0.707)² * fr2 + Wp² * fr4

= 6² * 1 + (0 )² * 1 + 0² * 1

= 0.360 cm²

Area Available in Element [A5]:

= (min(Dp,DL)-(Nozzle OD))*(min(tp,Tlwp,te)) * fr4

= ( 205 - 114 ) * 6 * 1

= 5.413 cm²

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0297 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 12.1046 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 12.1046 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 5.2578 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 5.26, max( 12.1, 1.5 ) ]

= 5.2578 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.03, 5.26 )

= 5.2578 mm.

Available Nozzle Neck Thickness = 0.875 * 6.020 = 5.267 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N3

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Intermediate Calc. for pad/shell Welds   TminPad      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Pad Weld             3.0000 = 0.5*TminPad  4.2420 = 0.7 * Wp mm.

Weld Strength and Weld Loads per UG-41.1, Sketch (a) or (b)

Weld Load [W]:

= max( 0, (A-A1+2*tn*fr1*(E1*t-tr))Sv)

= max( 0, (6.78 - 2.8 + 2 * 6.02 * 1 *

(1 * 16 - 13.3 ) )100 )

= 43254.56 N

Note: F is always set to 1.0 throughout the calculation.

Weld Load [W1]:

= (A2+A5+A4-(Wi-Can/.707)²*fr2)*Sv

= ( 2.53 + 5.41 + 0.36 - 0 * 1 ) * 100

= 83448.29 N

Weld Load [W2]:

= (A2 + A3 + A4 + (2 * tn * t * fr1)) * Sv

= ( 2.53 + 0 + 0.36 + ( 1.93 ) ) * 100

= 48422.59 N

Weld Load [W3]:

= (A2+A3+A4+A5+(2*tn*t*fr1))*S

= ( 2.53 + 0 + 0.36 + 5.41 + ( 1.93 ) ) * 100

= 102798.16 N

Strength of Connection Elements for Failure Path Analysis

Shear, Outward Nozzle Weld [Sonw]:

= (pi/2) * Dlo * Wo * 0.49 * Snw

= ( 3.14/2.0 ) * 114 * 6 * 0.49 * 100

= 53022. N

Shear, Pad Element Weld [Spew]:

= (pi/2) * DP * WP * 0.49 * SEW

= ( 3.14/2.0 ) * 214 * 6 * 0.49 * 100

= 99411. N

Shear, Nozzle Wall [Snw]:

= (pi *( Dlr + Dlo )/4 ) * ( Thk - Can ) * 0.7 * Sn

= (3.14 * 54.1 ) * ( 6.02 - 0 ) * 0.7 * 100

= 71994. N

Tension, Pad Groove Weld [Tpgw]:

= ( pi/2) * Dlo * Wgpn * 0.74 * Seg

=  (3.14/2 ) * 114 * 6 * 0.74 * 100

= 80108. N

Tension, Shell Groove Weld [Tngw]:

= (pi/2) * Dlo * ( T - cas ) * 0.74 * Sng

= ( 3.14/2.0 ) * 114 * ( 16 - 0 ) * 0.74 * 100

= 213532. N

See UG-41 and UW-15 for clarification of allowable stresses.

Strength of Failure Paths:

Failure Path [Path 1-1]:

= ( SPEW + SNW ) = ( 99411 + 71994 ) = 171405 N

Failure Path [Path 2-2]:

= ( Sonw + Tpgw + Tngw + Sinw )

= ( 53022 + 80108 + 213532 + 0 ) = 346663 N

Failure Path [Path 3-3]:

= ( Spew + Tngw + Sinw )

= ( 99411 + 213532 + 0 ) = 312944 N

Summary of Failure Path Calculations:

Path 1-1 = 171404  N  , must exceed W = 43254 N      or W1 = 83448 N

Path 2-2 = 346663  N  , must exceed W = 43254 N      or W2 = 48422 N

Path 3-3 = 312943  N  , must exceed W = 43254 N      or W3 = 102798 N

Maximum Allowable Pressure for this Nozzle at this Location:

Converged Maximum Allowable Pressure in the Operating case: 0.52 bars

Note: The MAWP of this junction was limited by the parent Shell/Head.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 166.0508 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.

Input,  Nozzle Desc: N2                                From:   40

Pressure for Reinforcement Calculations        P       0.522  bars

Temperature for Internal Pressure           Temp          85  °C

Parent Material                                  SA-240 316L

Parent Allowable Stress at Temperature        Sv      100.46  N./mm²

Parent Allowable Stress At Ambient           Sva      115.00  N./mm²

Outside Diameter of Bolted Blind Flange        D     1130.00  mm.

Head Finished (Minimum) Thickness              t     16.0000  mm.

Head Internal Corrosion Allowance              c      0.0000  mm.

Head External Corrosion Allowance             co      0.0000  mm.

Distance from Head Centerline                 L1    350.0000  mm.

User Entered Minimum Design Metal Temperature           4.00  °C

Type of Element Connected to the Parent : Nozzle

Material                                       SA-312 TP316L

Material UNS Number                                   S31603

Material Specification/Type                       Smls. pipe

Allowable Stress at Temperature               Sn      100.50  N./mm²

Allowable Stress At Ambient                  Sna      115.00  N./mm²

Diameter Basis (for tr calc only)                    Outside

Layout Angle                                          180.00  deg

Diameter                                             50.0000  mm

Size and Thickness Basis                             Nominal

Nominal Thickness                                        160

Flange Material                                 SA-182 F316L

Flange Type                                 Weld Neck Flange

Corrosion Allowance                          can      0.0000  mm.

Joint Efficiency of Shell Seam at Nozzle      E1        1.00

Joint Efficiency of Nozzle Neck               En        1.00

Outside Projection                            ho    150.0000  mm.

Weld leg size between Nozzle and Pad/Shell    Wo      6.0000  mm.

Groove weld depth between Nozzle and Vessel Wgnv     16.0000  mm.

Inside Projection                              h      0.0000  mm.

Weld leg size, Inside Element to Shell        Wi      0.0000  mm.

Pad Material                                     SA-240 316L

Pad Allowable Stress at Temperature           Sp      100.50  N./mm²

Pad Allowable Stress At Ambient              Spa      115.00  N./mm²

Diameter of Pad along vessel surface          Dp    162.0000  mm.

Thickness of Pad                              te      6.0000  mm.

Weld leg size between Pad and Shell           Wp      6.0000  mm.

Groove weld depth between Pad and Nozzle    Wgpn      6.0000  mm.

Reinforcing Pad Width                                50.8375  mm.

Flange Class                                             150

Flange Grade                                          GR 2.3

The Pressure Design option was MAWP + static head (to the nozzle).

Reinforcement CALCULATION, Description: N2

ASME Code, Section VIII, Div. 1, UG-37 to UG-45

Actual Outside Diameter Used in Calculation           60.325  mm.

Actual Thickness Used in Calculation                   8.738  mm.

Nozzle input data check completed without errors.

Reqd thk per UG-37(a) of Nozzle Wall, trn [Int. Press]

= P*Ro/(Sn*E+0.4*P) per Appendix 1-1 (a)(1)

= 0.52*30.2/(100*1+0.4*0.52)

= 0.0157 mm.

UG-40, Limits of Reinforcement: [Internal Pressure]

Parallel to Vessel Wall (Diameter Limit)            Dl    92.3250  mm.

Parallel to Vessel Wall                        Rn+tn+t    46.1625  mm.

Normal to Vessel Wall (Thickness Limit), pad side Tlwp    27.8440  mm.

The Pad diameter is greater than the Diameter Limit. The excess will not be considered.

Taking a UG-36(c)(3)(a) exemption for nozzle: N2.

This calculation is valid for nozzles that meet all the requirements of

paragraph UG-36.  Please check the Code carefully, especially for nozzles

that are not isolated or do not meet Code spacing requirements. To force

the computation of areas for small nozzles go to Tools->Configuration

and check the box to force the UG-37 small nozzle area calculation.

UG-45 Minimum Nozzle Neck Thickness Requirement: [Int. Press.]

Wall Thickness for Internal/External pressures      ta = 0.0157 mm.

Wall Thickness per UG16(b),                      tr16b = 1.5000 mm.

Wall Thickness, shell/head, internal pressure     trb1 = 12.1046 mm.

Wall Thickness                  tb1 = max(trb1, tr16b) = 12.1046 mm.

Wall Thickness                  tb2 = max(trb2, tr16b) = 1.5000 mm.

Wall Thickness per table UG-45                     tb3 = 3.4200 mm.

Determine Nozzle Thickness candidate [tb]:

= min[ tb3, max( tb1,tb2) ]

= min[ 3.42, max( 12.1, 1.5 ) ]

= 3.4200 mm.

Minimum Wall Thickness of Nozzle Necks [tUG-45]:

= max( ta, tb )

= max( 0.016, 3.42 )

= 3.4200 mm.

Available Nozzle Neck Thickness = 0.875 * 8.738 = 7.645 mm. -> OK

SA-240 316L, Min Metal Temp without impact per UHA-51:  -196 °C

SA-312 TP316L, Min Metal Temp without impact per UHA-51:  -196 °C

Weld Size Calculations, Description: N2

Intermediate Calc. for nozzle/shell Welds   Tmin      6.0000  mm.

Intermediate Calc. for pad/shell Welds   TminPad      6.0000  mm.

Results Per UW-16.1:

Required Thickness    Actual Thickness

Nozzle Weld          4.2000 = 0.7 * tmin.  4.2420 = 0.7 * Wo mm.

Pad Weld             3.0000 = 0.5*TminPad  4.2420 = 0.7 * Wp mm.

Skipping the nozzle attachment weld strength calculations. Per UW-15(b)(2) the nozzles

exempted by UG-36(c)(3)(a) (small nozzles) do not require a weld strength check.

The Cut Length for this Nozzle is, Drop + Ho + H + T : 166.0508 mm.

PV Elite is a trademark of Hexagon AB, 2024, All rights reserved.